3GPP TSG RAN WG1 #108-e		R1-2201247

e-Meeting, February 21st – March 3rd, 2022

Source:	OPPO

Title:	Discussion on “Response LS on the reporting of the Tx TEG association information”

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

RAN1 received a LS from RAN2 with regard to the higher layer parameters for the mitigation of UE/gNB Rx/Tx timing delays [1]. In this contribution, we will discuss the related questions raised by RAN2 and suggest the corresponding replies.

Discussion

- Duplicated higher layer parameters: For UE measurement, there are three higher layer parameters related to the number of Rx TEGs for the PRS reception, i.e., numOfUERxTEG-PerPRSResource, MeasPRSwithDiffRxTEGs_Request_RSTD, and MeasPRSwithDiffRxTEGs_Request_UXRxTx [2]. Only two of them are needed when there are separate configurations for DL-TDOA and multi-RTT positioning methods. Thus, the first one can be removed. The similar situation is also applied for numOfTRPRxTEG-PerPRSResource_RTOA and MeasPosSRSwithDiffRxTEGs_Request.  Thus, we suggest to confirm RAN2 understanding.

Proposed reply 1:

Regarding the duplicated parameters, RAN1 shares the same understanding with RAN2.

- Value of maxNumOfUE-RxTEG: In RAN1#107e meeting, there was an agreement as below:

Agreement Make the following modification on the previous agreement made in RAN#106bis-e: - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] (FFS: other values), where the maximum value of N depends on UE capability, and applies to all DL PRS positioning frequency layers - Note: If N is not explicitly included in the request, it is up to UE to determine the number of different UE Rx TEGs to measure the same DL PRS resource within its capability - The TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. - Note: All RSTD measurements are relative to a single reference timing

- Based on the agreement, the candidate values of N are {2,3,4,6,8}. Thus, the maximum number is 8. However, for UE reporting, the ID of Rx TEGs are indexed among all the positioning frequency layers. Thus, the maximum value of Rx TEG ID can be 31. In the current RAN2 CR [4], a single parameter is used for these two purposes. We need to have a clarification.

Proposed reply 2:

Regarding the value of maxNumOfUE-RxTEG, it is 8 rather than 32 according to RAN1 agreement.

For the ID of Rx TEG, the candidate values are 0, 1, …, 30 and 31.

- Need of srs-PosResourceSetId: According to the configuration of SRS resources for positioning in TS 38.331, the ID for each SRS resource for positioning is unique. Thus, if the new design signaling can indicate the corresponding SRS resource, the SRS resource set ID is not needed. Thus, we thus the following reply:

Proposed reply 3:

From RAN1 perspective, srs-PosResourceSetId is NOT needed in UE TxTEG association report.

Conclusion

In this contribution, we discussed the question of RAN2 LS and suggest replies as below:

Proposed reply 1:

Regarding the duplicated parameters, RAN1 shares the same understanding with RAN2.

Proposed reply 2:

Regarding the value of maxNumOfUE-RxTEG, it is 8 rather than 32 according to RAN1 agreement.

For the ID of Rx TEG, the candidate values are 0, 1, …, 30 and 31.

Proposed reply 3:

From RAN1 perspective, srs-PosResourceSetId is NOT needed in UE TxTEG association report.

References

- R1-2200878, Response LS on the reporting of the Tx TEG association information

- R1-2112976, Consolidated higher layers parameter list for Rel-17 NR

- RAN1#107e Report

- R2-2201723, Running LPP CR for NR positioning enhancements