3GPP TSG RAN WG1 #108-e		R1-2201248

e-Meeting, February 21st – March 3rd, 2022

Source:	OPPO

Title:	Discussion on “LS on SRS for multi-RTT positioning”

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

RAN1 received a LS from RAN4 with the question “whether Rel-15 SRS is applicable for UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement, and if so, from which release” [1]. In this contribution, we will provide discussed this issue and suggest our reply to this question.

Discussion

- Reuse of Rel-15 SRS for positioning was discussed in Rel-16 positioning and the following agreement was made in RAN1#99 [2]:

Agreement: Support reuse of Rel-15 SRS resource set for NR UL RTOA, AoA and gNB RSRP measurements for positioning in NR. - Note: There is no impact to specifications managed by RAN1 - Note: There is no impact to specifications managed by RAN4 for UE requirements - Note: No new UE behaviour is expected

- According to the above-mentioned RAN1 agreement, there is no consensus to support Rel-15 SRS for UE Rx-Tx time different measurement or gNB Rx-Tx time difference measurement.

- In the current version of TS 38.214, the following description is for UE Rx-Tx time difference measurements:

- TS 38.214 Section 5.1.6.2 The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different positioning frequency layers.

- That means, according to TS 38.214, the UE Rx-Tx time difference measurement is based on an SRS resource or an SRS resource set for positioning rather than Rel-15 SRS resource or resource set.

- Based on the above discussions, we suggest the following reply to RAN4’s question:

Proposed reply:

According to RAN1 agreement made in RAN1#99, Rel-15 SRS is NOT applicable for UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement.

Conclusion

In this contribution, we discussed the question of RAN4 LS and suggest a reply as below:

Proposed reply:

According to RAN1 agreement made in RAN1#99, Rel-15 SRS is NOT applicable for UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement.

References

- R1-2200900(R4-2202680), LS on SRS for multi-RTT positioning

- R1-2000151, Final Report of 3GPP TSG RAN WG1 #99 v1.0.0