3GPP TSG RAN WG1 #108-e		R1-2201258

e-Meeting, February 21st – March 03rd, 2022

Source:	OPPO

Title:	Remaining issues on UL feedback for MBS of RRC-CONNECTED UEs

Agenda Item:	8.12.2

Document for:	Discussion and Decision

Introduction

How to improve reliability of MBS for RRC-Connected UE was discussed and the following agreements were achieved [1]:

Agreement

When PUCCH carrying multicast HARQ-ACK only overlaps with PUSCH with the same priority, support UL-DAI indicating the number of HARQ-ACK bits for multicast as defined in Rel-16 for unicast HARQ-ACK.

  - FFS it is applied to a single G-RNTI or applied to all configured G-RNTIs.

Agreement

Support multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with the same priority for the following cases:

  - If the unicast and multicast HARQ-ACK codebooks are both Type-1

    - Option1-1: the 1-bit UL DAI with value “1” indicates multiplexing unicast and multicast HARQ-ACK codebooks onto the same PUSCH.

    - Option1-2: two bits UL DAI separately indicate whether multiplexing unicast and/or multicast HARQ-ACK codebooks onto the same PUSCH

      - FFS whether it is applied to a single G-RNTI or applied to all configured G-RNTIs.

  - If both unicast and multicast HARQ-ACK codebooks are Type-2, down-select from:

    - Option2-1: the 2-bit UL DAI is applicable to both HARQ-ACK codebooks.

    - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

      - FFS whether a single UL DAI field is applied to all G-RNTIs, or separate UL DAI fields are applied for each configured G-RNTI.

  - FFS the unicast and multicast HARQ-ACK codebooks are different Types.

In this contribution we discussed the remaining open issues on reliability enhancement for NR MBS of RRC-CONNECTED UEs.

Discussion

- More than one NACK-only based feedback bits

When more than one NACK-only based feedback bits are available for transmission in the same PUCCH slot, 2 alternatives were agreed in RAN1#106bis (copied as below) for down selection.

Agreement: When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, further decide based on the following subset of alternatives (from previous agreement) with potential further down-selection: - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - Alt2: Support sub-slot based PUCCH for this case. - Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot. - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - Alt5: NACK-only bundling

The issue was further discussed in the last meeting with following draft proposal:

#### Proposal 3.2.2-1 When more than one NACK-only feedback are available for transmission in the same PUCCH slot, - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - for more than 4 TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK. - The PUCCH resource to be used for the transmission is based on the last DCI scheduling multicast.

NACK-only based feedback is introduced for sake of reducing PUCCH overhead thanks to that multiple UEs can shared the same resource(s). However, if too many TBs are supported for NACK-only HARQ-ACK feedback, the benefit of NACK-only feedback may degrade as too many PUCCH resources are needed for different possible combinations. In view of above, it is necessary to define an upper bound for the number of orthogonal PUCCH resources, i.e., 15 as in the proposal. However, it does not make sense for the network to transmit more than 4 TBs if 15 was defined as the maximum number of orthogonal PUCCH resources for NACK-only HARQ feedback, as it is against the original intention of configuring the G-RNTI(s) as NACK-only feedback by network, and the traffic configured with NACK-only feedback is usually not so urgent.

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot,

- define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback,

- UE does not expect to report NACK-only feedback for more than 4 TBs.

- Multiplexing HARQ-ACK bits of MBS on PUSCH

In the last meeting it was agreed that UL-DAI can indicate the number of HARQ-ACK bits for multicast, however it is FFS whether the UL-DAI is applied to single G-RNTI or all configured G-RNTIs. As PUCCH resources are configured for all G-RNTIs, the PUCCH that overlapping with PUSCH may contain HARQ-ACK bits for multiple G-RNTIs, the UL-DAI should be applied to all configured G-RNTIs.

- When UL-DAI is used to indicate the number of HARQ-ACK bits for multicast, the UL-DAI is applied to all configured G-RNTIs.

- PUCCH power control

If HARQ-ACK for MBS is transmitted on dedicated PUCCH resource, as there is no associated UE specific PDCCH, gNB cannot adjust the PUCCH transmission power per UE, this may impact the reception of HARQ feedback for the group-common PDSCH at gNB. To address the problem, a TPC-PUCCH-RNTI should be configured for a UE within the MBS group such that gNB can adjust the PUCCH transmission power of each UE. Considering that the TRP for MBS and that for unicast may be different, and even though the TPR for unicast and MBS are same, gNB may expect different transmission power for PUCCH carrying NACK-only HARQ-ACK bit (if configured), hence it is preferable that a separate TPC-PUCCH-RNTI is configured for MBS.

- For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

Conclusion

In this contribution, we discuss the remaining issues on reliability enhancement of MBS. Based on the discussion, the following proposals are given:

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot,

- define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback,

- UE does not expect to report NACK-only feedback for more than 4 TBs.

- When UL-DAI is used to indicate the number of HARQ-ACK bits for multicast, the UL-DAI is applied to all configured G-RNTIs.

- For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

Reference

[1] Chairman note’s, RAN1#107bis-e, January 2022.