3GPP TSG RAN WG1 #108-e		R1-2201280

e-Meeting, February 21st – March 3rd, 2022

Source: 	OPPO

Title:	Further discussion on Paging enhancements for power saving

Agenda item:	8.7.1.1

Document for:	Discussion

Introduction

In RAN1#107bis-e meeting, paging enhancements for power saving was discussed, and the following agreements were made [1].

Agreement

A new PEI-RNTI is supported for DCI format 2_7.

The following text proposals are adopted:

TS 38.213 section 10.1: <Unchanged text is omitted> - a Type2A-PDCCH CSS set configured by peiSearchSpace in DownlinkConfigCommonSIB for a DCI format 2_7 with CRC scrambled by a PEI-RNTI on the primary cell of the MCG <Unchanged text is omitted>

TS 38.213 section 10.4B: <Unchanged text is omitted> A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by PEI-RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. <Unchanged text is omitted>

Agreement

For whether and how to accommodate PEI-O location determination for the case POnumPerPEI is smaller than Ns, decide one of the following alternatives

- Alt-2: It is supported, and UE applies the single value in PEI-F_offset for the frame-level offset and the -th value out of  configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset

Note: The number of PO mapping to one PEI should be multiple of Ns when POnumPerPEI is larger than Ns

Agreement

For PEI-O location determination:

- Range of PEI-F_offset, in unit of frame, is

- Range of firstPDCCH-MonitoringOccasionOfPEI-O, in unit of symbol, is decided as one of the following alternatives:

  - Alt-1: The same value range as firstPDCCH-MonitoringOccasionOfPO

Agreement

For the value PEI-RNTI, decide one of the following:

- Alt-1: PEI-RNTI is of fixed value, and value design is up to RAN2.

Conclusion

For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI.

- If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304.

- Note: No specification impact

Agreement

- The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-O.

Agreement

The transmission power of PEI is based on the same determination method as paging PDCCH, and the following TP is adopted.

.............................................................. Text Proposal for 38.213.............................................................................. 4.1 Cell search Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211]. A UE assumes that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE. The UE may assume that the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB. If the UE has not been provided dedicated higher layer parameters, the UE may assume that the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB when the UE monitors PDCCHs for a DCI format 1_0 with CRC scrambled by SI-RNTI, P-RNTI, or RA-RNTI, and DCI format 2_7 with CRC scrambled by PEI-RNTI. ............................................................... Text Proposal for 38.213..................................................................................

Agreement

UE expects at most one PEI outcome from each PDCCH monitoring occasion of PEI-O, and the following text proposal is adopted.

Section 10.1 of TS 38. 213 *** Unchanged text is omitted *** If a UE is provided - one or more search space sets by corresponding one or more ofsearchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation,pagingSearchSpace, peiSearchSpace, ra-SearchSpace, or a CSS set byPDCCH-Config, and - a SI-RNTI, a P-RNTI, a PEI-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. *** Unchanged text is omitted ***

Agreement

If one PEI-O is associated with POs of 2 PFs,

- The two PFs are consecutive PFs configured in SIB

  - FFS: two PFs are consecutive PFs within the same paging cycle

- Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) -

Agreement

- LS to RAN2 for sharing agreements with potential RAN2 specification impacts

Conclusion

- For Rel-17, inclusion of SI change indication or ETWS/CMAS notification in DCI format 2_7 is not supported.

Conclusion

For Rel-17 DCI format 2_7,

- Explicit starting position configuration for paging indication field or TRS availability indication field is not supported.

  - Paging indication field starts from the first bit, and TRS availability indication field starts from the next bit after the end of the paging indication field

- UE does not expect the total size of paging indication filed and TRS availability indication field is larger than the DCI payload size

Working assumption

- SearchSpaceId = 0 can be configured forpeiSearchSpace for the case of CORESET multiplexing pattern 2 or 3

Agreement

- Draft LS R1-2200788 is endorsed in principle

- Include above working assumption in the LS on paging enhancement to RAN2

Agreement

- Final LS R1-2200800 is endorsed.

In this contribution, the remaining issues on Paging enhancements for power saving are discussed.

Paging early indication for paging reception

UE Behv-A

In RAN#93-e, there agreed the following guidance for the design of paging early indication (PEI) [2]:

- Support PDCCH-based PEI as the only option • Only essential function for PEI is support • New DCI format • Higher layer configuration, including SS • Details of the procedures of PEI monitoring, and identification of MOs before PO • Only Behv-A (per RAN1#104e agreement) is supported • If TRS availability indication is agreed to be supported in both paging DCI and the DCI format for PEI, same mechanism/principle for TRS availability indication is adopted for the two DCI formats • Supporting TRS availability indication in DCI format for PEI shall not delay the completion of essential functionality of PEI

Agreements in RAN1#104-e: For the evaluation and comparison of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, the following are assumed: - Behv-A: - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO - Behv-B: - PEI indicates whether or not UE should monitor a PO - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

With the agreements about UE Behv-A, the UE behaviour should be clear in TS38213-h00. Thus we have the following text proposal for the draft CR in TS38213-h00.

Proposal 1: Clarify UE behaviour if UE does not detect PEI at all PEI occasion(s) for the PO. And adopt the TP in TS38.213.

TS 38.213 section 10.4: <Unchanged text is omitted> 10.4A PDCCH monitoring for early indication of paging UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO when provided the parameter for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state. <Unchanged text is omitted>

Conclusions

In this contribution, remaining issues are discussed. As summary, we have the following proposals:

Proposal 1: Clarify UE behaviour if UE does not detect PEI at all PEI occasion(s) for the PO. And adopt the TP in TS38.213.

TS 38.213 section 10.4: <Unchanged text is omitted> 10.4A PDCCH monitoring for early indication of paging UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO when provided the parameter for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state. <Unchanged text is omitted>

Reference

- Chair's Notes RAN1#107bis-e

- RP-212611, “Moderator’s summary for discussion [93e-18-PowSav-WI]: Final Round”, Moderator (CMCC), RAN#93-e