3GPP TSG RAN WG1 #108-e		R1-2201302

e-Meeting, February 21st – March 3rd, 2022

Source:	OPPO

Title:	Discussion on UE features for FR2-2

Agenda Item:	8.16.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the UE features for FR2-2. This discussion is based on the summary of UE features for supporting NR from 52.6 GHz to 71 GHz [1].

- Discussion

In the last RAN1#107b-e meeting, a great progress was made on the UE feature for FR2-2, where the FGs were agreed in the most details. In this section, we further discuss some of the remaining points to further polish this list to a final version.

FG 24-1a

FG24-1a should be a mandatory FG to support FG2-2, we propose to remove the corresponding bracket.

Proposal 1: for FG24-1a,

- removing bracket on “[A UE that supports FR2-2 must indicate this FG is supported]”.

FG 24-1b

In our opinion, the feature of wideband PRACH is not needed and motivated in the case where PSD limitation is not imposed. We propose to clearly mention the condition with PSD limitation as other FGs and remove the confusing description. Moreover, for UE supporting FG 24-2 to must indicate supporting FG 24-1b is not needed in the region where PSD limitation is not required. Thus, we suggest to remove this ‘must indicate’ requirement.

Proposal 2: for FG24-1b,

- replacing “[Note: This FG is only supported in bands for shared spectrum operation]” with “This FG is only supported in bands under PSD limitation in shared spectrum operation”.

- removing “[A UE that supports 24-2 must indicate this FG is supported]”.

FG 24-1c

Similar as FG24-1b, the feature of multi-RB PUCCH format should be supported under PSD limitation case, we propose to remove the confusing description.

Proposal 3: for FG24-1c,

- removing “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]”.

FG 24-1d, FG24-1e

In the last version, there is an FFS to extend these FGs of multi-PDSCH/PUSCH scheduling by single DCI for 120kHz SCS to other frequency range. In our opinion, multi-PDSCH/PUSCH scheduling by single DCI is introduced mainly for 480kHz and 960kHz to reduce UE PDCCH monitoring capability for FR2-2, and is extended to 120kHz for FR2-2 for an unified design. There is no motivation to extend multi-PDSCH/PUSCH scheduling by single DCI for 120kHz to other frequency range, besides, extending these FGs to other frequency range is out of the WI scope for FR2-2. For these reasons, we do not support extending these FGs to other frequency range.

Proposal 4: for FG24-1d and FG24-1e,

- removing “FFS: to extend this FG to other frequency ranges”.

FG 24-4

In our view, multi-PDSCH scheduling by single DCI and the corresponding HARQ enhancements should be a mandatory component to support 480kHz DL transmission, the FFS for component 3 should be removed. On the other hand, according to the agreement marked in yellow below, the UE behavior of monitoring slots of Group (2) SS should be independent from that of monitoring slots of Group (1) SS, and should be a mandatory component to support 480kHz DL transmission. In addition, according to the agreement marked in cyan, only (Xs, Ys) = (4, 1) for 480kHz is mandatory and it should be clearly mentioned in component 4.

Agreement

- For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS

  - A SS is monitored within Y consecutive slots within a slot group of X slots

  - The Y consecutive slots can be located anywhere within the slot group of X slots

    - Note: There is no requirement to align the Y consecutive slots across UEs or with slot n0

  - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups

  - BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots

- For Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS

  - SS monitoring locations can be anywhere within a slot group of X slots, with the following exception

    - BD attempts for Type0-CSS for SSB/CORESET 0 multiplexing pattern 1, and additionally for Type0A/2-CSS if searchSpaceId = 0, occur in slots with index n0 and n0+X0, where n0 is as in Rel-15, X0=4 for 480 kHz SCS and X0=8 for 960 kHz SCS.

- Supported combinations of (X,Y)

  - A UE capable of multi-slot monitoring mandatorily supports

    - For SCS 480 kHz: (X,Y) = (4,1)

    - For SCS 960 kHz: (X,Y) = (8,1)

  - A UE capable of multi-slot monitoring optionally supports

    - For SCS 480 kHz: (X,Y) = (4,2)

    - For SCS 960 kHz: (X,Y) = (8,4), (4,2), (4,1)

      - Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8

- A UE capable of multi-slot monitoring mandatorily supports the following PDCCH monitoring within Y slots

  - For Y>1: FG3-1 (monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots)

  - For 960 kHz SCS For Y=1: FG3-5b with set1 = (7, 3)

    - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)]

  - For 480 kHz SCS For Y=1: FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot

    - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)]

  - The following supersedes FG3-5b and FG3-1 definition:

  - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD

  - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD

Proposal 5: for FG24-4,

- removing “FFS” for component 3.

- removing “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)” for component 4.

- adding a new component “7. Monitoring capability within a slot group of X slots of Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS”.

- replacing “Within the Ys = 1 slot” with “Within the Ys=1 slot (with Xs=4)” for component 4.

FG 24-4f

Since the UE behavior of monitoring slots of Group (2) SS is defined in FG24-4, the FFS for component 2 should be removed. Besides, according to the agreement above marked in cyan, (Xs, Ys) = (2, 1) should not be the candidate value for 480kHz and the note should be removed.

Proposal 6: for FG24-4f,

- removing “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)” for component 2.

- replacing “Component 1 candidate values: [one or more of] {[(2,1),] (4,2) }” with “Component 1 candidate value: (4,2)”.

- removing “Note: If (2,1) is not agreed, this FG will have no component candidate values and the component 1 description will be updated from (Xs,Ys) to (Xs,Ys)=(4,2) similar to FG 24-4 and 24-5”.

FG 24-5

Similar as FG24-4, multi-PDSCH scheduling by single DCI and the corresponding HARQ enhancements should be a mandatory component to support 960kHz DL transmission, the FFS for component 3 should be removed. The UE behavior of monitoring slots of Group (2) SS should be independent from that of monitoring slots of Group (1) SS, and should be a mandatory component to support 960kHz DL transmission. In addition, only (Xs, Ys) = (8, 1) for 960kHz is mandatory and it should be clearly mentioned.

Proposal 7: for FG24-5,

- removing “FFS” for component 3.

- removing “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)”.

- adding a new component “6. Monitoring capability within a slot group of X slots of Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS”.

- replacing “Within the Ys = 1 slot” with “Within the Ys=1 slot (with Xs=8)”.

FG 24-5f

Similar as FG24-4f, since the UE behavior of monitoring slots of Group (2) SS is defined in FG24-5, the FFS for component 2 should be removed. Besides, (Xs, Ys) = (4, 1) is optionally supported for 960kHz, so “Within each of the Ys = 2 or 4 slots” for component 2 should be replaced with “Within each of the Ys = 1, 2 or 4 slots”.

Proposal 8: for FG24-5f,

- removing “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)” for component 2.

- replacing “Within each of the Ys = 2 or 4 slots” with “Within each of the Ys = 1, 2 or 4 slots”.

FG 24-5a

Similar as FG24-4a, the component of “Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS” can be included in the FG of 960kHz SCS support for UL. We propose to remove the corresponding bracket.

Proposal 9: for FG24-5a,

- removing bracket on “[3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS]”.

FG 24-10

Regarding this FG, “per band” is preferred. In our view, UE may optionally report 56 symbols or 112 symbols for this FG. On the other hand, we prefer to add 480kHz SCS in the description to make it clearer.

Proposal 10: for FG24-10,

- supporting “per band”.

- replacing “[Additional beam switching time delay is not supported]” with “Additional beam switching time delay is not supported for 480kHz SCS”.

New FG for 32 HARQ processes

In RAN1 #107b-emeeting, the following agreement was achieved:

Agreement

- In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability.

Therefore, a new FG should be introduced to define the capability of supporting 32 HARQ processes. If introduced, this FG should be supported per FSPC.

Proposal 11: introducing a new FG to define the capability of supporting 32 HARQ processes.

- Conclusion

In this document, we provide our views on some of the feature groups based on conclusion in last RAN1 meeting for supporting NR from 52.6 GHz to 71 GHz. Our proposals are made as follows:

Proposal 1: for FG24-1a,

- removing bracket on “[A UE that supports FR2-2 must indicate this FG is supported]”.

Proposal 2: for FG24-1b,

- replacing “[Note: This FG is only supported in bands for shared spectrum operation]” with “This FG is only supported in bands under PSD limitation in shared spectrum operation”.

- removing “[A UE that supports 24-2 must indicate this FG is supported]”.

Proposal 3: for FG24-1c,

- removing “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]”.

Proposal 4: for FG24-1d and FG24-1e,

- removing “FFS: to extend this FG to other frequency ranges”.

Proposal 5: for FG24-4,

- removing “FFS” for component 3.

- removing “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)” for component 4.

- adding a new component “7. Monitoring capability within a slot group of X slots of Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS”.

- replacing “Within the Ys = 1 slot” with “Within the Ys=1 slot (with Xs=4)” for component 4.

Proposal 6: for FG24-4f,

- removing “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)” for component 2.

- replacing “Component 1 candidate values: [one or more of] {[(2,1),] (4,2) }” with “Component 1 candidate value: (4,2)”.

- removing “Note: If (2,1) is not agreed, this FG will have no component candidate values and the component 1 description will be updated from (Xs,Ys) to (Xs,Ys)=(4,2) similar to FG 24-4 and 24-5”.

Proposal 7: for FG24-5,

- removing “FFS” for component 3.

- removing “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)”.

- adding a new component “6. Monitoring capability within a slot group of X slots of Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS”.

- replacing “Within the Ys = 1 slot” with “Within the Ys=1 slot (with Xs=8)”.

Proposal 8: for FG24-5f,

- removing “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)” for component 2.

- replacing “Within each of the Ys = 2 or 4 slots” with “Within each of the Ys = 1, 2 or 4 slots”.

Proposal 9: for FG24-5a,

- removing bracket on “[3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS]”.

Proposal 10: for FG24-10,

- supporting “per band”.

- replacing “[Additional beam switching time delay is not supported]” with “Additional beam switching time delay is not supported for 480kHz SCS”.

Proposal 11: introducing a new FG to define the capability of supporting 32 HARQ processes.

- Reference

- R1-2200766, “Summary of UE features for supporting NR from 52.6 GHz to 71 GHz”, Moderator (AT&T).

Agreement:  Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1a | Basic FR2-2 UL support | 1. PRACH with 120KHz SCS and length 139 2. Support transmission of 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 | 24-1 | Yes | N/A | UL in FR2-2 is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements | [24-1] | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported | Per band | N/A | N/A | N/A | FFS: to extend this FG to other frequency ranges such as FR1 and FR2-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | [24-1a] | Yes | N/A | Multiple PUSCH scheduling by single DCI for 120kHz is not supported | Per band | N/A | N/A | N/A | FFS: to extend this FG to other frequency ranges such as FR1 and FR2-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB support for SA/DC initial access in FR2-2 | - Support 120KHz SSB for SA/DC initial access in FR2-2 | [24-1, 24-1a] | N/A | N/A | 120KHz SSB based stand-alone initial access in FR2-2 is not supported | N/A Per band | N/A | N/A | N/A | per band FFS: whether to split this FG for SA and DC | Optional [with/without] capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB support for SA/DC initial access in FR2-2 | 1. Support 480KHz SSB for SA/DC initial access in FR2-2 | 24-1[, 24-2, 24-4, 24-4a] | FFS N/A | N/A | 480KHz SSB for initial access in FR2-2 is not supported | [per UE][per band] | N/A | N/A | N/A | FFS: whether to split this FG for SA and DC | Optional [with/without] capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4a | 480KHz SCS support for UL | 1. PRACH with 480KHz and length 139 2. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1a, 24-4 | Yes | N/A | 480KHz SCS support for UL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4f | Enhanced PDCCH monitoring for 480KHz in FR2-2 | 1.) Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) 2.) Within each of the Ys = 2 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot as in FG 3-1 (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) | 24-4 | Yes | N/A | Enhanced PDCCH monitoring for 480KHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: [one or more of] {[(2,1),] (4,2) } Note: If (2,1) is not agreed, this FG will have no component candidate values and the component 1 description will be updated from (Xs,Ys) to (Xs,Ys)=(4,2) similar to FG 24-4 and 24-5 FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-5a | 960KHz SCS support for UL | 1. PRACH with 960KHz and length 139 2. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 [3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS] | 24-1a, 24-5 | Yes | N/A | 960KHz SCS support for UL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-5f | Enhanced PDCCH monitoring for 960KHz | 1.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys) 2.) Within each of the Ys = 2 or 4 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot as in FG 3-1 (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) | 24-5 | Yes | N/A | Enhanced PDCCH monitoring for 960KHz is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: one or more of {(4,1), (4,2), (8,4)} FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-6 | Support [Type 1] channel access procedure in uplink for FR2-2 unlicensed operation with shared spectrum channel access | 1. Support [Type 1] channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] | 24-1a | Yes | N/A | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-7 | Support [Type 2] channel access procedure in uplink for FR2-2 unlicensed operation with shared spectrum channel access | 1. Support [Type 2] channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] | 24-1a, 24-6 | Yes | N/A | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:  Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-10 | Additional beam switching time delay | Supported additional beam switching time delay d for 480 kHz SCS |  | Yes | N/A | [Additional beam switching time delay is not supported] | [Per UE/per band] | N/A | N/A | N/A | Candidate value set: 56 or 112 symbols | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note: continue discussion on whether to define default values or whether UEs need to signal this FG is supported if one or more other FG is supported

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1 | Basic FR2-2 DL support | 1. Support reception of 120kHz subcarrier spacing for DL data and control channels, SSB, and reference signals in FR2-2 for non-initial access |  | Yes | N/A | FR2-2 is not supported | [per UE][per band] | N/A | N/A | N/A |  | Optional with capability signalling A UE that supports FR2-2 must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1a | Basic FR2-2 UL support | 1. PRACH with 120KHz SCS and length 139 2. Support transmission of 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 | [24-1] | Yes | N/A | UL in FR2-2 is not supported | [per UE][per band] | N/A | N/A | N/A |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 [with/without shared spectrum channel access] | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | [24-1a] | Yes | N/A | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 [with/without shared spectrum channel access] | Support multi-RB PUCCH format 0/1/4 for 480 kHz | 24-4a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling This FG is only supported in bands under PSD limitation in shared spectrum operation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 [with/without shared spectrum channel access] | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling This FG is only supported in bands under PSD limitation in shared spectrum operation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH for 120 kHz in FR2-2 [with/without shared spectrum channel access] | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | [24-1a] | Yes | N/A | Wideband PRACH for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | FFS: whether to split this FG for SA and DC | Optional [with/without]capability signalling [Note: This FG is only supported in bands for shared spectrum operation] [A UE that supports 24-2 must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4b | Wideband PRACH for 480 kHz in FR2-2 [with/without shared spectrum channel access] | PRACH with 480KHz and length 571 | 24-4a | Yes | N/A | Wideband PRACH for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | FFS: whether to split this FG for SA and DC [Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH] | Optional with capability signalling [Note: This FG is only supported in bands for shared spectrum operation] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support for DL | 1. 480KH SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with X=4 slots (Xs,Ys) = (4,1) FFS: 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot according to FG 3-5b with set2 = (4, 3) and (7, 3) symbols where set2 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component 6 of FG 3-5b) | 24-1 | Yes | N/A | 480KHz SCS for DL is not supported | [Per UE/band] | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) slots FFS: 3. Multi-PDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 3. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS according to FG 3-5b with set1 = (7, 3) symbols where set1 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 4. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 5. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component 6 of FG 3-5b) | 24-1 | Yes | N/A | 960KHz SCS support for DL is not supported | [Per UE/band] | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4f | Enhanced PDCCH monitoring for 480KHz in FR2-2 | [1.) Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys)=(2,1)] 1.) Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys)=[(4,2)] slots 2.) Within each of the Ys = 2 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot as in according to FG 3-1 (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) | 24-4 | Yes | N/A | Enhanced PDCCH monitoring for 480KHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: [one or more of] {[(2,1),] (4,2) } Note: If (2,1) is not agreed, this FG will have no component candidate values and the component 1 description will be updated from (Xs,Ys) to (Xs,Ys)=(4,2) similar to FG 24-4 and 24-5 FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-5f | Enhanced PDCCH monitoring for 960KHz | 1.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(4,1) 2.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)= (4,2) 1.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,4) slots 2.) Within each of the Ys = 2 or 4 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot as in according to FG 3-1 (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) | 24-5 | Yes | N/A | Enhanced PDCCH monitoring for 960KHz is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: one or more of {(4,1), (4,2), (8,4)} FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |