3GPP TSG RAN WG1 #108-e		                                                                R1-2201305

e-Meeting, February 21st – March 3rd, 2022

Title:	[Draft] Reply LS on PEMAX for NR-V2X

Response to:	R1-2112672 /R4-2120047

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	[RAN1], CATT, GOHIGH

To:	RAN4

Cc:	RAN2

Attachment:	-

Contact Person:

Name:	Rui Zhao

E-mail Address:	zhaorui@gohigh.com.cn

Overall description

RAN1 thanks RAN4 for the LS on PEMAX for NR-V2X, and would provide response to the following question from RAN4.

Issue 1: It is RAN4 understanding that the parameter to limit the transmitted power of PSSCH/PSCCH for V2X can be used in both in-coverage and out-of-coverage. RAN4 would like to check with RAN1 and RAN2 which parameter (sl-maxTxPower, sl-MaxTransPower, SL-TxPower) is the correct one to be used to fulfill the purpose?

RAN1 response:  sl-MaxTransPower is the correct one to indicate PEMAX,c in 38.101

Issue 2: RAN4 also had some discussion on whether the parameter should be associated with or without a serving cell on the NR V2X carrier, and three options are proposed:

- Option 1: The parameter can be associated either with a serving cell or without a serving cell, and it can be configured separately with p-max for Uu

- Option 2: The parameter can be associated either with a serving cell or without a serving cell, when the parameter is associated with a serving cell, PEMAX,c is the smaller value given by this parameter for SL and p-max for Uu of that serving cell.

- Option 3: when UE is associated with a serving cell on the NR V2X carrier, p-max is used for serving cell c; when the UE is not associated with a serving cell on the NR V2X carrier, the parameter given for SL in RAN2 specification is used.

RAN1 response:

Option 1 is aligned with RAN1 common understanding which is same as that in LTE-V2X. With option 1, option 2 can be supported by proper configuration of sl-MaxTransPower, i.e. the value of sl-MaxTransPower is not lager than p-max for Uu.

Actions:

To RAN4, RAN2:

RAN1 respectfully requests RAN4 and RAN2 to take the above responses into account in the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#109-e			16th -27th May 2022	   			E-Meeting

RAN1#110			22th -26th August 2022				Toulouse, France