3GPP TSG RAN WG1 Meeting #108-e								R1-2201312

# e-meeting, February 21st – March 3rd, 2022

Title:	[Draft] Reply LS on the applicability of PRS processing window in RRC_INACTIVE state

Response to:	R1-2200903 (R4-2202687) LS on the applicability of PRS processing window in RRC_INACTIVE state

Release:	Rel-17

Work Item:	NR_pos_enh

Source:	CATT [RAN1]

To:	RAN4

Cc:

Contact Person:

#### Name:	Ren Da

#### E-mail Address:	renda@catt.cn

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 thanks RAN4 for their LS R1-2200903 (R4-2202687) on the applicability of the PRS processing window in the RRC_INACTIVE state.

RAN1 discussed the following question raised in the LS:

Q1: Whether the PRS processing window defined for PRS measurements outside measurement gaps can be also applied for PRS measurements in RRC_INACTIVE state?

For UE positioning in RRC_INACTIVE state, RAN1 has made the following agreement. The reception of DL PRS has lower priority than other DL signals/channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT), which implies the reception of DL PRS during RRC_INACTIVE state should have no impact on the reception of other DL signals/channels.

Agreement: - From RAN1 perspective, in RRC_INACTIVE state, reception of DL PRS has lower priority than other DL signals/channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT) - FFS how to determine conflicts in DL PRS and other DL signals/channels reception by UE - FFS how to handle retuning time for the case when DL PRS and other DL signals/channels are allocated in different BW and/or have the same or different SCS as initial DL BWP - Send LS to RAN4 (cc RAN2) and ask if there is any feedback

Therefore, RAN1 would make the following response to RAN4’s question:

Answer 1: From RAN1’s perspective, there is no need to define the PRS processing window for PRS measurements in the RRC_INACTIVE state. The reception of DL PRS in the RRC_INACTIVE state should have no impact on the measurement requirements of other DL signals/channels. However, the reception of other DL signals/channels may potentially impact the measurement performance requirement of DL PRS if there are conflicts in the receptions of DL PRS and other DL signals/channels for a UE in the RRC_INACTIVE state. It will be up to RAN4 to decide the impact on the measurement performance requirement of DL PRS when the conflicts take place.

2. Actions:

To RAN4:

ACTION: 	RAN1 respectfully asks RAN4 to take the above information into account in their work on Release 17 NR positioning enhancements for RRC_INACTIVE state.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e 		May 16 – 27, 2022		        	Electronic Meeting

TSG-RAN WG1 Meeting #110 		August 22 – August 26, 2022       	Toulouse, FR