3GPP TSG RAN WG1 #108-e                            		R1-2201313

e-Meeting, February 21st – March 3rd, 2022

Source:	CATT

Title:	Discussion on reporting and definition of DL PRS path RSRP

Agenda item:	5

Document for:	Discussion and Decision

Introduction

In the LS (R4-2202780) [1] to RAN1, RAN4 provides following feedback related to DL-PRS RSRPP:

The PRS-RSRPP can be reported in ways below : - RAN4 understands that PRS-RSRPP should be defined as the path RSRP per RE, in the same way PRS-RSRP is defined as power per RE. RAN4 respectfully asks RAN1 to confirm if this understanding is correct. - PRS-RSRPP may be reported by reusing absolute and differential PRS-RSRP measurement report mapping tables in TS38.133 clause 10.1.24.3.1 and 10.1.24.3.2 respectively. - When differential reporting is used, PRS-RSRPP is reported as the difference in dB with respect to a reference measurement. RAN4 understands that it is up to RAN1/2 to decide what reference measurement would be for the PRS-RSRPP differential reporting.

In this contribution, we present our views on the definition and reporting of DL PRS-RSRPP with the consideration of  RAN4’s feedback.

Discussion

Definition of DL PRS-RSRPP

In RAN1#106bis-e [2], the following agreement on PRS-RSRPP has been achieved:

Agreement:

The measured path DL PRS RSRP for ith path delay is defined as the power of the received DL PRS signal configured for the measurement at the ith path delay of the channel response, and

- path DL PRS RSRP for 1st path delay is the power corresponding to the first detected path

- FFS: Whether the path RSRP measurement is normalized with PRS RSRP.

- FFS: Whether the definition of the ith path delay (other than i=1) is required.

- Note: UE may choose to use a time window to compute path DL PRS RSRP by UE implementation (there is no impact to specifications managed by RAN1 for this)

- Note: This does not imply that the path delay has to be reported in DL-AoD positioning

- Send LS to RAN4 to check the details of the definition and feedback if they identify any update is necessary

According to the above agreement, PRS-RSRPP for  ith path delay is defined as the power corresponding to ith detected path. At the UE side, the receiver needs to separate the signal power of each path from the total received signal power. Namely, PRS-RSRPP could be considered as a component of PRS-RSRP. Therefore, PRS-RSRPP should be defined in the same way as that of PRS-RSRP. According to the definition of PRS-RSRP in TS 38.215[3] and RAN4 LS[1], we propose the following TP for the modification of PRS-RSRPP definition:

Proposal-1: Adopt the following TP on the modification of the definition of DL PRS-RSRPP.

----------------Start of TP for TS38.215---------------------

5.1.35	DL PRS reference signal received path power (DL PRS-RSRPP)

| Definition | DL PRS reference signal received path power (DL PRS-RSRPP), is defined as linear average of the power contributions (in[W]) of the received DL PRS signal of the resource elements configured for the measurement at the i-th path delay of the channel response, where DL PRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

----------------End of TP for TS38.215---------------------

Reporting of DL PRS-RSRPP

When differential reporting is used, PRS-RSRPP is reported as the difference in dB with respect to a reference measurement. In our opinion, it is prefer to use the power of the strongest path as the reference measurement. Because the strongest path can be more reliably detected and the resulted differential value would lie in a reasonable range of [0,1], which is beneficial for AI/ML processing [4]. Besides the differential values of PRS-RSRPP, absolute value of this reference measurement should also be reported simultaneously.

Proposal-2: When differential reporting is used, the reference measurement should be the absolute value of PRS-RSRPP of the strongest path.

Conclusions

In this contribution, we discussed the issue raised in RAN4’s LS (R4-2202780). Based on the discussion, we have the following proposals. The corresponding draft response LS is prepared in [5].

Proposal-1: Adopt the following TP on the modification of the definition of DL PRS-RSRPP.

----------------Start of TP for TS38.215---------------------

5.1.35	DL PRS reference signal received path power (DL PRS-RSRPP)

| Definition | DL PRS reference signal received path power (DL PRS-RSRPP), is defined as linear average of the power contributions (in[W]) of the received DL PRS signal of the resource elements configured for the measurement at the i-th path delay of the channel response, where DL PRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

----------------End of TP for TS38.215---------------------

Proposal-2: When differential reporting is used, the reference measurement should be the absolute value of PRS-RSRPP of the strongest path.

References

- R4-2202780, Reply LS on reporting and definition of DL PRS path RSRP, RAN4 (Nokia).

- 3GPP TSG RAN WG1 Meeting #106bis-e, RAN1 Chairman's Notes.

- TS 38.215, “NR; Physical layer measurements”, v17.0.0.

- R1-2201365, Remaining issues on information reporting from UE and gNB for multipath/NLOS mitigation, CATT.

- R1-2201314, Draft reply LS on reporting and definition of DL PRS path RSRP, CATT.