3GPP TSG RAN WG1 #108-e                                              R1-2201360

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.4.3

Source:	CATT

Title:	Remaining issues on HARQ operation enhancement for NTN

Document for:	Discussion and Decision

Introduction

For the enhancements on NTN HARQ, there were some discussions on Disabling/enabling HARQ feedback, transmission enhancement and HARQ feedback enhancement etc. Moreover, a few agreements on HARQ operation were concluded as follows [1] in last RAN1 meeting:

Agreement

The bit-fields related to the HARQ-ACK feedback (i.e., PRI, PUSCH-to-HARQ_feedback timing, DAI) are unchanged for the DCI of PDSCH with feedback-disabled HARQ process in Rel-17 with the same interpretation from UE as for feedback-enabled HARQ process

- Note: The interpretation regarding the DAI for Type-2 codebook is up to the progress of codebook design.

Agreement

For Type-1 HARQ codebook, the UE will consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

Agreement

For Type-3 HARQ codebook in NTN, the UE should skip the codebook feedback for a feedback-disabled HARQ processes

Note: The Type-3 codebook size is reduced by excluding the bit positions of disabled HARQ processes

Agreement

HARQ feedback for SPS activation may be additionally enabled by the network by RRC configuration.

- If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of whether HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation

- Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation,

  - FFS between Alt1 and Alt2

    - [Alt-1: UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process

    - Alt-2: UE follows the feedback-enabled/disabled configuration of the SPS PDSCH]

In this contribution, we further analyze remaining technical issues of NTN HARQ operation and provide our views on these issues.

Discussion

  - Enhancement on HARQ-ACK codebook

Based on last meeting, for SPS PDSCH which is not scheduled by corresponding PDCCH, the proposal is made as follows:

HARQ feedback for SPS activation may be additionally enabled by the network by RRC configuration.

- If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of whether HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation

- Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation,

  - FFS between Alt1 and Alt2

    - [Alt-1: UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process

    - Alt-2: UE follows the feedback-enabled/disabled configuration of the SPS PDSCH]

For SPS service, in general, some companies that support the Alt-2 believe that the service quality should be same for all assoiated PDSCH, per SPS configuration disabling is one good choice. However, with this method, some specific impact will be introduced. Since HARQ feedback disable per HARQ process had been used in dynamic PDSCH, reuse it in SPS PDSCH is also appropriate. Hence, When the higher layer parameter about HARQ feedback for SPS activation is configured disabled, UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process.

- When the higher layer parameter about HARQ feedback for SPS activation is configured disabled, UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process.

PDSCH/PUSCH scheduling restriction

The OOO issue (out of order HARQ feedback scheduling restriction for PDSCHS with different HARQ process IDs) with consideration on the joint existence of feedback-enabled and feedback-disabled HARQ process has been discussed in last meeting. Following views are shared by companies:

- For the case that either one or all of two PDSCHs is associated with feedback-disabled HARQ process

  - The following legacy restriction is applied at least in case that HARQ-ACK feedback is reported for feedback-disabled HARQ process:

    - In a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority.

    - In a given scheduled cell, the UE is not expected to receive a first PDSCH, and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priority.

  - For the PDSCH associated feedback-disabled HARQ process, the determination of the resource assigned for the HARQ-ACK feedback is same as the case that this PDSCH is associated with feedback-enabled HARQ process at least in case that HARQ-ACK feedback is reported.

  - FFS: The HARQ-ACK feedback is true HARQ-ACK or an artificially generated value

Since the OOO issue is a NR feature, which is used for unnecessary waste of resources and power, then it is nature to use it as the baseline with consideration on the joint existence of feedback-enabled and feedback-disabled HARQ process. And when HARQ-ACK feedback needs to be reported for Type-1 codebook generation, the time domain resource assignment and PDSCH-to-HARQ feedback timing indication field are also valid for the PDSCH associated feedback-disabled HARQ process, and the HARQ-ACK feedback is NACK.

- The OOO legacy restriction is applied when feedback-disabled is configured.

Restriction on HARQ feedback disabling

According to agreement above on HARQ feedback disabling at RAN1#102e[2], Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signalling. Some companies proposed that MAC signalling is associated with HARQ-ACK feedback or not depends on gNB scheduling. However, if HARQ feedback of MAC signalling is disabled, the efficiency and reliability will be challenged. For example, if UE doesn’t detect one MAC packet carrying signalling information and will not send acknowledgement to gNB, then the gNB will wait for long time and take next steps, obviously, it is untolerable. Therefore, UE expects that any PDSCH carrying a MAC CE command, whose activation/deactivation time is coupled to the transmission time of the associated HARQ-ACK, is scheduled via a HARQ process with HARQ feedback enabled.

- UE expects that at least one HARQ process with feedback is configured for the scheduling of MAC-CE.

Enhancement on the transmission

Since satellite communication sometimes works in very bad weather conditions, such as heavy rain, some proposals for transmission enhancement were raised in the last meeting.

The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X]

- FFS: X = 8, 16 or 32

For slot aggregation, it is also a NR Rel-15 feature, which is used for enhancement on transmission of downlink and uplink, then it is nature to use it as the baseline to improve the reliability. Regarding the satellite scenario, increasing the repetition number would be important to improve the transmission reliability. As a trade-off between complexity and performance, the aggreation factor 16 would be one reasonable choice.

- Slot-aggregation factor can be extended to 16 for very low SINR case.

Conclusion

In the previous sections, we analyzed potential issues of HARQ impact due to longer propogation delay in NTN. We raised a set of solutions for HARQ operation enhancement:

- When the higher layer parameter about HARQ feedback for SPS activation is configured disabled, UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process.

- The OOO legacy restriction is applied when feedback-disabled is configured.

- UE expects that at least one HARQ process with feedback is configured for the scheduling of MAC-CE.

- Slot aggregation factor can be extended to 16 for very low SINR case.

References

- RAN1 #107-e Chairman notes

- RAN1 #102-e Chairman notes