3GPP TSG RAN WG1 #108			R1- 2201386

Online Meeting, 21st Feb – 03rd Mar 2022

Source:	Panasonic

Title: 	Remaining Issues on Sidelink Resource Allocation for Power Saving

Agenda Item:		8.11.1.1

Document for:	Discussion, Decision

# Introduction

This contribution discusses some remaining issues of resource allocation for power saving on the following aspects:

- DRX Aspects

- Sensing and Resource Selection

# Discussion

## DRX Aspects

In RAN1#107bis e-meeting, the following Proposal 2-1 (latest version VII) has been discussed to resolve the FFS points in the agreement made in RAN1#107-e meeting.

Proposal 2-1 (VII) (RAN1107bis): When SL DRX active time of RX UE is provided by the higher layer for candidate resource selection, · At least the first N slots of Y or Y’ candidate slots are to be selected within the provided SL DRX active time when partial sensing is configured in the UE by higher layer, where N is (pre-)configurable from a range of 1 to [10].. - FFS: whether This does not apply to re-evaluation and pre-emption checking (e.g. number of remaining Y or Y’ candidate slots is less than N) - Note (not to be captured in the spec): it is possible that the whole set of Y or Y’ candidate slots are selected within the provided SL DRX active time (e.g., when the provided SL DRX active time covers the entire remaining PDB) · T2 is selected by UE implementation such that at least the first N slots of RSW are to be selected within the provided SL DRX active time when full sensing is configured in the UE by higher layer, where N is (pre-)configurable from a range of 1 to [10].. - FFS: whether This does not apply to re-evaluation and pre-emption checking - Note (not to be captured in the spec): it is possible that the whole RSW is within the provided SL DRX active time (e.g., when the provided SL DRX active time covers the entire remaining PDB) · The same (pre-)configured N parameter applies to both partial sensing and full sensing cases. · The reported subset of the candidate resources within the provided SL DRX active time of RX UE shall satisfy a threshold.. - Option 1: The same higher layer parameter (sl-TxPercentageList) is reused for the ratio / threshold That is, number of candidate single-slot resources remaining within the SL DRX active time of the initialized set in Step 4) is to be met by using the RSRP threshold increment only for the SL DRX active time in Step 7, where the is the total number of candidate single-slot resources of the set within the SL DRX active time. - The UE shall satisfy this new threshold in addition to the remaining minimum number of candidate single-slot resources threshold for the whole set in Step 7. Agreement (RAN1#107) When SL DRX active time of Rx-UE is provided by the higher layer for candidate resource selection (including resource (re)selection and re-evaluation/pre-emption checking), the following working assumption is confirmed with option 2 as agreement (with modification in RED) Working Assumption (RAN1#106bis-e) When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer: - Option 1: PHY layer selects and reports candidate resources only within the indicated active time of the RX UE - Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE - FFS: Details on when the number of subsets of candidate resource is less than the threshold - FFS: The subset of candidate resource outside of the active time should consider each inactive time period - FFS: UE selection of resource selection window to overlap with indicated RX UE active time - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE Option 3: PHY layer selects and reports an additional candidate resource set of candidate resources within the indicated active time of the RX UE

Regarding to the candidate resources within TX UE’s resource selection window, we think they could be at least classified by considering the following two perspectives:

- Whether within the indicated RX UE(s)’ active time

- Whether satisfy the requirements (RSRP threshold, pre-emption, etc.) in legacy procedure

In our opinion, the candidate resources satisfying the legacy requirement and also within the indicated RX UE(s)’ active time should be considered as some kind of “first candidate resources” and included into the subset to be reported to higher layer. When the first candidate resources is lower than the threshold of X% of SA as required in release 16, it should be specified for a UE whether to only report the current subset of resources, or to further include additional resources.

For the additional resources, a UE may perform RSRP threshold increment within indicated RX UE(s)’ active time, or to include resources outside the indicated RX UE(s)’ active time. The former one would have less chance of transmission failure, while the latter one would have less noisy resources.

Proposal 1: It should be specified whether and what resources to be further included when the subset of resources is lower than threshold.

Furthermore, keep increasing the RSRP threshold for the candidate resources within indicated RX UE(s)’ active time will include noisy resources when the RSRP threshold is high. To avoid it, we propose that a maximum RSRP threshold or a maximum number of iterations should be applied for the candidate resources within the indicated RX UE(s)’ active time.  Up to configured value, RSRP threshold is increased within Rx UE's active time. If not enough, outside of Rx UE's active time should be used as the second step.

Proposal 2: Up to configured value, RSRP threshold is increased within Rx UE's active time. If not enough, outside of Rx UE's active time should be used as the second step.

Relation between Uu DRX and sidelink DRX

The Uu DRX features with LTE and NR have been specified. For sidelink UEs without Uu links, there would be no other choice than the sidelink DRX would be (pre)configured by regulator or sidelink operators. However, for sidelink UEs with Uu connection, it is not clear whether the sidelink DRX feature should be controlled by gNB.

If the sidelink DRX is controlled by gNB using Uu DRX operation, a dummy PDCCH and/or using drx-inactivity-timer would be required on Uu links to wake-up the UE for sidelink operations. It can be very complex as the Uu scheduler at gNB needs to know SL resource pool status, and the dummy PDCCH is redundant especially when sidelink is operated in shared frequency without involving the operators.

Therefore, in order to have some predictable UE behaviour, we think the sidelink DRX should be specified independently from Uu operation. For out of coverage, the UE would follow only such (pre-)configuration for wake-up. For UE in coverage or partial coverage, in addition to the (pre-)configuration, UE would further wake-up based on the additional Uu DRX operation. A sidelink UE's actual "off" is when both Uu and sidelink operations are "off" period. The ideal (pre)configuration from power saving is Uu DRX operation and SL DRX operation are aligned as much as possible.

Proposal 3: Uu DRX function is independent indication from sidelink sensing/resource allocation timing. On the other hand, a sidelink UE's actual "off" is when both Uu and sidelink operation (sensing/resource allocation timing and SL transmission) are “off”.

The ideal operation from power saving is Uu DRX operation and SL DRX operation are aligned as much as possible. For Uu DRX to be aligned based on sidelink activity/configuration, some mechanism to inform it to gNB is required as gNB may not know sidelink configuration. The gNB would request the sidelink UEs to SL DRX from the SL UEs to ensure certain power saving gain (could for either mode 1 or 2). Rel.16 specs (specifically, TS38.331 section 5.7.4) already supports for the UE to inform the network of its preference on DRX parameters for power saving. The sidelink UE can take sidelink information into account for UE assistance information, for better coordination.

Proposal 4: The sidelink UE can take sidelink information (including the sensing/resource allocation timing) into account for the UE assistance information for network to inform the gNB for a better coordination with Uu at the network.

## Sensing and Resource Selection

Random Resource Selection

The following was agreed in RAN1#106 and long discussion has been carried on during past meetings for the potential proposal. The Proposal 9 (IV) was the latest version discussed in last RAN1#107bis-e meeting.

Agreement: (RAN1#106) For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e - Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed - Note, lower value means higher priority - FFS whether resource pool partitioning can be additionally applied - Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement, - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement. - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking - Option 12: No special consideration Proposal 9 (IV) (RAN1#107bis): When UE performs random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, - A priority threshold value is (pre-)configured for the resource pool, below or equal to which random resource selection is allowed. Note, lower value means higher priority. The (pre-)configured priority threshold can be any of the 8 priority values.

In our understanding, the probability of potential in-air collision for random resource selection from full/partial sensing needs to be reduced (but not fully eliminated). It would be beneficial to segregate random resource selection from full/partial sensing. Associating priorities with resource pools or sensing type some would be easy achievable solutions and with less spec impact. We’d like to support option 1 that a priority threshold value is (pre-)configured for the resource pool.

Some discussions were carried whether to have “resource pool partitioning” or just to have “separated pool”. Our preference is to have the “partitioning” as realization of subset of resources to prioritize the random resource selection instead of sperate pool” as it does not increase the number of pools. In our understanding, the further segregation by partition a portion on top of priority would have better protection to avoid potential collisions. Also, Rx pool can be common while Tx pool can be separate pool configuration, which may introduce issue on inter-UE coordination since the different size between Tx and Rx pools. We are also ok to compromise without the “resource pool partitioning”

Proposal 5: The random resource selection needs to be segregated from full/partial sensing from resource selection procedures.  We’d like to support option 1 that a priority threshold value is (pre-)configured for the resource pool. We prefer to have the resource pool partitioning, although we are also ok to compromise without the resource pool partitioning.

Resource Pool Aspects

In RAN1#105-e meeting, the following was agreed on FFS the impact of resource collision.

Agreement: - For random resource selection, - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation. - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any) - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected. - FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions). - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

A partial-sensing UE, (e.g., a VRU-UE), should not be required to waked up always like a full-sensing UE (e.g., a vehicular UE), thus it would be desirable to improve the transmission/reception successful rate during their DRX active times. A dedicated resource pool could be configured for the partial-sensing UEs to at least eliminate the collision between partial-sensing UEs’ transmissions and full-sensing UEs’ transmissions.

The following characteristics are considered for the resource pool:

- The partial-sensing UEs’ Tx pool should not overlap with full-sensing UEs’ Tx pool;

- The partial-sensing UEs’ Tx pool should be within full-sensing UEs’ Rx pool;

- The full-sensing UEs should transmit partial-sensing UE targeted SL messages in the partial sensing UEs’ Rx pool.

Proposal 6: A dedicated resource pool should be allowed for partial sensing UEs that the Tx pool not overlap with full sensing UEs’ Tx pool and within full sensing UEs’ Rx pool; while the Rx pool is known to full-sensing UEs for partial-sensing UE targeted SL messages

# Conclusion

In this contribution, followings observation and proposals are made:

Proposal 1: It should be specified whether and what resources to be further included when the subset of resources is lower than threshold.

Proposal 2: Up to configured value, RSRP threshold is increased within Rx UE's active time. If not enough, outside of Rx UE's active time should be used as the second step.

Proposal 3: Uu DRX function is independent indication from sidelink sensing/resource allocation timing. On the other hand, a sidelink UE's actual "off" is when both Uu and sidelink operation (sensing/resource allocation timing and SL transmission) are “off”.

Proposal 4: The sidelink UE can take sidelink information (including the sensing/resource allocation timing) into account for the UE assistance information for network to inform the gNB for a better coordination with Uu at the network.

Proposal 5: The random resource selection needs to be segregated from full/partial sensing from resource selection procedures.  We’d like to support option 1 that a priority threshold value is (pre-)configured for the resource pool. We prefer to have the resource pool partitioning, although we are also ok to compromise without the resource pool partitioning.

Proposal 6: A dedicated resource pool should be allowed for partial sensing UEs that the Tx pool not overlap with full sensing UEs’ Tx pool and within full sensing UEs’ Rx pool; while the Rx pool is known to full-sensing UEs for partial-sensing UE targeted SL messages

# Reference

[1] RP-202846, “WID revision: NR sidelink enhancement,” LG Electronics, RAN#90-e

[2] R1-2200024, Remaining Issues on Sidelink Resource Allocation for Power Saving, Panasonic, RAN1#107bis-e

[3] R1-2200724, FL summary for AI 8.11.1.1 – resource allocation for power saving (EOM), Moderator (OPPO), RAN1#107bis-e

[4] Final Report of 3GPP TSG RAN WG1 #106-e v3.0.0, Online meeting, 16th – 27th August 2021

[5] Final Report of 3GPP TSG RAN WG1 #106bis-e v3.0.0, Online meeting, 11th – 20th October 2021

[6] Final Report of 3GPP TSG RAN WG1 #107-e v1.0.0, Online meeting, 11th – 19th November 2021

[7] Draft Report of 3GPP TSG RAN WG1 #107bis-e v0.2.0, Online meeting, 17th – 25th January 2022