3GPP TSG RAN WG1 #108-e	R1-2201392

e-Meeting, February 21th - March 3th, 2022

Title:	Remaining issues on the PDSCH/PUSCH enhancements for 52.6 to 71GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.5

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining issues on PUSCH/PDSCH enhancements for Rel-17 above 52.6GHz.

Discussion

Multi-PDSCH/PUSCH scheduling

In RAN1 #107bis e-meeting [1], the following case on DCI-to-data OOO was discussed but there is no consensus:

- Proposal #2.2-1a (DCI-to-data OOO):

- The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but these two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE.

  - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI.

For the rule DCI-to-data OOO in the Clause 6.1 of TS 38.214 as follows, OOO scheduling describes based on “PUSCH transmission”. Further, in the Clause 9 of TS 38.213 as follows, we can observe that the PUSCH transmission includes PUSCH transmission over the multiple slots. Related description is copied below:

Clause 6.1 of TS 38.214 For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. Clause 9 of TS 38.213 If the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information.

Based on the above specification description, we do not think that the current spec supports the OOO scheduling which two multi-slot PUSCH scheduling DCIs end in the different symbols but these two multi-slot PUSCH scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first repetition till the end of the last repetition of multi-slot PUSCH.

For this case, from our point of view, we think that it seems to have never been discussed in the previous other topics, only raised on Rel-17 above 52.6GHz. Even if it has been discussed before, it should not be regarded as a non out-of-order case. Further, considering the limited time in the maintenance stage, we do not expect to spend more time discussing a way that may have an impact on the existing spec. Consequently, we propose a simply and safe way to deal with this case, that is, the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but these two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans is considered as out-of-order scheduling and is not expected by UE.

Proposal 1: It is recommended that the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but these two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans is considered as out-of-order scheduling and is not expected by UE.

Clarification on “scheduled PXSCH”

In the email discussion of RAN1 #107b-e, clarification on whether “scheduled PXSCH” in previous agreements implies valid PXSCH or not was discussed [1] and some clarifications was listed as follow:

- For CSI-request, whether the number of M is determined based on the number of configured SLIVs or valid SLIVs?

- For out-of-order scheduling, whether the rule for OOO scheduling is determined based on configured SLIVs or valid SLIVs?

For A-CSI request, gNB should guarantee that the assigned PUSCH carrying the A-CSI is valid. Otherwise, the A-CSI report should be dropped. For out-of-order scheduling, only valid PDSCH(s)/PUSCH(s) which may actually schedule by gNB should be considered.

Proposal 2: Unnecessary optimization should not be introduced for “scheduled PXSCH”.

- gNB should guarantee the assigned PUSCH carrying the A-CSI is valid.

- Only valid PXSCH should be considered in out-of-order scheduling.

In RAN1 #107 e-meeting, there is a remaining issue about [configured or valid] SLIV to be resolved as follow:

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell.

  - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell.

  - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell.

  - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method.

    - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured)

    - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case

  - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook.

  - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook,

    - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

The pros and cons of the configured and valid SLIV have been fully discussed in previous meetings. In some cases, valid SLIV can achieve better re-transmission efficiency and HARQ-ACK feedback efficiency. However, configured SLIV can simplify the UE implementation. Considering the limited time left in this stage, for type-2 HARQ-ACK codebook with multi-PDSCH scheduling and HARQ-ACK time domain bundling, our preference is configured SLIV which is simple and valid.

Proposal 3: Configured SLIV should be support for type-2 HARQ-ACK codebook with multi-PDSCH scheduling and HARQ-ACK time domain bundling.

Type-2 CB PUCCH power control

If the UCI payload OACK+OSR+OCSI≤11, the UE determines a PUCCH transmission power based on nHARQ−ACK. However, the  Nm,creceived in nHARQ-ACK,TB formula does not consider the case multi-PDSCH scheduling by single DCI with HARQ-ACK time domain bundling. Therefore, the calculation of UCI payload should be modified base on the configuration of multi-PDSCH scheduling and/or numberOfHARQ-BundlingGroups. Wherein, the wording of corresponding text proposals can be further discussed.

Proposal 4: For type 2 HARQ-ACK codebook, multi-PDSCH scheduling by single DCI and HARQ-ACK time domain bundling should be considered to determine a PUCCH transmission power when the UCI payload OACK+OSR+OCSI≤11.

TP for Type-1 HARQ-ACK codebook

In order to keep the unified wording used between different specs or paragraphs on the following yellow highlight part, one editorial change seems to be needed. That is, “more than one entry” should be revised as “more than one SLIV entry” as shown in the following TP #1.

TS 38.213 section 9.1.2.1 if enableTimeDomainHARQ-Bundling is provided for serving cell c and a PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry ... - if enableTimeDomainHARQ-Bundling is provided for serving cell c and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers as received only a PDSCH associated with the last SLIV

TP #1

< Start of text proposal of 38.213>

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

< Unchanged parts are omitted >

If the set of rows R includes  a row with more than one SLIV entry as described in [6, TS 38.214] and enableTimeDomainHARQ-Bundling is not provided, the set of rows R and the set of slot timing values K1 are updated in this clause according to the following pseudo-code.

< End of text proposal of 38.213>

Proposal 5: Adopt TP #1 in Tdoc R1-2201392.

Conclusion

In this contribution, we discussed the remaining issues on PUSCH/PDSCH enhancement, including HARQ-ACK transmission, HARQ-ACK codebook and multiple PXSCH scheduling. Based on this, the following observations and proposals are provided:

Proposal 1: It is recommended that the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but these two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans is considered as out-of-order scheduling and is not expected by UE.

Proposal 2: Unnecessary optimization should not be introduced for “scheduled PXSCH”.

- gNB should guarantee the assigned PUSCH carrying the A-CSI is valid.

- Only valid PXSCH should be considered in out-of-order scheduling.

Proposal 3: Configured SLIV should be support for type-2 HARQ-ACK codebook with multi-PDSCH scheduling and HARQ-ACK time domain bundling.

Proposal 4: For type 2 HARQ-ACK codebook, multi-PDSCH scheduling by single DCI and HARQ-ACK time domain bundling should be considered to determine a PUCCH transmission power when the UCI payload OACK+OSR+OCSI≤11.

Proposal 5: Adopt TP #1 in Tdoc R1-2201392.

Reference

- R1-2200569, Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)