3GPP TSG RAN WG1 #108-e	R1-2201394

e-Meeting, February 21th - March 3rd, 2022

Title: 	Discussion on RRC parameters for 52.6 to 71 GHz

Source: 	ZTE, Sanechips

Agenda item:	8.2.7

Document for:	Discussion and Decision

Introduction

In this contribution, we will share our views on RRC parameters for 52.6 to 71 GHz. Wherein, the latest approved parameters list can be found in [1].

Discussion

channelAccessMode2-r17

According to Running RRC CR for 71GHz from RAN2 [2], we can observe that LBT mode is determined as per Cell. Specific IEs are copied as below:

ServingCellConfig ::=               SEQUENCE {

......

channelAccessConfig-r16             SetupRelease { ChannelAccessConfig-r16 }    OPTIONAL    -- Need M

]],

[[

-- FFS if this could e.g. be ENUMERATED {enabled, disabled} or only enabled

channelAccessMode2-r17              ENUMERATED {FFS}                  OPTIONAL    -- Need R

]]

}

ServingCellConfigCommon ::=         SEQUENCE {

.....

]],

[[

-- FFS: whether this could e.g. be ENUMERATED {enabled, disabled} or only enabled

channelAccessMode2-r17              ENUMERATED {FFS}                                                    OPTIONAL, -- Cond FR2-2

discoveryBurstWindowLength-r17      ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25}    OPTIONAL -- Need R

]]

}

ServingCellConfigCommonSIB ::=      SEQUENCE {

]],

[[

-- FFS: whether this could e.g. be ENUMERATED {enabled, disabled} or only enabled

channelAccessMode2-r17              ENUMERATED {FFS}                                            OPTIONAL, -- Cond FR2-2

discoveryBurstWindowLength-r17      ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25}    OPTIONAL -- Need R

]]

}

Besides, for the default value of the parameter “channelAccessMode2-r17”, we think that LBT and No LBT should be configured, not only for No LBT configuration. Wherein, in value range column, LBT mode is represented by “enabled”, while No LBT mode is represented by “disabled”. Further, the parameter “channelAccessMode2-r17” is an optional property. If it is not configured, we need to further consider what the default LBT type is.

Table 1 The updated RRC parameters list on LBT mode and LBT mode

| Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel access |  |  | FFS. For cell-specific configuration ServingCellConfigCommon, ServingCellConfigCommonSIB |  | LBT-Mode channelAccessMode2-r17 | New |  | To switch between LBT and no-LBT mode | FFS ENUMERATERD {enabled, disabled} Note: enabled represents LBT mode; Disabled represents NO LBT mode |  | Per cell | Cell-specific |  | Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode • Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication Conclusion (RAN1 #106bis e-meeting): There is no consensus to support per beam LBT mode or no-LBT mode UE specific gNB indication. |
| Channel access |  |  | FFS. For UE-specific configuration ServingCellConfig, ServingCellConfigCommonSIB |  | LBT-Mode channelAccessMode2-r17 | New |  | To switch between LBT and no-LBT mode | FFS ENUMERATERD {enabled, disabled} Note: enabled represents LBT mode; Disabled represents NO LBT mode |  | Per cell | UE-specific |  | Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode • Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication Conclusion (RAN1 #106bis e-meeting): There is no consensus to support per beam LBT mode or no-LBT mode UE specific gNB indication. |

Proposal 1: Adopt the above updated RRC parameters list on LBT mode and LBT mode for Rel-17 above 52.6GHz.

duration-r17

According to the agreements [3] made in RAN1#107bis e-meeting, the new parameter duration-r17 was introduced for search space set configuration of multi-slot PDCCH monitoring. Even though there exists an FFS about the definition of duration, in our understanding companies already reached a consensus on whether or not introducing this parameter and it is unreasonable to exclude duration-17 in the consolidated higher layers parameter list [1]. Therefore, we suggest adding the parameter duration-r17 to RRC parameter list for Rel-17 above 52.6GHz as follows:

| Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDCCH |  |  |  |  | duration-r17 | New |  | FFS: need to revise the definition of duration | {8, 12, …, 20476} |  | SearchSpace | UE-specific |  | For search space set configuration of multi-slot PDCCH monitoring: • monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and o For monitoringPeriodicityAndOffset-r17  The values represent slots  Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset • Note: Total list of supported periodicity values: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480}  For each periodicity value Xp • The value range for the offset O is {0 .. Xp-1} slots • Note: There may be no need to introduce the term "Xp" in the specifications  The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots  FFS: details of offset o For duration-r17  The values represent slots  The value range is { 8, 12, …, 20476}  The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs  FFS: need to revise the definition of duration • monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring o Note: This parameter can be directly re-used from earlier releases. • Introduce new parameter monitoringSlotsWithinSlotGroup-r17 o Working assumption:  The size is 8 bits  Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group  A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' • Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot  The slots indicated in the bitmap should be consecutive at least for Group (1) SSs |

Proposal 2: Add the parameter duration-r17 to RRC parameter list for Rel-17 above 52.6GHz.

Conclusion

In this contribution, we share some our views on RRC parameters for 52.6 GHz to 71GHz and provide the following proposals:

Proposal 1: Adopt the above updated RRC parameters list on LBT mode and LBT mode for Rel-17 above 52.6GHz.

Proposal 2: Add the parameter duration-r17 to RRC parameter list for Rel-17 above 52.6GHz.

Reference

R1-2200699, Consolidated higher layers parameter list for Rel-17 NR

R2-2200006, Running RRC CR for 71 GHz, Ericsson

RAN1 #107bis-e chairman’s notes