3GPP TSG RAN WG1 #108	R1-2201419

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		8.16.12

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR MBS

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined for selected topics [1, 2]. In this contribution we present our views on the UE features for NR MBS.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107bis are available in the Annex.

- 33-1:

  - Confirm it is per UE. Further discussion needed on whether there is a need for FR1/FR2 differentiation

  - Optional with capability signalling. The network needs to know if any UE supports the feature. Alternatively, it can be left for RAN2 to decide on this aspect, as currently done for some UE power saving features (e.g. FG 29-1).

  - Since this is a broadcast feature, it is preferred to have the DCI indicated slot-level repetition up to [8 or 16] for MTCH as a component to this FG, rather than a separate one.

  - Per UE

- 33-2:

  - Add 33-1 as pre-requisite, it is unclear why a UE would support multicast but not broadcast.

  - No need to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast

  - Merge 33-4 into this one

  - Per UE

- 33-2-x:

  - Confirm FG and provide a proper FG numbering

  - Per UE

- 33-3-2:

  - If CONNECTED_MODE UEs can receive broadcast without supporting multicast, component 1 may need to be separated from 2/3. More clarification needed here.

  - Per UE

- 33-3-3:

  - Same comment as for 33-3-2.

  - Per UE

- 33-4:

  - To be merged in 33-2, or at least to be mandatorily indicated for UEs supporting 33-2, as it is an essencial functionality for groupcast.

  - Per UE

- 33-5-1:

  - Per UE

- 33-5-2:

  - Per UE

- 33-6-1:

  - Per UE

- 33-6-2:

  - Per UE

- 33-6-3:

  - Per UE

- 33-7:

  - Per UE

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR MBS, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107bis-e, Jan. 2022

- R1-2200780, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), Jan. 2022

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH FFS DCI indicated slot-level repetition up to [8 or 16] for MTCH is defined as another FG |  | Yes |  |  | Per UE | No | No |  |  | Optional without capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicastFFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast |  | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling At least 33-2c or 33-2d is merged, FFS which one or both | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2c | PTM retransmission for multicast | Support of PTM retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | TBD | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot | 33-2b | Yes |  |  | Per FSPC | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-4 | Yes |  |  | Per UE |  |  |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast - Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | Per UE | No | No |  | FFS: Candidate values for M | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |