3GPP TSG RAN WG1 #108	R1-2201420

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		8.16.13

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR DSS and LTE NR DC

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined for selected topics [1, 2]. In this contribution we present our views on the UE features for NR DSS and LTE NR DC.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107bis are available in the Annex.

- 34-2:

[Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell  (Type B)]

- Cross-carrier scheduling from sSCell to PCell/PSCell with CIF

- sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell

- Configuration of scaling factor α  for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell

- FFS: #unicast DCI limits for PCell/PSCell scheduling

- Processing one unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s)

- Processing one unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s)

- N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60)

- Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS

- FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell

- FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured

- FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured

- FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot

- FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling)

- FFS: frame boundary alignment between PCell/PSCell and sSCell

- FFS: Precoder granularity of REG-bundle size when CCS from sSCell to PCell/PSCell is configured

- The same components as with Rel-15 cross-carrier scheduling and Rel-16 cross-carrier scheduling with different SCS should be automatically included by corresponding FGs, if supported by the UE, and the ability for the PCell to be the scheduled cell does not impact those functionalities. Hence, this component is redundant and should be removed.

- The USS set configuration for CCS to PCell must be supported for the feature to make sense, this component can be confirmed

- The SCell activation/deactivation support is a separate mandatory feature for UEs supporting CA already and the cell’s ability to schedule a particular other cell does not impact this feature. This component can be removed.

- The SCell dormancy support is a separate optional feature (see FGs 18-4/4a) and the cell’s ability to schedule a particular other cell does not impact this feature. This component can be removed as there are separate FGs for this already.

- This is a minimum support as defined for Rel-15 UEs. The component is redundant and can be removed.

- The fact that the scheduled cell is a PCell should not have any impact to this. The component should be removed.

- This is a basic requirement, could be confirmed or removed as redundant.

- The precoder granularity support has no relation to whether the cross-carrier scheduled cell with the PDCCH happens to be a PCell. This component should be removed.

- 34-1:

  - The same proposals as for FG34-2 apply here.

- 35-1:

  - Type can be confirmed as “per UE”.

  - Need for FR1/FR2 differentiation should be ”yes”, since testability needs have shown that in cases like this FR differentiation has been needed. It is OK include TDD/FDD and FR1 unlicensed differentiation as well.

  - Confirm the note stating that The NZP-CSI-RS configured as temporary RS for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of FG2-33.

The RAN2 LS to RAN1 in R1-2200890 LS on RAN2 agreements for TRS-based Scell activation provides the draft RRC and MAC CRs with value ranges for the feature and allows RAN1 to close the FFS points on components 5 to 9 in FG35-1. MAC-specification provides a 8-bit TRS-ID per SCell that is being activated and the maxNrofSCellActRS-r17    that can be configured is set to 255. That is, there are 256 triggering states (or rather SCellActivationRS-Configs) per SCell, of which zero-value is reserved to “no TRS” and range 1…255 can point to a maximum of 255 configured SCellActivationRS-Configs.

CSI-MeasConfig ::= SEQUENCE { [-clip-clip-clip-] [[ scellActivationRS-ConfigToAddModList-r17 SEQUENCE (SIZE ((1.. maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17 OPTIONAL, -- Need N scellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE ((1.. maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL -- Need N ]] **************************** SCellActivationRS-Config-r17 ::= SEQUENCE { scellActivationRS-Id-r17 SCellActivationRS-ConfigId-r17, resourceSet-r17 NZP-CSI-RS-ResourceSetID, gapBetweenBursts-r17 INTEGER (2..31) OPTIONAL, -- Need R qcl-Info-r17 SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId, ... } **************************** SCellActivationRS-ConfigId-r17 ::= INTEGER (1.. maxNrofSCellActRS-r17) **************************** maxNrofSCellActRS-r17 INTEGER ::= 255 -- Max number of RS configurations per SCell for SCell activation

- FFS: Maximum number of temporary RS resource sets that can be configured to UE per CC {1 … 16}

- FFS: Maximum number of temporary RS resource sets that can be configured to UE across CCs {1 … 256}

- FFS: Maximum number of triggering states for temporary RS based Scell activation by a MAC-CE {1 … 64}

- FFS: Maximum number of temporary RS resource sets that can be associated with a triggering state {1 … 16}

- FFS: Support of temporary RS based SCell activation on one or more from {FR1 FDD, FR1 TDD, FR1 unlicensed, FR2}

5) This value should be set to 16.

6) This value should be component 5 x # of SCells supported, that is, 16 x # of SCells.

7) This value should be 256, of which zero-value means no TRS triggered in this SCell and 255 SCellActivationRS-ConfigIDs each pointing to an SCellActivationRS-Config can be configured.

8) This component is obsolete and can be removed.

9) This is not a component as such, but carrier-type differentiation as commented above, and can be removed.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR DSS and LTE NR DC, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107bis-e, Jan. 2022

- R1-2200780, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), Jan. 2022

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | [Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B)] - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - FFS: #unicast DCI limits for PCell/PSCell scheduling - Processing one unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing one unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of REG-bundle size when CCS from sSCell to PCell/PSCell is configured Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5 [, 34-1] | Yes |  |  | Per BC | No | Applicable to FR1 only |  | [Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60) for N=4, {30,30}, {30,60},{60,60}) Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Note: The CCS from sSCell to Pcell is applicable to FR1 only but there can be other Scells in FR2 configured for the UE | Optional with capability signalling |
| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell [with search space restrictions] (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell [with search space restrictions] (Type A) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - FFS: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters and UE reporting of any associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling - Processing one unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing one unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 configured on sSCell for CCS from sSCell to Pcell/PSCell - FFS: sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of Pcell/PSCell and sSCell. FFS overlap handling - FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s) - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) per BWP are 1 and 3, respectively - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of REG-bundle size when CCS from sSCell to PCell/PSCell is configured | 6-5 | Yes |  |  | Per BC | No | Applicable to FR1 only |  | [Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60) for N=4, {30,30}, {30,60},{60,60}) Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Note: The CCS from sSCell to PCell is applicable to FR1 only but there can be other SCells in FR2 configured for the UE | Optional with capability signalling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. LTE_NR_DC_enh2 | 35-1 | TRS RS for SCell activation | - TRS for SCell activation is aperiodic and triggered by MAC CE - Temporary RS is based on aperiodic TRS - Temporary RS is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the sSCell - A P-TRS of the to-be-activated Scell is indicated as a QCL source for the temporary RS in case of known Scell same as existing specification - FFS: Maximum number of temporary RS resource sets that can be configured to UE per CC {1 … 16} - FFS: Maximum number of temporary RS resource sets that can be configured to UE across CCs {1 … 256} - FFS: Maximum number of triggering states for temporary RS based Scell activation by a MAC-CE {1 … 64} - FFS: Maximum number of temporary RS resource sets that can be associated with a triggering state {1 … 16} - FFS: Support of temporary RS based SCell activation on one or more from {FR1 FDD, FR1 TDD, FR1 unlicensed, FR2} [Note: following are reported via the legacy feature, FG2-33 - Maximum number of configured NZP-CSI-RS resources per CC - Maximum total number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Maximum number simultaneous NZP-CSI-RS resources per CC - Maximum total number of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs] | 6-5 | Yes | N/A |  | [Per UE/Per BC/Per band] | [No/Yes] | [No/Yes] |  | [The NZP-CSI-RS configured as temporary RS for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of FG2-33] | Optional with capability signalling |