3GPP TSG RAN WG1 #108-e	R1-2201437

e-Meeting, February 21st – March 3rd, 2022

Source:	Fujitsu

Title:	Discussion on partial sensing and DRX in NR Sidelink

Agenda item:	8.11.1.1

Document for:	Discussion and Decision

Introduction

During the RAN1#107b-e meeting, the progress has been made for both partial sensing and DRX. In this contribution, we continue to discuss some remaining issues. More specifically, the discussion includes the determination of Y candidate slots for periodic-based partial sensing, the update of Tscal for the resource selection in partial sensing and the resource selection procedures under SL DRX.

Discussion on partial sensing schemes

Remaining details on periodic-based partial sensing

The main purpose of periodic-based partial sensing design is to eliminate interference from other UEs which performing periodic transmission, this is like the mechanism in LTE V2X. Thus, it is agreed that the LTE V2X partial sensing mechanism should be a baseline for periodic-based partial sensing design in Rel-17 sidelink, which can reduce the specification efforts. The enhancement work may only need to be deployed when some features mentioned above cannot be supported by the existing mechanism.

In Rel-14 V2X, the maximum HARQ transmission number is 2, whereas in Rel-16 V2X, this number has been extended to 32. If Rel-17 P-UEs also want to support such a larger HARQ transmission number, the issue on partial sensing may arise that, the number of determined candidate slots in the selection window should ensure the target HARQ transmission number. Even though the HARQ transmission number is not noticed to physical layer explicitly, some implicit mapping rule can be made b/w this number and priority value, and/or CBR range, based on implementation. Therefore, we propose that the UE determine the minimum value for Y based on the priority and/or CBR value, from a range of values (pre-)configured in a resource pool.

When performing periodic-based partial sensing, UE should determine the minimum value of “Y” based on the priority and/or CBR value, from a range of values (pre-)configured in a resource pool.

Furthermore, when PSFCH is configured for the resource pool, a P-UE should also ensure the HARQ RTT related minimum time gap between any two selected resources of a TB. This timing restriction may have some impacts on the candidate slots determined in the partial sensing procedure. As illustrated in Figure 1, the P-UE selects 9 consecutive slots as its candidate slots. Since a HARQ RTT minimum time gap needs to be guaranteed for HARQ-ACK feedback, some slots (the slots colored as yellow) are not usable. In this case, the actual number of candidate slots is reduced to nearly half in comparison with the original determined number. This results in the number of candidate resources to be limited to a half, accordingly, after the step-1 in the resource selection procedure. As a result, the reliability performance may be degraded due to the limited number of candidate resources. A simple solution to address this issue is to determine a set of candidate slots for each transmission opportunity, with a minimum time gap preserved between any two sets of candidate slots. Note that, the minimum time gap is larger than or equal to the HARQ RTT minimum time gap.

Figure 1: An example of a reduction in candidate slots due to HARQ RTT relate timing restriction.

When PSFCH is configured for the resource pool, the impact of the HARQ RTT related timing restriction should also be considered when UE determines the “Y” candidate slots.

  - Remaining details on resource selection

In the RAN1#107b meeting, the following proposal has been discussed but not agreed [1].

Proposal 11: In Step 6 c) of TS38.214 Section 8.1.4, adopt the following changes: - t'n'SL=ty0SL-(Tproc,0SL+Tproc,1SL) if slot ty0SL-(Tproc,0SL+Tproc,1SL) belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise, slot t'n'SL’ is the first slot after slot ty0SL-(Tproc,0SL+Tproc,1SL) belonging to the set t'0SL,t'1SL,...,t'T'max-1SL. - Down-select between: - Alt. 1: Tscal=n+T2-(ty0SL-(Tproc,0SL+Tproc,1SL)) converted to milliseconds - Alt. 2: Tscal=tyLSL-(ty0SL-(Tproc,0SL+Tproc,1SL)) converted to milliseconds, where slot tyLSL is the last slot of the Y or Y’ candidate slots

According to the proposal, there are two alternatives for determining Tscal. In our view, Alt. 2 should be supported. For partial sensing, slot ty0SL-(Tproc,0SL+Tproc,1SL) is the last slot where a UE can perform sensing, and slot tyLSL is the last slot where the UE can perform resource selection. Here, ty0SL and tyLSL are the first slot and the last slot of the Y or Y’ candidate slots respectively. Then, it would be sufficient to just avoid collisions within the time interval between slot ty0SL-(Tproc,0SL+Tproc,1SL) and slot tyLSL no matter whether partial sensing slots are before slot n or not. To this end, Tscal should be equal to the aforementioned time interval, i.e., Tscal=tyLSL-(ty0SL-(Tproc,0SL+Tproc,1SL)). An example is provided in Figure 2 to show Tscal for both full sensing and partial sensing. For partial sensing, it is assumed that the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, and n+TB=ty0SL-(Tproc,0SL+Tproc,1SL).

Figure 2: An example for updating Tscal.

- For partial sensing, the parameter Tscal is updated as tyLSL-(ty0SL-(Tproc,0SL+Tproc,1SL)), where slot ty0SL is the first slot of the Y or Y’ slots and slot tyLSL is the last slot of the Y or Y’ slots.

Discussion on the impact of SL DRX

The impact of SL DRX on resource selection

In the RAN1#107b meeting, the following proposal has been discussed on how to select and report a subset of resources within the provided SL DRX active time [1].

Proposal 2-1 (VII): When SL DRX active time of RX UE is provided by the higher layer for candidate resource selection, · At least the first N slots of Y or Y’ candidate slots are to be selected within the provided SL DRX active time when partial sensing is configured in the UE by higher layer, where N is (pre-)configurable from a range of 1 to [10].. - FFS: whether This does not apply to re-evaluation and pre-emption checking (e.g. number of remaining Y or Y’ candidate slots is less than N) - Note (not to be captured in the spec): it is possible that the whole set of Y or Y’ candidate slots are selected within the provided SL DRX active time (e.g., when the provided SL DRX active time covers the entire remaining PDB) · T2 is selected by UE implementation such that at least the first N slots of RSW are to be selected within the provided SL DRX active time when full sensing is configured in the UE by higher layer, where N is (pre-)configurable from a range of 1 to [10].. - FFS: whether This does not apply to re-evaluation and pre-emption checking - Note (not to be captured in the spec): it is possible that the whole RSW is within the provided SL DRX active time (e.g., when the provided SL DRX active time covers the entire remaining PDB) · The same (pre-)configured N parameter applies to both partial sensing and full sensing cases. · The reported subset of the candidate resources within the provided SL DRX active time of RX UE shall satisfy a threshold.. - Option 1: The same higher layer parameter (sl-TxPercentageList) is reused for the ratio / threshold That is, number of candidate single-slot resources remaining within the SL DRX active time of the initialized set in Step 4) is to be met by using the RSRP threshold increment only for the SL DRX active time in Step 7, where the is the total number of candidate single-slot resources of the set within the SL DRX active time. - The UE shall satisfy this new threshold in addition to the remaining minimum number of candidate single-slot resources threshold for the whole set in Step 7.

- The proposal targets to make sure that there are sufficient candidate resources within the SL DRX active time of RX UE. We are generally fine with this direction although there were concerns on keeping at least N slots within the active time in the RAN1#107b meeting. To resolve any concern, it would be better to also address the case where N slots within the active time cannot be satisfied.

- When the SL DRX active time is provided, it has been agreed that the candidate resources are reported to higher layer and a subset of the reported candidate resources is within the active time. In our view, it is important for the reported candidate resources to satisfy the following two conditions. Firstly, the number of the reported candidate resources within the active time should be larger than a threshold. As in the above proposal, the number of X·Ntotal should be met, where Ntotal is the total number of the candidate resources within the active time. Secondly, the first slot of the reported candidate resources should be within the active time. Since the candidate resources before the active time can never be selected by higher layers, there is no need to report such candidate resources. An example is provided in Figure 3 to show the reported candidate resources satisfying the aforementioned conditions. For the reported candidate resources (green colour), the first candidate resource is within the active time, and the number of the reported candidate resources within the active time is larger than a threshold.

Figure 3: An example for the reported candidate resources

When SL DRX active time of RX UE is provided, the candidate resources reported to higher layer satisfy the following conditions.

- The number of the reported candidate resources within the active time is at least X·Ntotal, where Ntotal is the total number of the candidate resources within the active time.

- The first slot of the reported candidate resources is within the active time.

- When a TX UE transmits to a RX UE configured with DRX, the resources selected by the TX UE should be within the active time (e.g., on duration timer or inactivity timer, or retransmission timer) at the RX UE. However, a selected resource can be dropped due to prioritization/congestion control or can be re-selected due to pre-emption/re-evaluation/dropping. Furthermore, the dropping or re-selection of a resource can result in that another resource is not within the active time anymore. One example is shown in Figure 4 where resource C and resource D are within the retransmission timers. If resource C is dropped, the associated retransmission timer will not be initiated at the RX UE. Therefore, resource D will not be within the active time anymore. As one option, resource D can be dropped too. As another option, resource D (resource C) can be re-selected. Similarly, if resource C is re-selected, it may happen that resource D is outside of the active time depending on where the re-selected resource C is. Anyway, if a selected resource is not within the active time anymore, dropping or re-selection at the TX UE can be triggered.

Figure 4: An example for dropping of resource #1 makes resource #2 outside of the active time.

Dropping or re-selection of resource(s) at the TX UE is triggered if dropping or re-selection of a resource results in that another resource is not within the active time of the RX UE anymore.

Conclusions

In this contribution, we have expressed our views on the resource allocation for partial sensing and SL DRX. The proposals are summarized as follows.

- When performing periodic-based partial sensing, UE should determine the minimum value of “Y” based on the priority and/or CBR value, from a range of values (pre-)configured in a resource pool.

When PSFCH is configured for the resource pool, the impact of the HARQ RTT related timing restriction should also be considered when UE determines the “Y” candidate slots.

- For partial sensing, the parameter Tscal is updated as tyLSL-(ty0SL-(Tproc,0SL+Tproc,1SL)), where slot ty0SL is the first slot of the Y or Y’ slots and slot tyLSL is the last slot of the Y or Y’ slots.

When SL DRX active time of RX UE is provided, the candidate resources reported to higher layer satisfy the following conditions.

- The number of the reported candidate resources within the active time is at least X·Ntotal, where Ntotal is the total number of the candidate resources within the active time.

- The first slot of the reported candidate resources is within the active time.

Dropping or re-selection of resource(s) at the TX UE is triggered if dropping or re-selection of a resource results in that another resource is not within the active time of the RX UE anymore.

References

- R1-2200724, 3GPP TSG RAN1#107b, FL summary for AI 8.11.1.1 – resource allocation for power saving (EOM), January 17th – 25th, 2022.