3GPP TSG RAN WG1 Meeting #108-e	R1-2201450

e-Meeting, February 21st – March 3rd, 2021

Source:	ZTE

Title:	Discussion on SRS carrier switching

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

In RAN1#105-e, RAN1#106-e and RAN1#107e meetings [1][2][3], some issues related to SRS carrier switching were discussed including prioritization rule between SRS for carrier switching and other UL signals which are overlapped in time domain, timeline of aperiodic SRS for carrier switching, etc..

The achieved agreements are described as follows.

Agreement in RAN1#105-e The prioritization rules of SRS carrier switching apply to at least the source CC. - FFS : Whether the specification needs to be updated or not Agreement in RAN1#105-e Down select one from the following two options in RAN1#106-e meeting to determine which UL CCs other than the source CC should be used for SRS carrier switching priority rules: - Option 1: The UL CCs in the same band as the source CC - Option 2: The UL CCs can be any carriers which result in uplink transmissions beyond the UE ’s indicated uplink carrier aggregation capability. - Companies should indicate how “UE’s indicated uplink CA capability” is derived based on current ASN.1 signaling. Agreement in RAN1#106-e For a target CC, when multiple aperiodic SRS resource sets for carrier switching are triggered by the same DCI and all the SRS resource sets will be transmitted according to the dropping rule, regarding UE behaviour on switching back to the source CC after transmitting one SRS resource set, further discuss the following alternatives: - Alt 1) The behavior depends on the UE implementation - Alt 2) UE stays in the target CC in the period between the SRS resource sets. - Alt 3) If the time period between the SRS resource sets is smaller than the total required RF switching time to the source CC and back to the target CC and a higher priority UL transmission and/or DL reception is not scheduled on the source CC in the time period between the two SRS resources sets, the UE stays in the target CC in the period between the SRS resource sets; otherwise, the UE switches back to the source CC after transmitting each SRS resource set. - Alt 4) UE switches back to source CC between the SRS resource sets Agreement in RAN1#106-e For a target CC, In the case that multiple SRS resource sets are triggered by the same DCI, regarding the applicable timeline(s), further discuss the following alternatives: - Alt 1) Individual timeline is applied to each triggered SRS resource set - “Individual timeline” means that for each SRS resource set, the deadline to consider DCI triggering the SRS resource set or other uplink signals is applied and decision is made independently amongst the SRS resource sets. - Alt 2) The same timeline is applied to all the triggered SRS resource sets - “Same timeline” means that the deadline to consider DCI triggering the SRS resource sets or other uplink signals is applied considering the multiple SRS resource sets as a whole so that a single decision on collision handling is made for these SRS resource sets. Conclusion in RAN1#107-e Regarding SRS carrier switching priority rules: - For Rel-16, it is concluded that no modification in specifications should be made to clarify the current UE behaviour or to introduce a new UE behaviour regarding SRS carrier switching priority rules. - For releases later than Rel-16, it is concluded to consider introducing a new UE capability for indicating simultaneous transmission while switching, and/or clarify the UE behaviour in the case of intra-band CA. - Note: If introduced, the new UE capability should always assume no simultaneous transmission while SRS carrier switching for the bands in the band combinations that are signalled to not support simultaneous transmission within BandCombinationList-UplinkTxSwitch. Agreement in RAN1#107-e When multiple SRS resource sets for carrier switching are triggered by the same DCI, individual timeline is applied to each triggered SRS resource set (Alt 1 in RAN1 106-e agreement). - FFS: whether spec change is needed

In this contribution, we propose to specify or clarify some ambiguities for Rel-16 specification.

- Discussion

- Timeline

As shown in the last agreement of section 1, when multiple SRS resource sets for carrier switching are triggered by the same DCI, individual timeline is applied to each triggered SRS resource set. However, the current 38.214 seems not crystal to reflect this agreement. Hence, we propose to revise the specification accordingly.

Proposal 1: Adopt the following text proposal for Rel-16 38.214 [4]

- 6.2.1.3 UE sounding procedure between component carriers A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2. For an SRS resource set transmission starting in symbol Nc1 of carrier c1 and a conflicting transmission in carrier c2 starting in symbol Nc2, the UE shall apply the prioritization / dropping rules in the remainder of this clause taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and Nc1 is at least N2 symbols and an additional time duration TSRSCS, and the time interval between the last symbol of PDCCH and Nc2 is at least N2 symbols; and - semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSCS before Nc1, and considered active at least N2 symbols before Nc2. where TSRSCS=max⁡{switchingTimeUL,switchingTimeDL}, and the time interval unit of OFDM symbol is counted based on the smaller subcarrier spacing across c1, c2 and their corresponding scheduling cells.

- Switching to source CC

With respect to the third agreement listed in the section 1, for a target CC, when multiple aperiodic SRS resource sets for carrier switching are triggered by the same DCI and all the SRS resource sets will be transmitted according to the dropping rule, we propose to clearly specify UE behavior after transmitting one SRS resource set. Then, gNB can determine a proper time gap between two SRS resource sets when configuring SRS parameters by RRC signaling.

Based on the discussion before, Alt 3 and Alt 4 were supported by majority companies. In our view, Alt 3 is more efficient and also avoid blindly switching to source CC compared to Alt4 especially when there is not any UL transmission in the source CC. Hence, we propose to make Alt 3 as a conclusion for Rel-16.

Proposal 2: Make the following as a conclusion for Rel-16.

- For a target CC, when multiple aperiodic SRS resource sets for carrier switching are triggered by the same DCI and all the SRS resource sets will be transmitted according to the dropping rule, regarding UE behaviour on switching back to the source CC after transmitting one SRS resource set:

  - If the time period between the SRS resource sets is smaller than the total required RF switching time to the source CC and back to the target CC and a higher priority UL transmission and/or DL reception is not scheduled on the source CC in the time period between the two SRS resources sets, the UE stays in the target CC in the period between the SRS resource sets; otherwise, the UE switches back to the source CC after transmitting each SRS resource set.

- Conclusion

In this contribution, we provide our views on NR SRS carrier switching for Rel-16.

Proposal 1: Adopt the following text proposal for Rel-16 38.214 [4]

- 6.2.1.3 UE sounding procedure between component carriers A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2. For an SRS resource set transmission starting in symbol Nc1 of carrier c1 and a conflicting transmission in carrier c2 starting in symbol Nc2, the UE shall apply the prioritization / dropping rules in the remainder of this clause taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and Nc1 is at least N2 symbols and an additional time duration TSRSCS, and the time interval between the last symbol of PDCCH and Nc2 is at least N2 symbols; and - semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSCS before Nc1, and considered active at least N2 symbols before Nc2. where TSRSCS=max⁡{switchingTimeUL,switchingTimeDL}, and the time interval unit of OFDM symbol is counted based on the smaller subcarrier spacing across c1, c2 and their corresponding scheduling cells.

Proposal 2: Make the following as a conclusion for Rel-16.

- For a target CC, when multiple aperiodic SRS resource sets for carrier switching are triggered by the same DCI and all the SRS resource sets will be transmitted according to the dropping rule, regarding UE behaviour on switching back to the source CC after transmitting one SRS resource set:

  - If the time period between the SRS resource sets is smaller than the total required RF switching time to the source CC and back to the target CC and a higher priority UL transmission and/or DL reception is not scheduled on the source CC in the time period between the two SRS resources sets, the UE stays in the target CC in the period between the SRS resource sets; otherwise, the UE switches back to the source CC after transmitting each SRS resource set.

- References

- Chair's Notes RAN1#105-e final

- Chair's Notes RAN1#106-e final

- Final_Minutes_report_RAN1#107-e_v100

- 38.214-g80, NR Physical layer procedures for data (Release 16)