3GPP TSG RAN WG1 Meeting #108			     R1-2201474

E-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on Channel access mechanism for NR in FR2-2

Agenda Item:	8.2.6

Document for: 	Discussion and Decision

- Introduction

The status report [1] has declared that RAN1 completed its work for NR 52.6-71GHz WI. This contribution describes our view on the maintenance for channel access mechanism for NR in FR2-2.

- Discussion

  - Channel access type indication in fallback DCI

For 52.6 – 71 GHz unlicensed band operation, three types of channel access procedures are supported: Type 1 (LBT with random back-off without CWS adjustment), Type 2 (LBT without random back-off) and Type 3 (no LBT). To determine a type to be used for UL transmission at UE, gNB needs to indicate which type to be used by DCI. For non-fallback DCI formats, RAN1 has agreed the following already:

Agreement For Non-Fallback DCI formats, for FR2-2 operation, for the configuration of the ChannelAccess-CPext field in DCI to indicate the channel access type only, new tables are introduced indicating channel access types for FR2-2, with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

A remaining issue is how to support the indication by fallback DCI formats. Our best preference is to support the same manner as non-fallback DCI formats, i.e. having two bits for the indication of channel access type even in fallback DCI formats. In the previous e-meetings, some companies argued that only one bit with Type 1 and Type 3 indication is sufficient for fallback DCI formats since Type 2 channel access procedure is an optional capability. However, we would like to point out that which type of LBT is mandatory/optional and which type of LBT is deemed beneficial for operation would depend on multiple aspects, e.g., local regulation, interference condition, etc. As fallback DCIs can be used for RRC_CONNECTED UEs, which can indicate its support of both Type 1 and Type 2 channel access procedures, we believe in some situations two-bit indication would be beneficial even for fallback DCI formats.

Meanwhile, if only one bit is deemed necessary for channel access type indication in fallback DCI formats, we prefer to define the configurable (or variable) association between each entry and indicated channel access type. For example, depending on which type(s) of channel access procedure the UE supports and/or region, an association between an entry and the exact channel access type to be indicated (or, the table defining the associations itself) can be adapted to another association which can be more proper.

Proposal 1: For channel access type indication by fallback DCI formats, adapt either of the following TPs:

- TP#1 Alt-1: Support 2-bit indication to cover all the three channel access types

- TP#1 Alt-2:  Support 1-bit indication, and the association between entries and the indicated types to be configurable, where default table covers Type 1 and Type 3

TP#1 Alt-1:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: … - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise. … The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: … - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise … Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 for frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; gNB. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; gNB. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; Sensing as defined in Clause x.x in TS 37.213; 0; UE. Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved.. Table 7.3.1.1.1-4B: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. 2; Type 3 channel access defined in clause 4.4.3 of 37.213. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

TP#1 Alt 2:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: … - ChannelAccess-CPext – number of bits determined by the following: - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access in FR1. - 1 bit indicating channel access type as defined in Table 7.3.1.1.1-4B or Table 7.3.1.1.1-4C if [RRC parameter] is configured, or as defined in Table 7.3.1.1.1-4B if [RRC parameter] is not configured, for operation in a cell with shared spectrum channel access in FR2-2 - 0 bit otherwise. … The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: … - ChannelAccess-CPext – number of bits determined by the following: - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access in FR2. - 1 bit indicating channel access type as defined in Table 7.3.1.1.1-4B or Table 7.3.1.1.1-4C if [RRC parameter] is configured, or as defined in Table 7.3.1.1.1-4B if [RRC parameter] is not configured, for operation in a cell with shared spectrum channel access in FR2-2. - 0 bit otherwise. … Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 for frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided in FR1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; gNB. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; gNB. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; Sensing as defined in Clause x.x in TS 37.213; 0; UE. Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved.. Table 7.3.1.1.1-4B: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 3 channel access defined in clause 4.4.2 of 37.213. Table 7.3.1.1.1-4C: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

  - Details of L3-RSSI measurement and reporting

On L3-RSSI measurement and reporting, the following is the latest agreement, which still has a few FFS points:

Agreement: Support extending Rel.16 L3-RSSI to unlicensed operation in FR2-2 - Introduce RRC configuration for reference SCS, measurement duration, and measurement bandwidth - Extend the reference SCS/CP field (ref-SCS-CP-r16) and measurement duration field (measDurationSymbols-r16) in RMTC-Config - FFS value range and valid combinations for ref-SCS-CP-r16 and measDurationSymbols-r16 - Introduce parameter in RMTC-Config to indicate the measurement bandwidth - FFS: Value range for measurement bandwidth - For the QCL Type-D of L3-RSSI measurement, down-select one or both of the following alternatives - Alt 1: gNB configures the beam when configures the L3-RSSI measurement - Alt 2: Use the QCL type-D of the latest received PDSCH and the latest monitored CORESET Agreement Introduce new parameter in RMTC-Config for L3-RSSI to indicate measurement bandwidth. - The value range for the configured measurement bandwidth should include the maximum and the minimum channel bandwidth and the intermediate channel bandwidths defined by RAN4. Agreement On measDurationSymbols and reference SCS/CP for L3-RSSI - On measDurationSymbols-r16 with ref-SCS-CP-r16=120KHz, extend measDurationSymbols-r16 to {1,14,28,42,70,140} - On measDurationSymbols-r16 with ref-SCS-CP-r16=480KHz (if supported), extend measDurationSymbols-r16 to {1,14,28,42,70,140, 560} - On measDurationSymbols-r16 with ref-SCS-CP-r16=960KHz (if supported), extend measDurationSymbols-r16 to {1,14,28,42,70,140, 560,1120}

The only remaining issue would be QCL Type-D of L3-RSSI measurement. So far two options are discussed: Alt 1 where gNB configures the beam when configures the L3-RSSI measurement, and Alt 2 where QCL Type D of the latest received PDSCH and the latest monitored CORESET is used. Comparing the two alternatives, we believe Alt 1 would be more beneficial. Which direction a UE wants to sense (or which direction NW/gNB wants the UE to sense) would vary depending on the situation and/or actual condition of e.g., interference around the UE. It should be configurable even for L3-level functionality. Meanwhile, in case when such RRC-level QCL Type-D configuration doesn’t exist, we are fine with considering Alt-2.

To summarize, we propose the following:

Proposal 2: For QCL Type-D of L3-RSSI measurement, support to introduce RRC parameter for QCL Type-D configuration:

- If there is a case that the RRC parameter for QCL Type-D configuration of L3-RSSI measurement is not configured, QCL Type D of the latest received PDSCH and the latest monitored CORESET is used.

  - LBT bandwidth

Even after the discussion across multiple e-meetings, we still view it divergent what companies consider on bandwidth for LBT. Currently the bandwidth “to be actually sensed” is described as the one of “channel” defined in Clause 4.0 in 37.213 [2]. It is also related to EDT adaptation equation, as described in Clause 4.4.7 of the latest 37.213 as follows:

### 4.4.7 Energy detection threshold adaptation procedures A gNB/UE accessing a channel on which transmission(s) on beam(s) are performed within a channel occupancy, shall set the energy detection threshold to be less than or equal to the maximum energy detection threshold that is determined as follows: where: - is the RF output power limit in . - is the maximum EIRP of the intended transmission(s) by the gNB/UE to acquire a channel occupancy in where . The maximum EIRP used for the transmission(s) by the initiating gNB/UE during the channel occupancy is limited to . - is the [channel bandwidth or bandwidth part bandwidth] in MHz.

For this issue, our view is as follows:

- We generally believe the actual bandwidth “to be sensed” should be at least equal to or wider than the actual bandwidth of the transmission intended.

- During the discussion on multi-carrier LBT, following conclusion was made at RAN1#106bis-e meeting due to the lack of consensus on supporting single LBT covering multiple CCs. The note in the conclusion describes that even though gNB/UE implementation to perform single wideband LBT is not ruled out, the checking with the EDT is necessary to be based on the CC EDT, i.e., wideband EDT should not be used.

Conclusion: There is no consensus to support explicitly introducing in the spec using single LBT covering multiple CCs under CA. - Note: This does not rule out gNB/UE implementation to perform single LBT to cover multiple CCs. However, the EDT needs to be selected such that if interference on one of the CCs exceeds the CC EDT, the LBT is declared as failed

- Therefore, similar to above conclusion, we think this issue can be concluded as below.

  - LBT BW should be at least equal to or wider than the actual transmission bandwidth, and hence in the specification, either CC bandwidth or the bandwidth part bandwidth can be defined as the LBT BW. We slightly prefer the bandwidth part bandwidth at UE side and channel bandwidth (i.e., bandwidth of component channel defined in RAN4) at gNB side, as it would be closer to the actual transmission bandwidth.

  - The EDT for the LBT should be determined based on the above LBT BW (e.g., the bandwidth part bandwidth).

  - This does not rule out gNB/UE implementation to perform LBT with wider bandwidth than the bandwidth part bandwidth. However, the EDT needs to be selected such that if interference on the bandwidth part exceeds the EDT based on the bandwidth part bandwidth, the LBT is declared as failed.

With above, we propose the following:

Proposal 3: For LBT bandwidth, support TP#2

- Bandwidth to be sensed can be equal to or wider than the one configured for active BWP, which does not require any TP

- Bandwidth to be considered for EDT adaptation should be fixed (e.g. active BWP bandwidth at UE, or channel bandwidth defined in TS38.101-2) , as captured in TP#2, or determined from a limited range (e.g. consider channel bandwidth to be maximum)

TP#2

### 4.4.7 Energy detection threshold adaptation procedures A gNB/UE accessing a channel on which transmission(s) on beam(s) are performed within a channel occupancy, shall set the energy detection threshold to be less than or equal to the maximum energy detection threshold that is determined as follows: where: - is the RF output power limit in . - is the maximum EIRP of the intended transmission(s) by the gNB/UE to acquire a channel occupancy in where . The maximum EIRP used for the transmission(s) by the initiating gNB/UE during the channel occupancy is limited to . - , in MHz, is determined as follows: - the channel bandwidth, as defined in [reference to TS38.101-2] when the corresponding transmission is performed from gNB, or - the bandwidth part bandwidth when the corresponding transmission is performed from UE.

  - Handling when counter reaches 0 but the device is not ready to transmit

At RAN1#107bis-e, how to handle the case when counter for Type 1 channel access procedure reaches 0 but the device is not ready to transmit has been discussed while no consensus has been reached.

We believe the same issue was discussed in Rel-16 NR-U, which results in some texts in Clause 4.1.1 of 37.213. Since the issue is the same, the same approach should also be taken into consideration. Thus, we propose the following:

Proposal 4: Adopt the following TP#3

TP#3

### 4.4.1 Type 1 channel access procedures This clause describes channel access procedures to be performed by a gNB/UE where the time duration spanned by the sensing slots that are sensed to be idle before a transmission(s) is random based on a fixed contention window size. The clause is applicable to any transmission initiating a channel occupancy by the gNB/UE. The gNB/UE may transmit a transmission after first sensing the channel to be idle during the sensing slot duration of a defer duration and after the counter is zero in step 4. The counter is adjusted by sensing the channel for additional sensing slot duration(s) according to the steps below: 1) set , where is a random number uniformly distributed between 0 and , and go to step 4; 2) if and the gNB/UE chooses to decrement the counter, set ; 3) sense the channel for an additional sensing slot duration, and if the channel is idle for the additional sensing slot duration, go to step 4; else, go to step 5; 4) if , stop; else, go to step 2. 5) sense the channel until either it is detected busy within an additional defer duration or it is detected to be idle for the sensing slot of the additional defer duration ; 6) if the channel is sensed to be idle during the sensing slot duration of the additional defer duration , go to step 4; else, go to step 5; In the above procedures, is the contention window and . The defer duration is and includes a sensing slot duration for performing as least a single measurement to determine whether the channel is idle. A gNB/UE shall not transmit on a channel for a Channel Occupancy Time that exceeds . If a gNB/UE has not transmitted a transmission after step 4 in the procedure above, the the gNB/UE may transmit a transmission on the channel, if the channel is sensed to be idle at least in a sensing slot duration when the gNB/UE is ready to transmit and if the channel has been sensed to be idle during all the sensing slot durations of a defer duration immediately before this transmission. If the channel has not been sensed to be idle in a sensing slot duration when the gNB/UE first senses the channel after it is ready to transmit or if the channel has been sensed to be not idle during any of the sensing slot durations of a defer duration immediately before this intended transmission, the gNB/UE proceeds to step 1 after sensing the channel to be idle during the sensing slot durations of a defer duration .

  - Short control signaling exemption for UL transmission

Based on the following agreement and conclusion, short control signaling rule defined in BRAN can be applied only for msg1 or MsgA in UL:

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc Conclusion In Rel.17, there is no consensus to apply contention exemption short control signalling to UL transmissions other than msg1 and msgA.

One FFS point still remains in the above agreement; how to interpret the rule of 10% over 100 ms. We believe it should be “per-device” (i.e., Alt 2) because of the following reasons:

- Short control signaling is defined in BRAN regulation, which describes what needs to be ensured by “a device”. Meanwhile, there is no restriction on the behavior of the whole cell. If we follow the exact definition in BRAN, it is undoubtedly straightforward to interpret this rule as in Alt 2.

- Because of the point above, we think there is another RAT which implements some signals and/or channels without LBT by applying short control signaling rule per device. If NR shares a band with such RAT, and if only NR interprets this rule as “per cell”, the actual coexistence would be unfair.

- If we do not consider the coexistence issue described above, “per-cell” interpretation actually does not have any benefit, while it has much more specification impacts. For example, one approach to implement “per-cell” interpretation would be to limit available Ros as proposed in the last e-meeting. We do not think just to limit the available indexes of RO would be reasonable since whether a RO is used or not is totally up to the actual number of UEs that want to perform RACH procedure in the cell. Even if there is no limitation, any RO configuration can eventually comply with the short control signaling exemption rule depending on the actual situation. If we try to optimize the RO limitation to avoid the issue above, there will be much more specification impacts needed, which is not preferable at this stage at all in our view.

With above, we draw the following proposal:

Proposal 5: Define short control signaling by interpreting the exemption rule as “per device”

Moreover, we believe both in RRC and SIB1, there should be a signaling that configures whether short control signaling is applicable for msg1 and MsgA; otherwise, a UE cannot be aware that whether msg1/MsgA can be transmitted without performing LBT.

Proposal 6: Support a signaling to configure whether short control signaling is applicable or not

  - DCI format 2_0 enhancement

There was a proposal that DCI format 2_0 with COT duration indication supports an indication of beam(s) to indicate for which beam(s) the COT duration information is considered. We generally see two use cases for DCI 2_0 with COT duration, one is P-/SP-CSI-RS validation and the other is the upgrade of channel access type. For both of which we do not see the significant need to explicitly introduce this feature. Firstly, depending on the exact beam(s) for which COT duration indication is applicable, there would be no need of such explicit indication. For example, if COT duration indication is defined to be applicable only for the same beam used for DCI 2_0, it would be sufficient to define this behavior without any impact on the contents of DCI 2_0 as the beam for DCI 2_0 would be known to the receiver UEs. In other words, an explicit indication in DCI 2_0 can be motivated only if we want to support more than 1 beam indication. Another point that we believe should be well considered is the relationship between the number of beams and DCI 2_0 payload limitation. When we are to define the indication of multiple beams in DCI 2_0, we should determine how many beams have to be covered. Given the high frequency, we believe so many beams are not necessary in a single DCI 2_0 in practice. However, the sufficient number would be unclear (and we assume would be much divergent among companies), so we do not think RAN1 can reach a consensus on issue within the remaining time in WI. Instead, just to cover maximum beams (e.g. 64 as assumed for SSB) might be another possibility. However, we do not prefer this approach since it leads to significant impact on DCI 2_0 payload.

With above, we propose the following:

Proposal 7: Not support to introduce beam indication in DCI 2_0.

- To define COT duration indication in DCI 2_0 to be applicable only for the beam used by the DCI 2_0 is sufficient to achieve the desired behavior

Together with Proposal 6, we also draw the following proposal for CSI-RS validation and upgrading the type of channel access:

Proposal 8: Support both P-/SP-CSI-RS validation and upgrading the type of channel access based on COT duration indication DCI 2_0

- Support to define the rule so that COT duration indication is applicable only for the beam used for the corresponding DCI 2_0

  - Type A multi-channel access

Agreement Type A multi-channel channel access is supported. - FFS whether legacy mechanisms such as type A1 is supported

The agreement above has been reached in the last e-meeting. “Type A” above refers to the legacy Type A supported in Rel-16 NR-U in our understanding, where two sub-types are defined: Type A1 and Type A2.

For both Type A1 and Type A2, we believe some more clarifications are needed in the specification. For example, in legacy Type A1 and Type A2, the behavior for determining CW_p (per cell in Type A1, and for a cell with maximum CW_p in Type A2) is captured. We do not think CW_p is defined in FR2-2 since the supported channel access procedure is cat-3 LBT, which means CWS adjustment is not defined. Therefore, regardless of Type A1 or Type A2, whether/how to define the exact Type A1 and/or Type A2 needs further discussion. One potential approach is just to copy and paste the legacy text in Rel-16 NR-U other than the parts related to CW_p. Another could be to make sure that CW_p is always expected to be 3 when the legacy Type A1 and A2 are referred.

Moreover, we actually do not see the significant need to support Type A2 since CWS adjustment is not supported in FR2-2. Type A1 and Type A2 eventually achieve the same behavior, so capturing both in the specification could be just redundant.

Proposal 9: For Type A1/A2 multi-channel access,

- Support either of the following approach for the exact specification text:

  - Alt 1: Newly define Type A1 and/or Type A2 for FR2-2, which generally follows the same text as in Clause 4.1.6.1 of 37.213 other than the parts related to CW_p

  - Alt 2: Refer to Clause 4.1.6.1, and add a clarification that CW_p is always expected to be 3 in FR2-2

- Support not to consider Type A2 in FR2-2

- Conclusion

Proposal 1: For channel access type indication by fallback DCI formats, adapt either of the following TPs:

- TP#1 Alt-1: Support 2-bit indication to cover all the three channel access types

- TP#1 Alt-2:  Support 1-bit indication, and the association between entries and the indicated types to be configurable, where default table covers Type 1 and Type 3

TP#1 Alt-1:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: … - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise. … The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: … - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise … Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 for frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; gNB. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; gNB. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; Sensing as defined in Clause x.x in TS 37.213; 0; UE. Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved.. Table 7.3.1.1.1-4B: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. 2; Type 3 channel access defined in clause 4.4.3 of 37.213. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

TP#1 Alt 2:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: … - ChannelAccess-CPext – number of bits determined by the following: - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access in FR1. - 1 bit indicating channel access type as defined in Table 7.3.1.1.1-4B or Table 7.3.1.1.1-4C if [RRC parameter] is configured, or as defined in Table 7.3.1.1.1-4B if [RRC parameter] is not configured, for operation in a cell with shared spectrum channel access in FR2-2 - 0 bit otherwise. … The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: … - ChannelAccess-CPext – number of bits determined by the following: - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access in FR2. - 1 bit indicating channel access type as defined in Table 7.3.1.1.1-4B or Table 7.3.1.1.1-4C if [RRC parameter] is configured, or as defined in Table 7.3.1.1.1-4B if [RRC parameter] is not configured, for operation in a cell with shared spectrum channel access in FR2-2. - 0 bit otherwise. … Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 for frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided in FR1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; gNB. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; gNB. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; Sensing as defined in Clause x.x in TS 37.213; 0; UE. Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved.. Table 7.3.1.1.1-4B: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 3 channel access defined in clause 4.4.2 of 37.213. Table 7.3.1.1.1-4C: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

Proposal 2: For QCL Type-D of L3-RSSI measurement, support to introduce RRC parameter for QCL Type-D configuration:

- If there is a case that the RRC parameter for QCL Type-D configuration of L3-RSSI measurement is not configured, QCL Type D of the latest received PDSCH and the latest monitored CORESET is used.

Proposal 3: For LBT bandwidth, support TP#2

- Bandwidth to be sensed can be equal to or wider than the one configured for active BWP, which does not require any TP

- Bandwidth to be considered for EDT adaptation should be fixed (e.g. active BWP bandwidth at UE, or channel bandwidth defined in TS38.101-2) , as captured in TP#2, or determined from a limited range (e.g. consider channel bandwidth to be maximum)

TP#2

### 4.4.7 Energy detection threshold adaptation procedures A gNB/UE accessing a channel on which transmission(s) on beam(s) are performed within a channel occupancy, shall set the energy detection threshold to be less than or equal to the maximum energy detection threshold that is determined as follows: where: - is the RF output power limit in . - is the maximum EIRP of the intended transmission(s) by the gNB/UE to acquire a channel occupancy in where . The maximum EIRP used for the transmission(s) by the initiating gNB/UE during the channel occupancy is limited to . - , in MHz, is determined as follows: - the channel bandwidth, as defined in [reference to TS38.101-2] when the corresponding transmission is performed from gNB, or - the bandwidth part bandwidth when the corresponding transmission is performed from UE.

Proposal 4: Adopt the following TP#3

TP#3

### 4.4.1 Type 1 channel access procedures This clause describes channel access procedures to be performed by a gNB/UE where the time duration spanned by the sensing slots that are sensed to be idle before a transmission(s) is random based on a fixed contention window size. The clause is applicable to any transmission initiating a channel occupancy by the gNB/UE. The gNB/UE may transmit a transmission after first sensing the channel to be idle during the sensing slot duration of a defer duration and after the counter is zero in step 4. The counter is adjusted by sensing the channel for additional sensing slot duration(s) according to the steps below: 1) set , where is a random number uniformly distributed between 0 and , and go to step 4; 2) if and the gNB/UE chooses to decrement the counter, set ; 3) sense the channel for an additional sensing slot duration, and if the channel is idle for the additional sensing slot duration, go to step 4; else, go to step 5; 4) if , stop; else, go to step 2. 5) sense the channel until either it is detected busy within an additional defer duration or it is detected to be idle for the sensing slot of the additional defer duration ; 6) if the channel is sensed to be idle during the sensing slot duration of the additional defer duration , go to step 4; else, go to step 5; In the above procedures, is the contention window and . The defer duration is and includes a sensing slot duration for performing as least a single measurement to determine whether the channel is idle. A gNB/UE shall not transmit on a channel for a Channel Occupancy Time that exceeds . If a gNB/UE has not transmitted a transmission after step 4 in the procedure above, the the gNB/UE may transmit a transmission on the channel, if the channel is sensed to be idle at least in a sensing slot duration when the gNB/UE is ready to transmit and if the channel has been sensed to be idle during all the sensing slot durations of a defer duration immediately before this transmission. If the channel has not been sensed to be idle in a sensing slot duration when the gNB/UE first senses the channel after it is ready to transmit or if the channel has been sensed to be not idle during any of the sensing slot durations of a defer duration immediately before this intended transmission, the gNB/UE proceeds to step 1 after sensing the channel to be idle during the sensing slot durations of a defer duration .

Proposal 5: Define short control signaling by interpreting the exemption rule as “per device”

Proposal 6: Support a signaling to configure whether short control signaling is applicable or not

Proposal 7: Not support to introduce beam indication in DCI 2_0.

- To define COT duration indication in DCI 2_0 to be applicable only for the beam used by the DCI 2_0 is sufficient to achieve the desired behavior

Proposal 8: Support both P-/SP-CSI-RS validation and upgrading the type of channel access based on COT duration indication DCI 2_0

- Support to define the rule so that COT duration indication is applicable only for the beam used for the corresponding DCI 2_0

Proposal 9: For Type A1/A2 multi-channel access,

- Support either of the following approach for the exact specification text:

  - Alt 1: Newly define Type A1 and/or Type A2 for FR2-2, which generally follows the same text as in Clause 4.1.6.1 of 37.213 other than the parts related to CW_p

  - Alt 2: Refer to Clause 4.1.6.1, and add a clarification that CW_p is always expected to be 3 in FR2-2

- Support not to consider Type A2 in FR2-2

- Reference

- RP-212990, “SR for Extending current NR operation to 71GHz”, Qualcomm CDMA Technologies, RAN#94-e, 2020

- TS 37.213 V17.0.0, 3GPP TSG RAN, December 2021