3GPP TSG RAN WG1 #108-e			R1-2201492

e-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on resource multiplexing

Agenda Item:	8.10.1

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss remaining issues on resource multiplexing between child and parent links of an IAB node to support simultaneous operation of transmission and/or reception.

- Resource management in frequency domain

In Rel-17, configuration of H/S/NA resource types per RB set is supported. However, IAB node behavior on an RB set configured as H/S/NA is not defined in current specification in TS 38.213 [1]. Thus, it is proposed to capture in TS 38.213 the IAB node behavior on H/S/NA RB sets.

Proposal 1: Adopt the following CR for section 14 of TS 38.213.

## <Unchanged text is omitted> When an RB set of a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive on the RB set in the symbol. When an RB set of a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive on the RB set in the symbol only if - the IAB-MT does not transmit or receive on the RB set during the symbol of the IAB-DU cell, or - the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set during the symbol of the IAB-DU cell is not changed due to a use of the RB set in the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available When an RB set of a symbol is configured as unavailable, the IAB-DU neither transmits nor receives on the RB set in the symbol. <Unchanged text is omitted>

It was agreed that for a given RB set at a symbol, if Rel-17 frequency domain H/S/NA configuration is not provided, the Rel-16 time domain H/S/NA is applied. And a working assumption was made that if both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, an IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied, which was captured in current specification in TS 38.213 [1]. However, in current specification, if an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by HSNA Slot Configuration List when the IAB-node uses simultaneous transmission and reception in the slot, where HSNA Slot Configuration List refers to Rel-16 time domain H/S/NA, which is not correct because Rel-17 frequency domain H/S/NA should be applied in such case. Thus, it is proposed to adopt following CR.

Proposal 2: Adopt the following CR for section 14 of TS 38.213.

## <Unchanged text is omitted> With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config HSNA Slot Configuration List when the IAB-node uses simultaneous transmission and reception in the slot. <Unchanged text is omitted>

- Dual connectivity

Following agreement was made for soft resource availability indication in DC scenario.

Agreement For an IAB-MT with multiple serving cells (including the case with two parent nodes), a per-cell IAB-DU soft resource is considered as available if the resource is either explicitly indicated (via DCI 2_5), or implicitly determined as available with respect to all serving cells. - If the IAB-DU per-cell soft resource neither explicitly indicated as Available, nor implicitly determined as Available by the IAB-DU with respect to at least one serving cell - Alt 1. The IAB-DU per-cell resource is assumed to be not available

With the above agreement, in our understanding, IAB node behavior is as shown in the following table.

Table I. IAB node behavior with respect to availability indication of two serving cells.

| Serving cell 1 | Serving cell 2 | IAB node behavior |
| --- | --- | --- |
| Explicit IA or implicit IA | Explicit IA or implicit IA | Available |
| Explicit IA or implicit IA | Neither explicit IA nor implicit IA | Not available |
| Neither explicit IA nor implicit IA | Explicit IA or implicit IA | Not available |
| Neither explicit IA nor implicit IA | Neither explicit IA nor implicit IA | Not available |

However, in current specification in TS 38.213 [1], a DU soft resource is available if explicitly indicated as available from at least one serving cell, which is not aligned the agreement. Thus, it is proposed to adopt following CR.

Proposal 3: Adopt the following CR for section 14 of TS 38.213.

## <Unchanged text is omitted> When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive in the symbol only if - the IAB-MT does not transmit or receive during the symbol of the IAB-DU cell, or - with respect to all serving cells - the IAB-MT would transmit or receive during the symbol of the IAB-DU cell, and the transmission or reception during the symbol of the IAB-DU cell is not changed due to a use of the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available from at least one serving cell ## <Unchanged text is omitted>

- Conclusion

In this contribution, we discussed enhancements on resource multiplexing between child and parent links of an IAB. Based on the discussion, we made following proposals.

Proposal 1: Adopt the following CR for section 14 of TS 38.213.

## <Unchanged text is omitted> When an RB set of a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive on the RB set in the symbol. When an RB set of a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive on the RB set in the symbol only if - the IAB-MT does not transmit or receive on the RB set during the symbol of the IAB-DU cell, or - the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set during the symbol of the IAB-DU cell is not changed due to a use of the RB set in the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available When an RB set of a symbol is configured as unavailable, the IAB-DU neither transmits nor receives on the RB set in the symbol. <Unchanged text is omitted>

Proposal 2: Adopt the following CR for section 14 of TS 38.213.

## <Unchanged text is omitted> With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config HSNA Slot Configuration List when the IAB-node uses simultaneous transmission and reception in the slot. <Unchanged text is omitted>

Proposal 3: Adopt the following CR for section 14 of TS 38.213.

## <Unchanged text is omitted> When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive in the symbol only if - the IAB-MT does not transmit or receive during the symbol of the IAB-DU cell, or - with respect to all serving cells - the IAB-MT would transmit or receive during the symbol of the IAB-DU cell, and the transmission or reception during the symbol of the IAB-DU cell is not changed due to a use of the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available from at least one serving cell ## <Unchanged text is omitted>

References

- 3GPP TS 38.213 V17.0.0 (2021-12)