3GPP TSG RAN WG1 #108-e			R1-2201493

e-Meeting, February 21st – March 3rd, 2021

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on IAB timing mode

Agenda Item:	8.10.2

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss remaining issues on IAB-node Case #7 timing mode to support simultaneous operation.

- Discussion on enhancements on #7 timing mode

At the RAN1#107-e meeting, derivations of Tx timing for case #7 were discussed and following agreement was made [1].

Agreement

The dynamic range of the MAC CE case #7 timing offset indication is 12 bits.

- FFS the numerical values of the endpoints of the range

Agreement

The dynamic range of the MAC CE case #7 timing offset indication is 12 bits.

- FFS the numerical values of the endpoints of the range

At the RAN1#107-e meeting, the value range of the offset values was discussed, and the remaining discussion point is the endpoints of the value range. An example of the timing relationship for Case #7 timing mode is shown in Fig.1. As shown in Fig.1, the offset value is obtained by 1) the propagation delay between the parent node and IAB node and 2) Rx/Tx switching gap at the IAB node. For the propagation delay and the switching gap, the value range of 1/2∙NTA in RAR (e.g. from 0 to 3938304/2 (Tc) for SCS = 15 kHz) and 2∙Tdelta (e.g. from -70528∙2 to 6208∙2 for FR1) can be referred, respectively. Therefore, the offset value is derived by -12∙NTA+2∙Tdelta, and the endpoints of the offset value are summarized in Table I.

Proposal 1: The offset value for Case #7 timing mode is derived by -12∙NTA+2∙Tdelta, so that the endpoint is set as -2110208 (Tc) for SCS = 15 kHz.

Figure 1: Example of timing relationship for Case #7 timing mode.

Table I  Endpoints of offset value for Case #7 timing mode.

| SCS (kHz) | Endpoints of offset value for Case #7 timing mode (Tc) |
| --- | --- |
| 15 | -2110208 |
| 30 | -1125632 |
| 60 | -633344 |
| 120 | -281472 |
| 240 | -158400 |

- Conclusion

In this contribution, we discussed enhancements on IAB-node timing mode to support simultaneous operation. Based on the discussion, we made following proposal.

Proposal 1: The offset value for Case #7 timing mode is derived by -12∙NTA+2∙Tdelta, so that the endpoint is set as -2110208 (Tc) for SCS = 15 kHz.

References

- 3GPP, RAN1#107-e, RAN1 Chairman’s Notes, Nov. 2021.