3GPP TSG-RAN WG1 Meeting #108			R1-2201502

E-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.2

Title:	Views on UE features for supporting NR in FR2-2

Source:	NTT DOCOMO, INC.

Document for:	Discussion and Decision

- Introduction

This contribution describes our views on UE features for supporting NR from 52.6 GHz to 71 GHz in Rel-17, captured in R1-2200780 [1].

- Discussion

2.1. General views

A general issue would be how to consider FR-related differentiation. In this WI, companies discuss on various types of new features and enhancements of the existing NR functionalities to extend NR to 52.6 – 71 GHz frequency range. Any UE feature to be supported in this WI will then be applicable at least for 52.6 – 71 GHz. However, companies may or may not argue that some of the UE features to be specified in this WI could be technically beneficial even in another frequency range, and then desire to discuss whether/how to expand the applicability of such UE features to other frequency ranges. It may consume quite a lot of time in RAN1 to discuss the range of applicability one by one. Note that 52.6 – 71 GHz frequency range at least include unlicensed bands, while licensed bands may also be identified in the future. The WI also target the support for both licensed and unlicensed band in this frequency range. This fact may also make the discussion more complex.

Also, frequency range notation itself should be carefully considered. In the latest WID for supporting NR from 52.6 GHz to 71 GHz [2], two important aspects on frequency range definition regarding beyond 24 GHz are captured; one is to extend the definition of FR2 up to 71GHz, and the other is to introduce new FR sub-labels, FR2-1 and FR2-2, to be used for differentiating 24.25 – 52.6 GHz and 52.6 – 71 GHz if needed. Therefore, as well as FR1/2 differentiation, FR2-1/2-2 differentiation may also need to be considered.

With the consideration above, how to have FR-related differentiation would depend on each UE feature in our view. We see the following alternatives at this stage.

- One potential approach to easily solve this issue could be to define all the UE features to be specified in this WI per-band (or per BC). With this, UE can report its capability regarding functionalities for 52.6 – 71 GHz operation per band that the UE supports, which means vendors have a freedom for its own implementation. However, it may increase the overhead for UE capability signalling depending on the number of bands/band combinations to be specified.

- For UE features which can be applied regardless of licensed or unlicensed band, extending per-FR capability signalling may be another possibility. For example, by enabling per-FR capability signalling to differentiate FR2-1 and FR2-2, it would be possible to indicate a certain UE feature is applicable for FR2-2 only if needed. Or, if a UE feature is applicable to both FR2-1 and FR2-2 without any difference, just to use the existing per-FR capability signalling would also be possible. By defining in this manner, vendors still have a freedom to implement a certain feature for a certain frequency range, while overhead for capability signalling can be suppressed.

- To decrease signalling overhead more, per-UE signalling with some Notes can also be considered. This approach, however, may be applicable to particular UE features only, for which the targeted FR is crystal clear. For example, if a UE feature is clearly applicable for FR2-2 unlicensed band only, it could be possible to define it as a UE feature with per-UE capability signalling with a Note saying i.e., “this is applicable only for unlicensed band in FR2-2”. While this approach achieves much less overhead on UE capability signalling, an issue may be less implementation flexibility.

Table 1. Comparison of FR differentiation approaches

| FR differentiation | Flexibility for implementation | UE capability signalling overhead |
| --- | --- | --- |
| Per-band | Very flexible | Heavy |
| Per FR | Less flexible | Relatively light |
| Per UE | Much less flexible | Light |

Proposal 1: For the discussion on Rel-17 UE features at least regarding 52.6 – 71 GHz WI, the following alternatives can be considered in case-by-case manner, in terms of FR differentiation.

- Alt 1: define as per-band

- Alt 2: define as per-FR

  - Differentiation of FR2-1/2-2 may or may not be needed

- Alt 3: define as per-UE

  - A fixed limitation (e.g., as a Note) on applicable frequency range may be needed

2.2. Views on the latest Rel-17 UE feature list

We have the following comments/suggestions on the latest Rel-17 UE feature list related to NR 52.6 – 71GHz.

For FG24-1b and 1c, there is an FFS on whether to have a Note that makes these features mandatory in a certain case, e.g., when a UE supports FG24-2 (i.e., SA operation in FR2-2 with 120kHz SCS). We actually support the Note, i.e., prefer to ask UEs supporting SA to mandatorily support these FGs to make them available even during initial access. Without the Note, these FGs will be just optional ones in any scenario, which means gNB in general cannot configure them for initial access since gNB does not have prior knowledge on whether UEs support them or not. believe it is essential to have such a Note for these features since NW may not be able to configure these features for any UEs during initial access. We believe these FGs are well understood as features for improving coverage performance under PSD limitation. Thus, if they are not available during initial access, practical coverage is limited in SA scenario even if they are available after initial access. Although SA operation without them could work (with limited coverage), we hope to make these FGs available regardless of scenarios, including SA.

For FG24-1b, there is another FFS on whether to limit the applicable case within bands for shared spectrum operation. In our understanding, this feature is to avoid coverage degradation due to PSD limitation that needs to be considered in unlicensed band in some regions. Also, we are not sure whether it is beneficial even in licensed band operation where PSD limitation does not need to be considered. Unless there is clear gain by this feature in licensed band, we are ok with limiting this FG for unlicensed band only.

For FG24-1d and FG24-1e, whether to extend it to other FR remains as a FFS. First, we would like to point out that this feature is defined per band. Thus, there is generally no significant reason to explicitly limit the applicable FR on UE feature list. Moreover, this FG is not essential for 120 kHz SCS even in FR2-2, while it is defined there. Therefore, as an optional capability, we think it would be ok to allow this FG to be supported for 120 kHz SCS in FR2-1 if there is a UE that want to support this. We do not support to have an explicit text to limit the applicable FR.

For FG24-4, some FFSs remain there. Our view is as follows:

- On whether to include component 3 (multi-PDSCH scheduling), we support to include it in this FG. It has been agreed already that multi-slot PDCCH monitoring with (Xs, Ys) = (4, 1) is also a component of this FG, which essentially needs multi-PDSCH scheduling in the practical operation.

- On monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS, there was an agreement that the same behaviour as in FG3-1 is supported. Since some FG3-1 components are not applicable for 480 kHz SCS operation even when FG24-4 is supported, we think it would be good to capture this explicitly.

- For the description refinement for component 4, we think the existing text for FG3-5b can be reused. Meanwhile, just to refer FG3-5b would also be ok for us.

FG24-4b has a similar FFS to FG24-1b (i.e. whether to limit the applicable case within bands for shared spectrum operation). We believe the same handling as for FG24-1b should be applied anyway. We are ok with limiting the applicable case to unlicensed band only.

FG24-4f has a similar FFS to the 2nd point of FG24-4 above, that is, monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS. We believe it would be sufficient to follow FG24-4 according to the agreement at the last RAN1 e-meeting. If FG24-4 captures this point, FG24-4f doesn’t need to capture this since they are technically equivalent.

For FG24-5, similar to FG24-4, a few FFS points remain, and generally we have the same proposals:

- On whether to include component 3 (multi-PDSCH scheduling), we support to include it in this FG. It has been agreed already that multi-slot PDCCH monitoring with (Xs, Ys) = (8, 1) is also a component of this FG, which essentially needs multi-PDSCH scheduling in the practical operation.

- On monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS, there was an agreement that the same behaviour as in FG3-1 is supported. Since some FG3-1 components are not applicable for 960 kHz SCS operation even when FG24-5 is supported, we think it would be good to capture this explicitly.

- For the description refinement for component 4, we think the existing text for FG3-5b can be reused.

FG24-5f has the same issue as in FG24-4f, so it would be straightforward to follow the direction to be taken for FG24-4f.

For FG24-6 and FG24-7, an explicit description on LBT bandwidth may be captured, while the exact text is still FFS. In our understanding, we need to wait for WI progress a bit more, if needed, since there is an on-going discussion under AI 8.2.6.

FG24-8 and 24-9 define the capabilities for the support of 32 HARQ processes. RAN1 reached the following agreement at the last RAN1 e-meeting.

Agreement - In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability.

The agreement above can be interpreted by itself such that 1) we have to define another FG on the support of 32 HARQ processes for 120 kHz SCS, and 2) the new capability is defined as a prerequisite of FG24-8 and 24-9. However, considering the discussion so far, we are not sure if we should define such separate FG for 120 kHz SCS. 32 HARQ processes in FR2-2 is motivated based on the fact that larger number of symbols are defined for some processing related timeline parameters for larger SCSs, which potentially leads to HARQ process number starvation. In other words, we do not see it technically necessary to support 32 HARQ processes for 120 kHz SCS. Rather, we understand the intention of the agreement above to aim for an unified PHY design across the supported SCSs. Given above, we are not sure if 32 HARQ processes support deserves a separate FG. Instead, we propose to delete all the SCS-related texts in FG24-8 and 24-9.

There is another important discussion on this issue – whether this FG is merged with other FG from other WI. In NR NTN WI, the same mechanism was agreed and there are corresponding FGs as FG26-5. In addition, it seems that the discussion includes another big issue behind, which is whether maximum of 32 HARQ processes can be applied to cell other than NTN/FR2-2. In short, our view is that the feature should not be applicable for other cases and correspondingly there is no need to merge the FGs. From the applicability perspective, there are two rationales:

- No agreements in any WIs. The 32 HARQ processes feature was agreed in NTN WI and FR2-2 WI for these purposes, but not for other purposes. Without certain agreements at appropriate WG or TEI, such an expansion should not be allowed.

- UE burden or signaling overhead. If this feature is applicable for any cell/band and corresponding FG is per UE, then UE that would like to indicate “support” shall support this feature for any cell/band. In our view, there is motivation of this FG only for NTN/FR2-2, so the excessive support is not preferable. Alternatively if this feature is applicable for any cell/band and corresponding FG is per band, then UE needs to report support/not support for all the bands that UE supports including bands without any motivation of this feature, which is meaningless overhead.

Also, as FG24-8 and FG24-9, there is still an issue specific to FR2-2. If we are to merge it with the one in NR NTN WI, such WI-specific issues may need to be considered even in other WIs. We assume such direction just makes the discussion much more complex, and no clear need to do so is observed at this stage.

Given above, we suggest the following update for NR 52.6 – 71 GHz feature list:

Proposal 2: FG24-8 and 24-9 are not merged with FG26-5 defined in NR NTN

Proposal 3: Update the latest UE feature list on 52.6 – 71 GHz as shown in the following Table 2.2-1

Table 2.2-1: The proposed update for UE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | Basic FR2-2 DL support | 1. Support reception of 120kHz subcarrier spacing for DL data and control channels, SSB, and reference signals in FR2-2 for non-initial access |  | Yes | N/A | FR2-2 is not supported | per band | N/A | N/A | N/A | A UE that supports FR2-2 must indicate this FG is supported | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1a | Basic FR2-2 UL support | 1. PRACH with 120KHz SCS and length 139 2. Support transmission of 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 | 24-1 | Yes | N/A | UL in FR2-2 is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Yes | N/A | Wideband PRACH for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | A UE that supports FG 24-2 must indicate this FG is supported Note: This FG is only supported in bands for shared spectrum operation | Optional withcapability signalling |
| 24. NR_ext_to_71GHz | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Yes | N/A | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements | 24-1 | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | Yes | N/A | Multiple PUSCH scheduling by single DCI for 120kHz is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB support for initial access in FR2-2 | 1. Support 120KHz SSB for initial access in FR2-2 | 24-1, 24-1a | N/A | N/A | 120KHz SSB based initial access in FR2-2 is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB support for initial access in FR2-2 | 1. Support 480KHz SSB for initial in FR2-2 | 24-2, 24-4, 24-4a | N/A | N/A | 480KHz SSB for initial access in FR2-2 is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support for DL | 1. 480KH SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) 3. Multi-PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot, where there is a minimum time separation of X symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols of a slot, with (X, Y) = (4, 3) and (7, 3) symbols. Spans do not overlap. Every span is contained in a single slot. The same span pattern repeats in every slot. The separation between consecutive spans within and across slots may be unequal but the same (X, Y) limit must be satisfied by all spans. Every monitoring occasion is fully contained in one span. In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max{maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration. A particular PDCCH monitoring configuration meets the UE capability limitation if the span arrangement satisfies the gap separation for at least one (X, Y) in the UE reported candidate value set in every slot, including cross slot boundary. For type 1 CSS without dedicated RRC configuration, type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(S) of a slot. 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) | 24-1 | Yes | N/A | 480KHz SCS for DL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4a | 480KHz SCS support for UL | 1. PRACH with 480KHz and length 139 2. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1a, 24-4 | Yes | N/A | 480KHz SCS support for UL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Yes | N/A | Wideband PRACH for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | [Note: This FG is only supported in bands for shared spectrum operation] | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 480 kHz | 24-4a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4f | Enhanced PDCCH monitoring for 480KHz in FR2-2 | 1. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) 2.) Within each of the Ys = 2 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot | 24-4 | Yes | N/A | Enhanced PDCCH monitoring for 480KHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: [one or more of] {[(2,1),] (4,2) } Note: If (2,1) is not agreed, this FG will have no component candidate values and the component 1 description will be updated from (Xs,Ys) to (Xs,Ys)=(4,2) similar to FG 24-4 and 24-5 | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) 3. Multi-PDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 3. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, where there is a minimum time separation of X symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols of a slot, with (X, Y) = (7, 3) symbols. Spans do not overlap. Every span is contained in a single slot. The same span pattern repeats in every slot. The separation between consecutive spans within and across slots may be unequal but the same (X, Y) limit must be satisfied by all spans. Every monitoring occasion is fully contained in one span. In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max{maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration. A particular PDCCH monitoring configuration meets the UE capability limitation if the span arrangement satisfies the gap separation for at least one (X, Y) in the UE reported candidate value set in every slot, including cross slot boundary. For type 1 CSS without dedicated RRC configuration, type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(S) of a slot. 4. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 5. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) | 24-1 | Yes | N/A | 960KHz SCS support for DL is not supported | Perband | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5a | 960KHz SCS support for UL | 1. PRACH with 960KHz and length 139 2. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 [3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS] | 24-1a, 24-5 | Yes | N/A | 960KHz SCS support for UL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5f | Enhanced PDCCH monitoring for 960KHz | 1. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys) 2.) Within each of the Ys = 2 or 4 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot | 24-5 | Yes | N/A | Enhanced PDCCH monitoring for 960KHz is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: one or more of {(4,1), (4,2), (8,4)} | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-6 | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 1 channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] | 24-1a | Yes | N/A | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band |  |  |  | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-7 | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 2 channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] | 24-1a, 24-6 | Yes | N/A | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band | N/A | N/A | N/A | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL | 24-1 |  |  |  | [Per UE/per FSPC/per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL | 24-1 |  |  |  | [Per UE/per FSPC/per band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-10 | Additional beam switching time delay | Supported additional beam switching time delay d for 480 kHz SCS | Yes | N/A | [Additional beam switching time delay is not supported] | [Per UE/per band] | N/A | N/A | N/A | Yes | Candidate value set: 56 or 112 symbols | Optional with capability signalling |

Moreover, we believe additional FG(s) need to be defined for HARQ-ACK bundling. We believe it should be associated with the support of multi-PDSCH scheduling. Since separate FG is defined multi-PDSCH scheduling per SCS, the FG for HARQ-ACK bundling may also have to be defined per SCS. Another issue is whether to have a unified FG for both Type 1 and Type 2 HARQ-ACK codebook. We are open for this issue. Based on that, we suggest adding the following. Note that we are also fine with merging FG24-11 and FG24-11a, FG24-12 and FG24-12a, and FG24-13 and FG24-13a (i.e., having single capability for each SCS on HARQ-ACK bundling).

Proposal 4: Adde new FGs for HARQ-ACK bundling, e.g., as in Table 2.2-2

- It should be per SCS

- It can be per type of HARQ-ACK codebook

Table 2.2-2: The proposed additional FGs

| 24. NR_ext_to_71GHz | 24-11 | HARQ-ACK bundling for Type 1 HARQ codebook multi-PDSCH scheduling for 120 kHz SCS | Support HARQ-ACK bundling for Type 1 HARQ codebook for multi-PDSCH scheduling for 120 kHz SCS | 24-1d |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-11a | HARQ-ACK bundling for Type 2 HARQ codebook for multi-PDSCH scheduling for 120 kHz SCS | Support HARQ-ACK bundling for Type 2 HARQ codebook for multi-PDSCH scheduling for 120 kHz SCS | 24-1d |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-12 | HARQ-ACK bundling for Type 1 HARQ codebook for multi-PDSCH scheduling for 480 kHz SCS | Support HARQ-ACK bundling for Type 1 HARQ codebook for multi-PDSCH scheduling for 480 kHz SCS | 24-4 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-12a | HARQ-ACK bundling for Type 2 HARQ codebook for multi-PDSCH scheduling for 480 kHz SCS | Support HARQ-ACK bundling for Type 2 HARQ codebook for multi-PDSCH scheduling for 480 kHz SCS | 24-4 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-13 | HARQ-ACK bundling for Type 1 HARQ codebook for multi-PDSCH scheduling for 960 kHz SCS | Support HARQ-ACK bundling for Type 1 HARQ codebook for multi-PDSCH scheduling for 120 kHz SCS | 24-5 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-13a | HARQ-ACK bundling for Type 2 HARQ codebook for multi-PDSCH scheduling for 960 kHz SCS | Support HARQ-ACK bundling for Type 2 HARQ codebook for multi-PDSCH scheduling for 120 kHz SCS | 24-5 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |

2.3. Views on applicability of Rel-15/16 NR UE features to FR2-2

In Appendix, we show a brief set of analysis regarding Rel-15/16 UE features in terms of applicability to 52.6 – 71 GHz frequency range, based on the UE features specified in 38.822 [2], where we have focused with the principles below:

- Check mandatory UE features in Rel-15/16 if it is applicable to 52.6 – 71 GHz frequency range

- Check UE features with per-UE signalling if it is applicable to 52.6 – 71 GHz frequency range when it is reported applicable to FR2

- For UE features with per-FR capability signalling, we have not analysed yet since it may be straightforward that per-FR signalling will indicate sub-FR level applicability, although it needs further discussions

- For UE features with per-band or per-BC capability signalling, we have checked only for the ones supported in Rel-16 NR-U

Below are some particular aspects that may require discussions

2.3.1 On mandatory UE features

Some UE features are defined as mandatory for NR in Rel-15. It has to be supported even for UEs supporting Rel-17 functionalities. However, they didn’t consider the operation in 52.6 – 71 GHz when specified, especially with larger SCSs. Therefore, some UE features, even the ones defined as mandatory in Rel-15/16, may or may not be feasible in case of the operation in 52.6 – 71 GHz frequency range.

One potential issue among the mandatory features is related to FG3-1 on basic DL control channel, with the following components:

1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP - This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD

For UEs supporting NR from 52.6 GHz to 71 GHz, at least the operation with 120 kHz SCS is to be supported. As 120 kHz SCS is the one which was supported in Rel-15/16 NR already, it does not cause any issue to support the existing UE features, including the ones specified as mandatory, e.g., FR3-1.

However, the UEs supporting NR in 52.6 – 71 GHz may support larger SCS(s), i.e., 480 and/or 960 kHz SCS, as well in order to achieve the operation with larger absolute bandwidth per CBW. Since symbol duration is scaled based on SCS in the same manner as in FR1 and FR2, and definition of slot is same as in Rel-15/16 NR, the operation with 480/960 kHz SCS automatically means the one with shortened duration of a slot.

When operating with shortened duration of a slot by supporting 480 and/or 960 kHz SCS, some components supported as mandatory in FG3-1 may not be feasible. For example, in the second component, up to 3 search space sets in a slot for a scheduled SCell per BWP is supported. Since NR in 52.6 – 71 GHz will be operated with SCS of 120 kHz or larger, whether up to 3 SS sets in a slot is always possible may not be clear. Moreover, in the sixth component, per-slot and per-CC maximum limitation of DCI processing is described, where one unicast DCI scheduling DL and 2 unicast DCI scheduling UL are supported. The feasibility of this may also be affected by shortened duration of a slot, e.g., only smaller number of DCIs may be possible for UE to process per slot with shortened time duration.

Given above, we propose to discuss on how to interpret FG3-1 for the operation with SCS of 480 and/or 960 kHz. We see some alternatives to deal with the situation can be considered; one is to add a Note in a new UE feature to support 480 and 960 kHz SCS in 52.6 – 71 GHz frequency range such as “[a certain components of] FG 3-1 is not applicable to the SCS supported by this FG”.

Proposal 4: For UEs supporting NR in 52.6 – 71 GHz frequency range, how to treat a mandatory UE feature, FG 3-1, should be discussed at least when the UE supports the operation with 480 and/or 960 kHz SCS

2.3.2 On UE features with per-UE capability signalling

As well as mandatory UE features, UE features with per-UE capability signalling also need to be checked in terms of their applicability to the operation in 52.6 – 71 GHz. When UEs report their support of a certain UE feature with per-UE capability signalling, NW will understand that the UE supports the feature regardless of the operating band, frequency range (or even duplex). However, it may not always the case that UE features with per-UE capability signalling are applicable to 52.6 – 71 GHz when it is applicable to the existing frequency ranges.

Our brief analysis is shown on the 6th column on the tables in Appendix. We generally believe most of the UE features with per-UE capability signalling are also applicable to FR2-2 as well. On the other hand, to support such UE features in practice in 52.6 – 71 GHz frequency range, some maintenances in the specifications will be needed, especially in terms of SCS. For example, FG 12-6 is a UE feature with per-UE capability signalling to report whether the UE supports DL SPS with the periodicity shorter than 10 ms. In Rel-16, an RRC parameter periodicityExt-r16 is supported for configuring DL SPS periodicity shorter than 10 ms. However, how to use the value configured via periodicityExt-r16 has not been defined in case that larger SCS than 120 kHz is configured. In other words, even if the UE feature reporting is supported for NR in 52.6 – 71 GHz as it is, when larger SCS than 120 kHz is used, this functionality (i.e., DL SPS with shorter than 10 ms periodicity) cannot be configured in practice.

Observation 1: While most of Rel-15/16 UE features with per-UE capability signalling can be reused as they are for UE to report their support for NR in 52.6 – 71 GHz, some maintenances will be required in the specifications to support the functionalities in practice.

Proposal 5: For Rel-15/16 UE features with per-UE capability signalling, whether to be applicable to FR2-2 when they are reported as applicable should be analysed a case-by-case manner

2.3.3 On UE features with per-FR/band/BC capability signalling

There would be other types of UE features in NR in terms of FR differentiation, that is, UE features with per-FR/band/BC capability signalling. For the ones with per band/BC capability signalling, we do not see the need to check their validity since per-band/BC signalling naturally differentiate FR2-2 as well as the other FRs. Thus we do not incorporate them with the table in Appendix.

On the other hand, some UE features with per-band/BC signalling include a Note associated with FR and/or whether it is licensed or unlicensed. For example, FG22-6/7 on PUCCH grouping, there are the descriptions on carrier type, which differentiate between the existing FRs and licensed/unlicensed band for FR1. Here, whether “FR2” can include 52.6 – 71 GHz or not is unclear at this stage. Moreover, as 52.6 – 71 GHz includes unlicensed bands, differentiation between licensed/unlicensed band may be required even if the wording “FR2” is kept as it is.

Proposal 6: For Rel-15/16 UE features with per-FR capability signalling,

- If FR-related description is included in e.g., component, whether/how to consider 52.6 – 71 GHz may need to be discussed.

- Otherwise, as it can naturally differentiate FR2-2 from other FRs, there is no need to discuss in terms on FR2-2

The ones with per-FR capability signalling may not have any issue either since Rel-15/16 defines FR2 as a frequency range between 24.25 – 52.6 GHz. Also, even if FR2-2 is additionally considered, as well as FR1/2-1 differentiation which has already been done via per-FR capability signalling, FR2-2 will need to be differentiated from the other FRs in many cases. Given that, we have not analysed yet on the ones with per-FR capability signalling on the tables in Appendix.

An issue which may be lying on the ones with per-FR capability signalling would be whether to be applicable when they are reported as applicable to FR2 if no differentiation between FR2-1 and FR2-2 is considered. Some could be applicable to FR2-2 in the same manner as to FR2-1, while some others may not. This issue may also need to be checked in a case-by-case basis. We think it should also be discussed in RAN1 in the future.

Proposal 7: For Rel-15/16 UE features with per-FR capability signalling, how to treat when it is reported as applicable to FR2 should be discussed

- Option 1: Differentiation between FR2-1 and FR2-2 is introduced

- Option 2: All the UE features are treated as applicable or inapplicable to FR2-2 as well as FR2-1 when it is reported for FR2, while the ones for which such treatment cannot be appropriate are defined as exceptional cases via e.g., adding Note

For the ones with per-band signalling, at least how to treat the ones related to Rel-16 NR-U is worth more clarification in our view. For example, FG10-2 is defined for “SSB-based RRM with Q with dynamic channel access mode”, which is the same functionality as DBTW to be supported for FR2-2. Thus, it can be reused to report that a UE supports RRM with DBTW in FR2-2 by reporting FG10-2 with a band in FR2-2. On the other hand, there has already been some new FGs agreed for FR2-2, which is the same as (or similar to) the existing one for Rel-16 NR-U, e.g., multi-PUSCH scheduling. To align with how to treat Rel-16 NR-U FGs, all the functionalities supported for FR2-2 unlicensed band need to be re-defined, even if the same (or similar) FG has been defined in Rel-16 NR-U already. We believe this aspect should be clarified more.

Proposal 8: How to treat Rel-15/-16 UE features with per-band (at least the ones defined for Rel-16 NR-U) should be clarified.

- Alt-1: The existing FG (e.g., FG10-2 for RRM with DBTW) is reused to report that the UE supports it in FR2-2 by indicating for a band in FR2-2.

- Alt-2: A dedicated FG is newly defined for any functionality supported in FR2-2, even if the same functionality has already been defined for Rel-15/-16

- Conclusion

Our proposals/observations are summarized as follows:

Proposal 1: For the discussion on Rel-17 UE features at least regarding 52.6 – 71 GHz WI, the following alternatives can be considered in case-by-case manner, in terms of FR differentiation.

- Alt 1: define as per-band

- Alt 2: define as per-FR

  - Differentiation of FR2-1/2-2 may or may not be needed

- Alt 3: define as per-UE

  - A fixed limitation (e.g., as a Note) on applicable frequency range may be needed

Proposal 2: FG24-8 and 24-9 are not merged with FG26-5 defined in NR NTN

Proposal 3: Update the latest UE feature list on 52.6 – 71 GHz as shown in the following Table 2.2-1

Proposal 4: Adde new FGs for HARQ-ACK bundling, e.g., as in Table 2.2-2

- It should be per SCS

- It can be per type of HARQ-ACK codebook

Proposal 4: For UEs supporting NR in 52.6 – 71 GHz frequency range, how to treat a mandatory UE feature, FG 3-1, should be discussed at least when the UE supports the operation with 480 and/or 960 kHz SCS

Observation 1: While most of Rel-15/16 UE features with per-UE capability signalling can be reused as they are for UE to report their support for NR in 52.6 – 71 GHz, some maintenances will be required in the specifications to support the functionalities in practice.

Proposal 5: For Rel-15/16 UE features with per-UE capability signalling, whether to be applicable to FR2-2 when they are reported as applicable should be analysed a case-by-case manner

Proposal 6: For Rel-15/16 UE features with per-FR capability signalling,

- If FR-related description is included in e.g., component, whether/how to consider 52.6 – 71 GHz may need to be discussed.

- Otherwise, as it can naturally differentiate FR2-2 from other FRs, there is no need to discuss in terms on FR2-2

Proposal 7: For Rel-15/16 UE features with per-FR capability signalling, how to treat when it is reported as applicable to FR2 should be discussed

- Option 1: Differentiation between FR2-1 and FR2-2 is introduced

- Option 2: All the UE features are treated as applicable or inapplicable to FR2-2 as well as FR2-1 when it is reported for FR2, while the ones for which such treatment cannot be appropriate are defined as exceptional cases via e.g., adding Note

Proposal 8: How to treat Rel-15/-16 UE features with per-band (at least the ones defined for Rel-16 NR-U) should be clarified.

- Alt-1: The existing FG (e.g., FG10-2 for RRM with DBTW) is reused to report that the UE supports it in FR2-2 by indicating for a band in FR2-2.

- Alt-2: A dedicated FG is newly defined for any functionality supported in FR2-2, even if the same functionality has already been defined for Rel-15/-16

- Appendix

4.2. Rel-15 UE feature list for supporting NR from 52.6 to 71 GHz

4.1.1. Initial access and mobility

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 1-1 | Basic initial access channels and procedures |  | n/a | n/a | Ok with no change |
| 1-2 | SS block based SINR measurement (SS-SINR) | 1-1 | ss-SINR-Meas | MeasAndMobParametersFRX-Diff |  |
| 1-3 | SS block based RLM | 1-1 | ssb-RLM | MeasAndMobParametersCommon | Ok to keep common (for licensed band only) |
| 1-4 | CSI-RS based RRM measurement with associated SS-block | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithSSB | MeasAndMobParametersFRX-Diff | (for licensed band only) |
| 1-5 | CSI-RS based RRM measurement without associated SS-block | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithoutSSB | MeasAndMobParametersFRX-Diff | (for licensed band only) |
| 1-6 | CSI-RS based RS-SINR measurement | 1-1, 1-4 | csi-SINR-Meas | MeasAndMobParametersFRX-Diff | (for licensed band only) |
| 1-7 | CSI-RS based RLM | 1-1, CSI-RS | csi-RS-RLM | MeasAndMobParametersFRX-Diff | (for licensed band only) |
| 1-8 | RLM based on a mix of SS block and CSI-RS signals within active BWP | 1-3 and 1-7 | ssb-AndCSI-RS-RLM | MeasAndMobParametersCommon | Ok to keep common (for licensed band only) |
| 1-9 | CSI-RS based contention free RA for HO | 1-1, CSI-RS, 1-4 or 1-5 | csi-RS-CFRA-ForHO | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 1-12 | E-UTRA RS-SINR measurement |  | rs-SINR-MeasEUTRA | EUTRA-ParametersCommon | Ok to keep common |
| 1-13 | Maximal number of CSI-RS resources for RRM and RS-SINR measurement across all measurement frequencies per slot | 1-4 or 1-5 or 1-6 | maxNumberCSI-RS-RRM-RS-SINR | MeasAndMobParametersCommon | Ok to keep common |
| 1-14 | Maximal number of CSI-RS resources within a slot per PCell/PSCell for CSI-RS based RLM | 1-7 or 1-8 | maxNumberResource-CSI-RS-RLM | MeasAndMobParametersFRX-Diff |  |

4.1.2. MIMO

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 2-1 | Basic PDSCH reception |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-5 | Basic downlink DMRS for scheduling type A | 2-1 | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-6 | Basic downlink DMRS for scheduling type B |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-7 | Supported 2 symbols front-loaded DMRS (downlink) | 2-5 | twoFL-DMRS (MSB) | Phy-ParametersFRX-Diff |  |
| 2-10 | Support DMRS type (downlink) |  | supportedDMRS-TypeDL | Phy-ParametersFRX-Diff |  |
| 2-11 | Downlink dynamic PRB bundling (downlink) | 2-1 | dynamicPRB-BundlingDL | Phy-ParametersCommon | Ok to keep common |
| 2-12 | Basic PUSCH transmission |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-16 | Basic uplink DMRS (uplink) for scheduling type A |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-16a | Basic uplink DMRS for scheduling type B |  | n/a | n/a | Ok to keep mandatory w/o capability signaling |
| 2-16b | Support 1+2 DMRS (uplink) | 2-16a and 2-16 | oneFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff |  |
| 2-17 | Support DMRS type (uplink) | 2-16 | supportedDMRS-TypeUL | Phy-ParametersFRX-Diff |  |
| 2-18 | Supported 2 symbols front-loaded DMRS (uplink) | 2-16 | twoFL-DMRS (LSB) | Phy-ParametersFRX-Diff |  |
| 2-18a | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) | 2-16 | twoFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff |  |
| 2-19 | Support 1+3 uplink DMRS symbols(uplink) | 2-16 | oneFL-DMRS-ThreeAdditionalDMRS-UL | Phy-ParametersFRX-Diff |  |
| 2-32 | Basic CSI feedback |  | n/a | n/a | Ok to keep (but needs further check) |
| 2-32a | Semi-persistent CSI report on PUCCH |  | sp-CSI-ReportPUCCH | Phy-ParametersCommon | Ok to keep common |
| 2-32b | Semi-persistent CSI report on PUSCH |  | sp-CSI-ReportPUSCH | Phy-ParametersCommon | Ok to keep common |
| 2-32c | New CQI table |  | cqi-TableAlt | Phy-ParametersFRX-Diff |  |
| 2-33a | Supported PDSCH RE-mapping patterns |  | 1. pdsch-RE-MappingFR1-PerSymbol 2. pdsch-RE-MappingFR1-PerSlot | Phy-ParametersFR1 |  |
|  |  |  | 1. pdsch-RE-MappingFR2-PerSymbol 2. pdsch-RE-MappingFR2-PerSlot | Phy-ParametersFR2 | Ok to keep the whole FR2 |
| 2-33b | SP CSI-RS | 2-1 | sp-CSI-RS | Phy-ParametersFRX-Diff |  |
| 2-33c | SP CSI-IM | 2-1 | sp-CSI-IM | Phy-ParametersFRX-Diff |  |
| 2-34 | NZP-CSI-RS based interference measurement | 2-33 | nzp-CSI-RS-IntefMgmt | Phy-ParametersCommon | Ok to keep common |
| 2-37 | Support Semi-open loop CSI | 2-35 | semiOpenLoopCSI | Phy-ParametersFRX-Diff |  |
| 2-38 | CSI report without PMI | 2-35 | csi-ReportWithoutPMI | Phy-ParametersFRX-Diff |  |
| 2-39a | CSI report without CQI | 2-35 | csi-ReportWithoutCQI | Phy-ParametersFRX-Diff |  |
| 2-42 | Support Type II SP-CSI feedback on long PUCCH | 2-41 | type2-SP-CSI-Feedback-LongPUCCH | Phy-ParametersCommon | Ok to keep common |
| 2-44 | Basic DL PTRS |  | onePortsPTRS (MSB) | Phy-ParametersFRX-Diff |  |
| 2-47 | Basic UL PTRS |  | onePortsPTRS (LSB) | Phy-ParametersFRX-Diff |  |
| 2-50 | Basic TRS |  | n/a | n/a | Ok to keep as mandatory w/o signaling |
| 2-52 | Basic SRS |  | n/a | n/a | Ok to keep as mandatory w/o signaling |

4.1.3. DL control channel and procedure

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 3-1 | Basic DL control channel |  | n/a | n/a | How to treat Component (2) and (6) should be discussed when larger SCSs are supported |
| 3-2 | PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot |  | pdcchMonitoringSingleOccasion | Phy-ParametersFR1 |  |
| 3-3 | More than one CORESET configurations per BWP in addition to CORESET0 |  | multipleCORESET | Phy-ParametersFRX-Diff |  |
| 3-6 | Dynamic SFI monitoring |  | dynamicSFI | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 3-7 | Precoder-granularity of CORESET size |  | precoderGranularityCORESET | Phy-ParametersCommon | Ok to keep as common |
| 3-8 | Up to 10 search spaces in a SCell |  | maxNumberSearchSpaces | Phy-ParametersCommon | Ok to keep as common |

4.1.4. UL control channel and procedure

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 4-1 | Basic UL control channel |  | n/a | n/a | Ok to keep mandatory without signaling |
| 4-2 | 2 PUCCH of format 0 or 2 in consecutive symbols |  | twoPUCCH-F0-2-ConsecSymbols | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 4-3 | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as "enabled" |  | pucch-F2-WithFH | Phy-ParametersFRX-Diff |  |
| 4-4 | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as "enabled" |  | pucch-F3-WithFH | Phy-ParametersFRX-Diff |  |
| 4-5 | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as "enabled" |  | pucch-F4-WithFH | Phy-ParametersFRX-Diff |  |
| 4-6 | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as "disabled" |  | freqHoppingPUCCH-F0-2 | Phy-ParametersFRX-Diff |  |
| 4-7 | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as "disabled" |  | freqHoppingPUCCH-F1-3-4 | Phy-ParametersFRX-Diff |  |
| 4-10 | Dynamic HARQ-ACK codebook |  | dynamicHARQ-ACK-Codebook | Phy-ParametersCommon | Ok to keep common |
| 4-11 | Semi-static HARQ-ACK codebook |  | semiStaticHARQ-ACK-Codebook | Phy-ParametersCommon | Ok to keep common |
| 4-12 | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group |  | spatialBundlingHARQ-ACK | Phy-ParametersCommon | Ok to keep common |
| 4-13 | More than one SR configurations per PUCCH group |  | multipleSR-Configurations | MAC-ParametersXDD-Diff |  |
| 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same starting symbol on the PUCCH resources in a slot |  | sameSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff |  |
| 4-19a | SR/HARQ-ACK multiplexing once per slot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent with different starting symbols in a slot | 4-19 | mux-SR-HARQ-ACK-PUCCH | Phy-ParametersFRX-Diff |  |
| 4-19b | SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same or different starting symbol in a slot | 4-19c | mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Phy-ParametersFRX-Diff |  |
| 4-19c | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with different starting symbols in a slot | 4-19a | diffSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff |  |
| 4-20 | UCI code-block segmentation |  | uci-CodeBlockSegmentation | Phy-ParametersFRX-Diff |  |
| 4-21 | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI |  | dynamicBetaOffsetInd-HARQ-ACK-CSI | Phy-ParametersCommon | Ok to keep common |
| 4-22 | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3, or 4 in the same slot |  | onePUCCH-LongAndShortFormat | Phy-ParametersFRX-Diff |  |
| 4-22a | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 |  | twoPUCCH-AnyOthersInSlot | Phy-ParametersFRX-Diff |  |
| 4-23 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 |  | pucch-Repetition-F1-3-4 | Phy-ParametersCommon | Ok to keep common |
| 4-27 | More than one group of overlapping channels for control multiplexing |  | mux-MultipleGroupCtrlCH-Overlap | Phy-ParametersFRX-Diff |  |
| 4-28 | HARQ-ACK multiplexing on PUSCH with different PUCCH/PUSCH starting OFDM symbols | 4-1 | mux-HARQ-ACK-PUSCH-DiffSymbol | Phy-ParametersFRX-Diff |  |

4.1.5. Scheduling/HARQ operation

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 5-1 | Basic scheduling/HARQ operation |  | n/a | n/a | Ok to keep as mandatory |
| 5-1b | More than one DL/UL switch point in a slot |  | tdd-MultiDL-UL-SwitchPerSlot | Phy-ParametersFRX-Diff |  |
| 5-2 | RA Type 0 for PUSCH |  | ra-Type0-PUSCH | Phy-ParametersCommon | Ok to keep common |
| 5-3 | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH |  | dynamicSwitchRA-Type0-1-PDSCH | Phy-ParametersCommon | Ok to keep common |
| 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH | 5-2 | dynamicSwitchRA-Type0-1-PUSCH | Phy-ParametersCommon | Ok to keep common |
| 5-6 | PDSCH mapping type A with less than 7 OFDM symbols |  | pdsch-MappingTypeA | Phy-ParametersCommon | Ok to keep common |
| 5-6a | PDSCH mapping type B |  | pdsch-MappingTypeB | Phy-ParametersCommon | Ok to keep common |
| 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH |  | interleavingVRB-ToPRB-PDSCH | Phy-ParametersCommon | Ok to keep common |
| 5-9 | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 CSS before RRC connection |  | intraSlotFreqHopping-PUSCH | Phy-ParametersFRX-Diff |  |
| 5-10 | Inter-slot frequency hopping for PUSCH |  | interSlotFreqHopping-PUSCH | Phy-ParametersCommon | Ok to keep common |
| 5-14 | Type 1 configured PUSCH repetitions over multiple slots |  | type1-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-16 | Type 2 configured PUSCH repetitions over multiple slots |  | type2-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-17 | PUSCH repetitions over multiple slots |  | pusch-RepetitionMultiSlots | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-17a | PDSCH repetitions over multiple slots |  | pdsch-RepetitionMultiSlots | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-18 | DL SPS |  | downlinkSPS | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-19 | Type 1 Configured UL grant |  | configuredUL-GrantType1 | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-20 | Type 2 Configured UL grant |  | configuredUL-GrantType2 | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-21 | Pre-emption indication for DL |  | pre-EmptIndication-DL | Phy-ParametersCommon | Ok to keep common (for licensed band only) |
| 5-22 | CBG-based re-transmission for DL using CBGTI |  | cbg-TransIndication-DL | Phy-ParametersCommon | Ok to keep common |
| 5-23 | CBGFI for CBG-based re-transmission for DL | 5-22 | cbg-FlushIndication-DL | Phy-ParametersCommon | Ok to keep common |
| 5-24 | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL |  | dynamicHARQ-ACK-CodeB-CBG-Retx-DL | Phy-ParametersCommon | Ok to keep common |
| 5-25 | CBG-based re-transmission for UL using CBGTI |  | cbg-TransIndication-UL | Phy-ParametersCommon | Ok to keep common |
| 5-26 | Semi-static rate-matching resource set configuration for DL |  | rateMatchingResrcSetSemi-Static | Phy-ParametersCommon | Ok to keep common |
| 5-27 | Dynamic rate-matching resource set configuration for DL |  | rateMatchingResrcSetDynamic | Phy-ParametersCommon | Ok to keep common |
| 5-27a | Dynamic rate-matching control resource set for DL |  | rateMatchingCtrlResrcSetDynamic | Phy-ParametersCommon | Ok to keep common |
| 5-29 | LBRM for PUSCH |  | pusch-LBRM | Phy-ParametersFRX-Diff |  |
| 5-30 | DL scheduling slot offset greater than zero for PDSCH mapping type A |  | dl-SchedulingOffset-PDSCH-TypeA | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 5-30a | DL scheduling slot offset greater than zero for PDSCH mapping type B |  | dl-SchedulingOffset-PDSCH-TypeB | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 5-31 | UL scheduling slot offset greater than 12 |  | ul-SchedulingOffset | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 5-34 | New 64QAM MCS table for PDSCH |  | dl-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff |  |
| 5-34a | New 64QAM MCS table for PUSCH |  | ul-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff |  |

4.1.6. CA/DC, BWP, SUL

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 6-1 | Basic BWP operation with restriction |  | n/a | n/a | Ok to keep as mandatory without signaling |
| 6-5a | PDCCH blind detection capability for CA | 6-5 | pdcch-BlindDetectionCA | Phy-ParametersFRX-Diff | ok Note: FR differentiation is not supported now. |
| 6-25a | PDCCH blind detection capability for MCG and for SCG in synchronous NR-NR DC | 6-5, 6-25 | pdcch-BlindDetectionMCG-UE pdcch-BlindDetectionSCG-UE | Phy-ParametersFRX-Diff |  |

4.1.7. Channel coding

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 7-1 | Channel coding |  | n/a | n/a | Ok to keep as mandatory without signaling |

4.1.8. UL TPC

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 8-3 | Basic power control operation |  | n/a | n/a | Ok to keep as mandatory |
| 8-4 | TPC-PUSCH-RNTI |  | tpc-PUSCH-RNTI | Phy-ParametersFRX-Diff |  |
| 8-5 | TPC-PUCCH-RNTI |  | tpc-PUCCH-RNTI | Phy-ParametersFRX-Diff |  |
| 8-6 | TPC-SRS-RNTI |  | tpc-SRS-RNTI | Phy-ParametersFRX-Diff |  |
| 8-7 | Absolute TPC command mode |  | absoluteTPC-Command | Phy-ParametersFRX-Diff |  |
| 8-8 | UL power control with 2 PUSCH closed loops |  | twoDifferentTPC-Loop-PUSCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |
| 8-9 | UL power control with 2 PUCCH closed loops |  | twoDifferentTPC-Loop-PUCCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff |  |

4.2. Rel-16 UE feature list for supporting NR from 52.6 to 71 GHz

4.2.1 NR_2step_RACH

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 9-1 | Basic channel structure and procedure of 2-step RACH |  | twoStepRACH-r16 | Phy-ParametersCommon | Ok to keep common |

4.2.2 NR_unlicensed

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 10-32 | SS block based SINR measurement (SS-SINR) for unlicensed spectrum |  | ss-SINR-Meas-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-33 | Semi-persistent CSI report on PUCCH for unlicensed spectrum |  | sp-CSI-ReportPUCCH-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-33a | Semi-persistent CSI report on PUSCH for unlicensed spectrum |  | sp-CSI-ReportPUSCH-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-34 | Dynamic SFI monitoring for unlicensed spectrum |  | dynamicSFI-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-35 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same starting symbol on the PUCCH resources in a slot for unlicensed spectrum |  | mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot-r16 { sameSymbol-r16, diffSymbol-r16 } | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-35a | SR/HARQ-ACK multiplexing once per slot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent with different starting symbols in a slot for unlicensed spectrum | 10-35 | mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-35b | SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same or different starting symbol in a slot for unlicensed spectrum | 10-35c | mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-35c | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with different starting symbols in a slot for unlicensed spectrum | 10-35a | mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot-r16 { sameSymbol-r16, diffSymbol-r16 } | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-36 | HARQ-ACK multiplexing on PUSCH with different PUCCH/PUSCH starting OFDM symbols for unlicensed spectrum |  | mux-HARQ-ACK-PUSCH-DiffSymbol-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-37 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 for unlicensed spectrum |  | pucch-Repetition-F1-3-4-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-38 | Type 1 configured PUSCH repetitions over multiple slots for unlicensed spectrum |  | type1-PUSCH-RepetitionMultiSlots-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-39 | Type 2 configured PUSCH repetitions over multiple slots for unlicensed spectrum |  | type2-PUSCH-RepetitionMultiSlots-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-40 | PUSCH repetitions over multiple slots for unlicensed spectrum |  | pusch-RepetitionMultiSlots-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-40a | PDSCH repetitions over multiple slots for unlicensed spectrum |  | pdsch-RepetitionMultiSlots-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-41 | DL SPS for unlicensed spectrum |  | downlinkSPS-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-42 | Type 1 Configured UL grant for unlicensed spectrum |  | configuredUL-GrantType1-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-43 | Type 2 Configured UL grant for unlicensed spectrum |  | configuredUL-GrantType2-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |
| 10-44 | Pre-emption indication for DL for unlicensed spectrum |  | pre-EmptIndication-DL-r16 | Phy-ParametersSharedChAccess-r16 | Ok to keep applicable for the whole unlicensed band operation |

4.2.3 NR_L1enh_URLLC

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 |  | dci-Format1-2And0-2-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-1a | Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 11-1 | monitoringDCI-SameSearchSpace-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-1b | Type 1 HARQ-ACK codebook support for relative TDRA for DL | 11-1 | type1-HARQ-ACK-Codebook-r16 | Phy-ParametersFRX-Diff |  |
| 11-4b | DL priority indication in DCI with mixed DCI formats | 11-1, 11-4 | dci-DL-PriorityIndicator-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-6 | PUSCH repetition Type A | One of {5-16, 5-17] | pusch-RepetitionTypeA-r16 { sharedSpectrumChAccess-r16, non-SharedSpectrumChAccess-r16 } | Phy-ParametersCommon | Ok to keep common |
| 11-8 | Enhanced UL power control scheme |  | enhancedPowerControl-r16 | Phy-ParametersFRX-Diff |  |
| 11-10 | Type 2 configured grant release by DCI format 0_1 | 5-20 | type2-CG-ReleaseDCI-0-1-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-11 | Type 2 configured grant release by DCI format 0_2 | 5-20, 11-1 | type2-CG-ReleaseDCI-0-2-r16 | Phy-ParametersCommon | Ok to keep common |
| 11-12 | CBG-based re-transmission for UL using CBGTI with only in-order CBG-based re-transmission(s) for cancelled initial PUSCH transmission |  | cbg-TransInOrderPUSCH-UL-r16 | Phy-ParametersCommon | Ok to keep common |

4.2.4 NR_IIOT

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 12-1a | UL priority indication in DCI with mixed DCI formats | 12-1 and 11-1 | dci-UL-PriorityIndicator-r16 | Phy-ParametersCommon | Ok to keep common |
| 12-3 | SPS release by DCI format 1_1 | 5-18 DL SPS | sps-ReleaseDCI-1-1-r16 | Phy-ParametersCommon | Ok to keep common |
| 12-3a | SPS release by DCI format 1_2 | 5-18 DL SPS and 11-1 | sps-ReleaseDCI-1-2-r16 | Phy-ParametersCommon | Ok to keep common |
| 12-5 | Configuration of aggregation factor per SPS configuration | 5-18 DL SPS | aggregationFactorSPS-DL-r16 | Phy-ParametersFRX-Diff | Ok to keep common |
| 12-6 | Support of SPS periodicity shorter than 10 ms | 5-18 DL SPS | extendedSPS-Periodicities-r16 | Phy-ParametersCommon | Seems technically possible, but the specification needs to be updated to support larger SCSs |

4.2.5 NR positioning

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 13-1a | Max number of positioning frequency layers UE supports across all positioning methods across all bands |  | maxSupportedFreqLayers-r16 | NR-DL-PRS-ProcessingCapability-r16 LPP | Ok to keep common |
| 13-2 | DL PRS Resources for DL AoD | 13-1 | 1 maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 2 maxNrOfTRP-AcrossFreqs-r16 3 maxNrOfPosLayer-r16 | NR-DL-PRS-ResourcesCapability-r16 LPP | Ok to keep common |
| 13-3 | DL PRS Resources for DL-TDOA | 13-1 | 1 maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 2 maxNrOfTRP-AcrossFreqs-r16 3 maxNrOfPosLayer-r16 | NR-DL-PRS-ResourcesCapability-r16 LPP | Ok to keep common |
| 13-4 | DL PRS Resources for Multi-RTT | 13-1 | 1 maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 2 maxNrOfTRP-AcrossFreqs-r16 3 maxNrOfPosLayer-r16 | NR-DL-PRS-ResourcesCapability-r16 LPP | Ok to keep common |
| 13-5 | DL PRS Measurement Report for DL-AoD | 13-2, | maxDL-PRS-RSRP-MeasurementFR1-r16 maxDL-PRS-RSRP-MeasurementFR2-r16 | NR-DL-AoD-MeasurementCapability LPP |  |
| 13-6 | DL PRS Measurement Report for DL-TDOA | 13-3 | dl-RSTD-MeasurementPerPairOfTRP-FR1-r16 dl-RSTD-MeasurementPerPairOfTRP-FR2-r16 | NR-DL-TDOA-MeasurementCapability-r16 LPP |  |
| 13-9f | PathLoss estimate maintenance across all cells | One of {13-9, 13-9a, 13-9b, 13-9c} | LPP maxNumberSRS-PosPathLossEstimateAllServingCells-r16 RRC maxNumberSRS-PosPathLossEstimateAllServingCells-r16 | LPP NR-UL-SRS-Capability-r16 RRC Phy-ParametersCommon |  |
| 13-10f | Spatial relation maintenance | One of {13-10, 13-10a, 13-10b, 13-10d, 13-10e} | LPP maxNumberSRS-PosSpatialRelationsAllServingCells-r16 RRC maxNumberSRS-PosSpatialRelationsAllServingCells-r16 | LPP NR-UL-SRS-Capability-r16 RRC Phy-ParametersFR2 |  |
| 13-11a | Association between SRS for positioning and DL PRS for Multi-RTT | 13-4 and 13-8 | LPP srs-AssocPRS-MultiLayersFR1-r16 srs-AssocPRS-MultiLayersFR2-r16 | LPP NR-Multi-RTT-MeasurementCapability-r16 | Ok to keep common |
| 13-11 | UE Rx-Tx Measurement Report for Multi-RTT | 13-4 and 13-8 | LPP 1 maxNrOfRx-TX-MeasFR1-r16 maxNrOfRx-TX-MeasFR2-r16 2 supportOfRSRP-MeasFR1-r16 supportOfRSRP-MeasFR2-r16 | LPP NR-Multi-RTT-MeasurementCapability-r16 |  |
| 13-12 | SS-RSRP RRM measurements for NR E-CID Positioning | 1-1 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | Ok to keep common |
| 13-12a | SS-RSRQ RRM measurements for NR E-CID Positioning | 1-1 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | Ok to keep common |
| 13-12b | CSI-RSRP RRM measurements for NR E-CID Positioning | 1-4 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | Ok to keep common |
| 13-12c | CSI-RSRQ RRM measurements for NR E-CID Positioning | 1-4 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | Ok to keep common |
| 13-18 | Support of parallel processing of LTE PRS and NR PRS |  | simulLTE-NR-PRS-r16 | NR-DL-PRS-ProcessingCapability-r16 LPP | Ok to keep common |

4.2.6 NR_TEI

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 14-6 | New RACH configuration for FR1 TDD |  |  |  | Ok to keep mandatory without signaling |
| 14-8 | CSI trigger states containing non-active BWP |  | csi-TriggerStateNon-ActiveBWP-r16 | Phy-ParametersCommon | Ok to keep common |

4.2.7 NR_eMIMO

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 16-1b-1 | TCI state activation across multiple CCs | Component 1: 2-1, 2-4 | simultaneousTCI-ActMultipleCC-r16 | Phy-ParametersFRX-Diff |  |
| 16-1b-2 | Spatial relation update across multiple CCs | Component 1: 2-59, 2-60 | simultaneousSpatialRelationMultipleCC-r16, cli-RSSI-FDM-DL-r16, cli-SRS-RSRP-FDM-DL-r16 | Phy-ParametersFRX-Diff |  |
| 16-1c | Default spatial relation | 2-53, 2-59 | defaultSpatialRelationPathlossRS-r16 | Phy-ParametersFR2 | Ok to keep common in FR2 |
| 16-1d | MAC CE spatial relation update for AP-SRS | 2-53, 2-59 | spatialRelationUpdateAP-SRS-r16, maxNumberSRS-PosSpatialRelationsAllServingCells-r16 | Phy-ParametersFR2 | Ok to keep common in FR2 |
| 16-1e | Pathloss reference RS activation via MAC CE | 8-3 | maxNumberPathlossRS-Update-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-1g | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification | 2-24, 2-31 | maxTotalResourcesForOneFreqRange-r16 { maxNumberResWithinSlotAcrossCC-OneFR-r16, maxNumberResAcrossCC-OneFR-r16} | Phy-ParametersFRX-Diff |  |
| 16-1g-1 | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification across frequency ranges | 2-24, 2-31, 16-1g | maxTotalResourcesForAcrossFreqRanges-r16 { maxNumberResWithinSlotAcrossCC-AcrossFR-r16, maxNumberResAcrossCC-AcrossFR-r16} | Phy-ParametersCommon | Ok to keep common |
| 16-1j-1 | 2 port CSI -RS for new beam identifications |  | newBeamIdentifications2PortCSI-RS-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-1j-2 | 2 port CSI -RS for pathloss estimation |  | pathlossEstimation2PortCSI-RS-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | 16-2a | harqACK-separateMultiDCI-MultiTRP-r16 { maxNumberLongPUCCHs-r16 } | Phy-ParametersCommon | Ok to keep common |
| 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | 16-2a | harqACK-jointMultiDCI-MultiTRP-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-2a-8 | Indicates that retransmission scheduled by a different CORESETPoolIndex for multi-DCI multi-TRP is not supported. | 16-2a | supportRetx-Diff-CoresetPool-Multi-DCI-TRP-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | 2-32 | csi-ReportFrameworkExt-r16 | Phy-ParametersFRX-Diff AND MIMO-ParametersPerBand |  |
| 16-x RAN2 | Mulit-CC simultaneous TCI activation with multi-TRP | If the UE indicates support of 16-1b-1 for a FR and support of at least one of 16-2b-1, 16-2b-2, 16-2b-3, 16-2b-4 or 16-2b-5 for at least one band or component carrier of this FR, the UE shall indicate support of 16-x for this FR | twoTCI-Act-servingCellInCC-List-r16 | Phy-ParametersFRX-Diff |  |
| 16-y RAN2 | Slot based repetition | 16-2b-5 and maxNumberTCI-states-r16 is set to 2 for at least one band | supportRepetitionZeroOffsetRV-r16 | Phy-ParametersCommon | Ok to keep common |
| 16-z RAN2 | spCell-BFR-CBRA-r16 |  | spCell-BFR-CBRA-r16 | BeamFailureRecoveryConfig | Ok to keep common |

4.2.8 NR_CLI_RIM

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 17-1 | CLI-RSSI measurement |  | cli-RSSI-Meas-r16 maxNumberCLI-RSSI-r16 | MeasAndMobParametersFRX-Diff |  |
| 17-2 | SRS-RSRP measurement |  | cli-SRS-RSRP-Meas-r16 maxNumberCLI-SRS-RSRP-r16 maxNumberPerSlotCLI-SRS-RSRP-r16 | MeasAndMobParametersFRX-Diff |  |
| 17-3 | Simultaneous reception of DL signals/channels and CLI-RSSI measurement resource | 17-1 | cli-RSSI-FDM-DL-r16 | Phy-ParametersFRX-Diff |  |
| 17-4 | Simultaneous reception of DL signals/channels and SRS-RSRP measurement resource | 17-2 | cli-SRS-RSRP-FDM-DL-r16 | Phy-ParametersFRX-Diff |  |

4.2.9 MR-DC/CA enhancement

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | 6-7 | harqACK-CB-SpatialBundlingPUCCH-Group-r16 | Phy-ParametersCommon | Ok to keep common |
| 18-9 | Type2 HARQ-ACK codebook for >1 unicast DL DCIs in same Monitoring Occasion | 3-1 | type2-HARQ-ACK-Codebook-r16 | Phy-ParametersCommon | Ok to keep common |
| 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern in case of FDD PCell | One of {18-2a, 18-3} | fdd-PCellUL-TX-AllUL-Subframe-r16 | Phy-ParametersMRDC | FR1 only |
| 18-3b | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern in case of TDD PCell | 18-2 | tdd-PCellUL-TX-AllUL-Subframe-r16 | Phy-ParametersMRDC | FR1 only |

4.2.10. UE power saving

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 19-1 | DRX Adaptation | N/A | drx-Adaptation-r16 { non-SharedSpectrumChAccess-r16 MinTimeGap-r16, sharedSpectrumChAccess-r16 MinTimeGap-r16 } | MAC-ParametersFRX-Diff-r16 |  |
| 19-2 | Cross Slot Scheduling |  | crossSlotScheduling-r16 { non-SharedSpectrumChAccess-r16, sharedSpectrumChAccess-r16 } | Phy-ParametersCommon | ok |
| 19-3 | Maximum MIMO Layer Adaptation | See Note | maxLayersMIMO-Adaptation-r16 | Phy-ParametersFRX-Diff |  |
| 19-4a | UE assistance information | 19-2 | minSchedulingOffsetPreference-r16 | PowSav-ParametersCommon-r16 | Seems technically possible, but the specification needs to be updated to support larger SCSs |

4.2.11. NR_IAB

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration |  | seperateSMTC-InterIAB-Support-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources |  | seperateRACH-IAB-Support-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-5a | UL-Flexible-DL slot formats | 5-1a | ul-flexibleDL-SlotFormatSemiStatic-IAB-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-5b | UL-Flexible-DL slot formats | 3-6 | ul-flexibleDL-SlotFormatDynamics-IAB-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-6 | Dynamic indication of soft resource availability |  | dci-25-AI-RNTI-Support-IAB-r16 | Phy-ParametersCommon | Ok to keep common |
| 20-7 | Case 1 OTA timing alignment |  | t-DeltaReceptionSupport-IAB-r16 | t-DeltaReceptionSupport-IAB-r16 | Ok to keep common |
| 20-8 | Guard symbols |  | guardSymbolReportReception-IAB-r16 | t-DeltaReceptionSupport-IAB-r16 | Ok to keep common |

4.2.12. New FGs that are not dedicated to a specific Rel-16 work item/TEI

| Index | FG | Prerequisite feature groups | Field name | Parent IE | Assessment |
| --- | --- | --- | --- | --- | --- |
| 22-10 | Support of pdcch-MonitoringAnyOccasionsWithSpanGap in case of cross-carrier scheduling with different SCSs in the scheduling cell and the scheduled cell | 3-5b, 18-5 | pdcch-MonitoringAnyOccasionsWithSpanGapCrossCarrierSch-r16 | Phy-ParametersCommon | Depending on 8.2.2 discussion |
| 22-11 | Support of 'cri-RI-CQI' report without non-PMI-PortIndication | 2-35 | cri-RI-CQI-WithoutNon-PMI-PortInd-r16 | Phy-Parameters |  |

- References

- R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), 3GPP RAN1#107bis-e, January 2022

- TR 38.822 V16.1.0 (2021-09), “User Equipment (UE) feature list”, 3GPP