3GPP TSG RAN WG1 #108-e			R1-2201506

e-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for RedCap

Agenda Item:	8.16.6

Document for: 	Discussion and Decision

- Introduction

The updated RAN1 UE features list for Rel-17 NR after RAN1#107bis-e is available in [1]. In this contribution, we present our views regarding the UE features for RedCap.

- Rel-17 UE features for RedCap UE

- FG 28-1: RedCap UE

  - Regarding FFS whether to add any other basic features for RedCap UE,

    - Following capabilities can be added as components in FG 28-1 based on the agreements in RAN1#107-e [2]:

      - Separate initial DL BWP for RedCap UEs

      - Separate initial UL BWP for RedCap UEs

      - Active DL BWP with CD-SSB or NCD-SSB

    - RAN2 made following agreement highlighted in yellow in RAN2#116bis-e [3]:

Agreements via email - via offline 105 second round:

- Capture “Support of RedCap early indication based on Msg1, MsgA and Msg3 for RACH” in the field description of capability bit “support of RedCap”;

2.	Capture the limitation on BW, Rx and MIMO in 4.2.xx RedCap Parameters of TS38.306 running CR as:

-	The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2; UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs;

-	1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported. UE features and corresponding capabilities related to more than 2 UE Rx branches and more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 2 UE Tx branches and more than 2 UL MIMO layers are not supported by RedCap UEs;

      - Therefore, following capabilities can be added as components in FG 28-1

        - Early indication of RedCap UE in Msg.3 for 4-step RACH

        - Early indication of RedCap UE in Msg.A preamble part for 2-step RACH

          - Conditioned on support of 2-step RACH

        - Early indication of RedCap UE in Msg.A PUSCH part for 2-step RACH

          - Conditioned on support of 2-step RACH

  - Report type of FG 28-1 should be per UE without FDD/TDD or FR1/FR2 differentiation

    - We don’t think a UE operates as RedCap UE in a band but as non-RedCap UE in another band. UE can report its supported bands where it can operate as RedCap UE. Also, RAN2 made following WA in RAN2#116bis-e [3]

Working Assumption:

- The capability “support of RedCap” is per UE capability. Take a final agreement in the next meeting based on possible further feedback from RAN1

  - Note of FG 28-1 can be kept

- FG 28-2: Number of UE Rx branches and DL MIMO layers for RedCap UE

  - An reply LS to RAN2 [4] was agreed in RAN1#107-e to inform that “From RAN1 perspective, it would be enough to indicate the maximum number of PDSCH MIMO layers per band for RedCap UEs, but RAN1 notes that the type of FG2-3 (maxNumberMIMO-LayersPDSCH) is currently per FSPC and that it is up to RAN2 whether to signal per band or per FSPC”.

  - RAN2 made following agreement highlighted in yellow in RAN2#116bis-e that maxNumberMIMO-LayersPDSCH is still per FSPC for RedCap UEs [3]. Therefore, no further discussion is necessary for FG 28-2

Agreements via email - from offline 105:

- ANR feature is optional for RedCap UE;

- CHO related capabilities are applicable for RedCap UEs (understanding that CHO is already defined as an optional feature). “FFS on CHO” can be removed.

- RAN2 confirms RAN1 agreements, i.e. introduce explicit bit to indicate the support of RedCap; To be captured in Mega CR;

- RAN2 confirms RAN1 agreements, i.e. introduce capability bit on Half-duplex FDD operation type A for RedCap UEs; To be captured in Mega CR.

- RAN2 confirms that for RedCap UEs, “maxNumberMIMO-LayersPDSCH ” is still per FSPC although per band is enough.

- Clarify in the field description of shortSN and am-WithShortSN that, RedCap UE should always report "1" in TS 38.306 section 4.2.4 and 4.2.5.

- FG 28-3: Half-duplex FDD operation type A for RedCap UE

  - Report type of FG 28-3 should be per band for more flexibility to indicate the support of either HD-FDD or FD-FDD for different FDD bands.

    - FG 28-3 is applicable only to FR1 FDD bands

- New FG for active DL BWP without CD-SSB or NCD-SSB

  - Based on the agreements in RAN1#107-e [2], active DL BWP without CD-SSB or NCD-SSB is optional capability. We are fine with either defining a new FG for RedCap or reusing existing FG 6-1a (BWP operation without restriction on BW of BWP(s)).

  - Report type of this FG should be per UE with FR1/FR2 differentiation or per band for more flexibility to indicate the support of this BWP operation for different FR.

- Rel-15/16 UE features not applicable for RedCap UEs

It was discussed in the last RAN1 meeting [5] whether there are any Rel-15/16 UE features which should not be applicable for RedCap UEs other than the identified capabilities, i.e.,

- Carrier aggregation

- Dual connectivity

- UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2

- More than 2 UE Rx branches or more than 2 DL MIMO layers

- More than [4 or 8] DRBs

- NE-DC and (NG)EN-DC

- DAPS and CPAC related capabilities

- IAB related capabilities

and following agreements were made:

Agreement: The following Rel-15/16 capabilities (FGs) for L1 UE features in TR 38.822 V16.1.0 are related to more than 2 UE Rx branches or more than 2 DL MIMO layers and should therefore not be applicable to RedCap UEs. - 4-12 - 16-3a-3 - 16-3b-2 Agreement: - A RedCap UE does not support capabilities related to more than 2 UE Tx branches or more than 2 UL MIMO layers. Agreement: - RAN1 does not provide a complete list of Rel-15/16 capabilities (FGs) for L1 UE features in TR 38.822 V16.1.0 that should not be applicable to RedCap UEs because they are related to IAB. Agreement: Capture the following earlier RAN1 agreements regarding RF/RRM FGs 1-4 and 1-5 in the LS reply to RAN2: - RedCap UE supports FG1-4 (256QAM for PDSCH) as optional with capability signalling both for FR1 and FR2 - Add a note in FG 1-4 (256QAM for PDSCH) that “For RedCap UEs, the 256QAM MCS table for PDSCH and CQI table 2 are only supported if the UE supports 256QAM for PDSCH” - Add a note in FG 1-5 (256QAM for PUSCH) that “For RedCap UEs, the 256QAM MCS table for PUSCH is only supported if the UE supports 256QAM for PUSCH”

Also, it was discussed in RAN#93-e meeting whether or not RedCap UEs can support V2X, operate in unlicensed bands, and support SUL. It was concluded that there will be no work on any RedCap specific specification update for these features but the specification will not contain any explicit restriction to prevent implementation of RedCap UEs with these features [6]. Therefore, V2X, NR-U, and SUL related L1 UE features are applicable for RedCap UEs without any changes.

For mandatory features for non-RedCap UEs that should not be applicable for RedCap UEs, following FGs were discussed but no consensus was achieved.

- 2-16b: Support 1+2 DMRS (uplink)

  - This FG is applicable for the RedCap UE supporting more than one port uplink. Therefore, this FG can be supported by RedCap UEs as optional with capability signaling

- 2-55: SRS Tx switch

  - Components 2 and 3 are not applicable for RedCap UEs

For mandatory features for non-RedCap UEs that should be optional for RedCap UEs, following FGs were discussed but no consensus was achieved.

- 1-7: CSI-RS based RLM

  - We don’t see any reason for optional support of this FG. Keep mandatory support as legacy.

- 2-4a: Additional active TCI state for PDCCH

  - We don’t see any reason for optional support of this FG. Keep mandatory support as legacy.

- 2-61: Additional active spatial relation for PUCCH

  - We don’t see any reason for optional support of this FG. Keep mandatory support as legacy.

For mandatory features for non-RedCap UEs that should be supported for RedCap UEs but with different value, following FGs were discussed but no consensus was achieved.

- 2-33: CSI-RS and CSI-IM reception for CSI feedback

  - Components 4 and 6 are not applicable for RedCap UEs

- 2-35: CSI report framework

  - Component 9 is not applicable for RedCap UEs

- 2-51: TRS (CSI-RS for tracking)

  - Component 4 is not applicable for RedCap Ues

- 6-1: Basic BWP operation with restriction

  - We don’t see any reason for supporting different value. Keep mandatory support as legacy.

For optional features for non-RedCap UE that should not be applicable for RedCap UEs, following FGs were discussed but no consensus was achieved.

- 2-13: Codebook based PUSCH MIMO transmission

  - This FG is applicable for the RedCap UE supporting more than one port uplink.

- 2-14: non-codebook based PUSCH transmission

  - This FG is applicable for the RedCap UE supporting more than one port uplink.

- 4-27: More than one group of overlapping channels for control multiplexing

  - This FG should not be applicable for RedCap Ues

- 5-5b: UE PDSCH processing capability #2 with scheduling limitation for 30kHz-SCS

  - Component 2 does not have dedicated capability bit, and some clarification note would be enough for RedCap UEs not to support max BW of 136 PRBs

- 10-20: Support search space set configuration with freqMonitorLocation-r16

  - This FG is not applicable for RedCap UEs since only one RB set is defined for RedCap UEs (maximum 20 MHz BW for FR1 unlicensed bands)

- 10-20a: Support coreset configuration with rb-Offset

  - We don’t see any reason for not supporting this FG. Keep optional support as legacy.

- 10-29: Support available RB set indicator field in DCI 2_0

  - This FG is not applicable for RedCap UEs since only one RB set is defined for RedCap UEs (maximum 20 MHz BW for FR1 unlicensed bands)

- 15-18: Support of rank 2 transmission

  - This FG is applicable for the RedCap UE supporting more than one port Tx.

- 15-19: Support of rank 2 reception

  - This FG is applicable for the RedCap UE supporting more than one port Rx.

For optional features for non-RedCap UE that should be mandatorily supported for RedCap UEs, following FGs were discussed but no consensus was achieved.

- 5-17a: PDSCH repetitions over multiple slots

  - Keep optional support as legacy

- 6-1a: BWP operation without restriction on BW of BWP(s)

  - As mentioned in Section 2, we are fine with either defining a new FG for active DL BWP without CD-SSB or NCD-SSB for RedCap UEs or reusing existing FG 6-1a as optional capability

References

- Moderators (AT&T, NTT DOCOMO, INC.), R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, Jan. 2022.

- Rapporteur (Ericsson), R1-2112506, RAN1 agreements for Rel-17 NR RedCap, Nov. 2021.

- Vice Chairman (ZTE Corporation), R2-2201662, Report from Break-out session on R17 NTN, REDCAP and CE, Jan. 2022.

- RAN1, R1-2112754, Reply LS on capability related RAN2 agreements for RedCap, Nov. 2021.

- Moderator (Ericsson), R1-2112503, FL summary on LS on capability related RAN2 agreements for RedCap, Nov. 2021.

- Intel, RP-212634, Moderator's summary for discussion [93e-16-RedCap-WI], Sept. 2021.