3GPP TSG RAN WG1 #108-e			R1-2201513

e-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on other UE feature related discussions

Agenda Item:	8.16.17

Document for: 	Discussion and Decision

- Introduction

The updated RAN1 UE features list for Rel-17 NR after RAN1#107bis-e has been agreed in [1]. In this contribution, we present our views regarding other UE feature related discussions.

- Discussion

- Whether to introduce FG 37-1 as “Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA”

  - The relationship between proposed new FG 37-1 and FG 22-1 for Rel-16 UL Tx switching has been discussed [2]. It was argued by proponents of new FG 37-1 that UE supporting both option 1 and option 2 for FG 22-1 (Rel-16 UL Tx switching between case 1 and 2) may not support one of the options for FG 37-1 (Rel-17 UL Tx switching among case 1, 2 and 3), and hence having FG 37-1 is beneficial. However, some other companies commented that UE supporting Rel-16 UL Tx switching option 1 (or option 2) and 2Tx-2Tx should also support Rel-17 UL Tx switching option 1 (or option 2), and hence the indication of supported option in FG 22-1 can be reused for Rel-17 UL Tx switching.

  - Since Rel-17 UL Tx switching requires additional implementation compared with Rel-16 UL Tx switching, at least separate capabilities on the support of Rel-17 UL Tx switching and the support of Rel-16 UL Tx switching are necessary, and it may be covered by capabilities agreed in RAN2. So, here we can focus on whether supporting the same option as in Rel-16 is mandatory for Rel-17 UL Tx switching or not. Although we don’t have a strong view on it, we are fine to not mandate it as there are several companies supporting to have flexibility on the support of different options between Rel-16 (1Tx-2Tx) and Rel-17 (2Tx-2Tx) UL Tx switching. As Rel-17 is more advanced feature, supporting the same option as in Rel-17 can be mandatory for Rel-16 UL Tx switching.

  - Therefore, we think that FG 37-1 can be defined as optional with capability signalling to report the supported option(s) for Rel-17 2Tx-2Tx UL Tx switching, where FG 22-1 is prerequisite FG for the FG 37-1 and the type is per BC (FR1 only). The UE’s supported option(s) for Rel-17 2Tx-2Tx UL Tx switching in FG 37-1 should also be supported for FG 22-1.

| 37. NR_RF_FR1_enh | 37-1 | Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA | Indicating supported option for 2Tx-2Tx UL Tx switching for inter-band UL CA - Candidate values set is {option1, option2, both option 1 and option 2} | 22-1 | Yes | N/A |  | Per BC | N/A | N/A (FR1 only) | N/A | If UE supports Option1 for FG 37-1, the UE also needs to support Option1 for FG 22-1; If UE supports Option2 for FG 37-1, the UE also needs to support Option2 for FG 22-1; If UE supports both Option1 and Option2 for FG 37-1, the UE also needs to support both Option1 and Option2 for FG 22-1; | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1:

The new FG 37-1 is defined for “Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA”.

- Candidate values set is {option1, option2, both option1 and option2}

- The prerequisite FG is FG22-1

- The type of the FG is Per BC (FR1 only)

- The FG is optional with capability signalling

- The UE’s supported option(s) for Rel-17 2Tx-2Tx UL Tx switching in FG 37-1 should also be supported for FG 22-1

- Conclusion

In this contribution, we presented our views regarding the UE features for Rel-17 UL Tx switching, and following proposal was made.

Proposal 1:

The new FG 37-1 is defined for “Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA”.

- Candidate values set is {option1, option2, both option1 and option2}

- The prerequisite FG is FG22-1

- The type of the FG is Per BC (FR1 only)

- The FG is optional with capability signalling

- The UE’s supported option(s) for Rel-17 2Tx-2Tx UL Tx switching in FG 37-1 should also be supported for FG 22-1

References

- Moderators (AT&T, NTT DOCOMO, INC.), R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, Jan. 2022.

- Moderator (NTT DOCOMO, INC.), R1-2112147, Summary on other UE feature related discussions, Nov. 2021.