3GPP TSG RAN WG1 #108-e 	        		             R1-2201527 e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.10.2

Source: 	Samsung

Title: 	Enhancements to Timing, Power Control and CLI for NR IAB

Document for:	Discussion and Decision

Introduction

A new work item on “Enhancements to Integrated Access and Backhaul for NR” was approved in RAN#86 [1]. One of main objectives in the work item is to specify IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links to support simultaneous operations by IAB-node’s child and parent links as follows:

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In this contribution, we discuss remaining issues for timing to support simultaneous transmission and/or receptions of IAB in Rel-17 and provide our views.

Discussion

In RAN1#106b-e and RAN1#107-e, the following agreements were made for Case 6 timing:

Agreement

RAN1 to downselect in RAN1#107-e one of the following for an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode:

- Alt 1: no change or enhancement to the Rel-16 OTA synchronization specification is supported in Rel-17 for Case 6 timing.

- Alt 2: in Rel-17 the Rel-16 OTA synchronization specification is updated to support OTA synchronization for an IAB-node operating solely in Case 6 timing during IAB-MT Tx.

  - FFS range of T_delta.

NOTE: this is to provide a feasible solution to the RAN1#103-e agreement: “An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode”

Agreement

Select Alt 2 from the aforementioned RAN1#106b-e agreement without specification impact other than the following:

- Alt A: the T_delta range is updated to support Case 6 timing.

FFS: Update of one way delay estimation equation in TS38.213 subclause 14

A remaining aspect for Case 6 timing is to update the T_delta range and whether or not one way delay estimation equation in TS38.213 subclause 14 needs to be updated.

The current T_delta index range (0, 1, 2, ... , 1199) for Case 1 timing is specified in section 6.1.3.21 of TS38.321. Based on the current T_delta index range with 11 bits, there was a discussion about a possibility that T_delta index range is extended to 2047 for Case 6 timing in previous RAN1 meetings. In our understanding, it can be a possible approach to support Case 6 timing without the impacts on current T_delta MAC CE signaling and therefore, it is proposed to update max. value of 1199 in current T_delta index range to max. value of 2047.

Proposal 1: Support T_delta index range with max. value of 2047 in Rel-17.

Regarding the second issue about whether or not one way delay estimation equation in TS38.213 subclause 14 needs to be updated, NTA in Case 6 timing is not set by parent IAB anymore but, NTA should be assumed by the IAB node as a time difference between MT UL transmission (which should be set to DU DL transmission by the definition of Case 6 timing) and MT DL reception. As a result, the time difference between DU DL transmission and MT DL reception for Case 6 timing can be derived as -NTA,offset +2⋅Ndelta+2⋅Tdelta,index⋅Gstep⋅Tc and therefore, it can be reflected in TS38.213.

Proposal 2: For Case 6 timing, the time difference between DU DL transmission and MT DL reception is revised to -NTA,offset +2⋅Ndelta+2⋅Tdelta,index⋅Gstep⋅Tc.

In RAN1#106-e, the followings were agreed for Case 7 timing at the parent node:

Agreement

For Case 7 timing at a parent node, the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

- FFS range, granularity, and signaling details of the offset.

In RAN1#106b-e and RAN1#107-e, further agreements were made for Case 7 timing:

Agreement

Case 7 UL timing offset is indicated by the parent-node via MAC-CE.

Agreement

The granularity of Case 7 UL timing offset is the same as the UL TA granularity.

Agreement

The dynamic range of the MAC CE case #7 timing offset indication is 12 bits.

- FFS the numerical values of the endpoints of the range

A remaining aspect is about the numerical values of the endpoints of the range. There was an email discussion about the number of bits and offset range for Case 7 timing in RAN1#107-e. In our understanding, the offset can be derived as -TP +Tg=2∙Tdelta-TP  where TP is a propagation delay for parent IAB MT DL reception and Tg is a time difference between DU UL reception and DU DL transmission at parent IAB. Taking into account RAN4 feedback in Rel-16 about min. Tdelta and max. TA value in TS38.213, the following table can be derived and it can be a possible option for the offset range. Therefore, the offset range of [– 2,150,144, 0] at the max. can be supported for Case 7 timing.

| SCS [kHz] | min. Tdelta [Tc] | min. offset [Tc] | Range of offset [Tc] |
| --- | --- | --- | --- |
| 15 | –/2 – 70,528 | – 39,936 – 2∙70,528 – 1,969,152= – 2,150,144 | [– 2,150,144, 0] |
| 30 | –/2 – 35,328 | – 39,936 – 2∙35,328 – 984,576 = – 1,095,168 | [– 1,095,168, 0] |
| 60 | –/2 – 17,664 | – 39,936 – 2∙17,664 – 492,288 = – 567,552 | [– 567,552, 0] |
| 120 | –/2 – 8,816 | – 13,792 – 2∙8,816 – 246,144 = – 277,568 | [– 277,568, 0] |

Proposal 3: The offset range of [– 2,150,144, 0] is supported for Case 7 timing.

Conclusions

This contribution discusses necessary enhancements for NR IAB and then proposes the following depending on the discussion:

Proposal 1: Support T_delta index range with max. value of 2047 in Rel-17.

Proposal 2: For Case 6 timing, the time difference between DU DL transmission and MT DL reception is revised to -NTA,offset +2⋅Ndelta+2⋅Tdelta,index⋅Gstep⋅Tc.

Proposal 3: The offset range of [– 2,150,144, 0] is supported for Case 7 timing.

- References:

[1] RP-193251, New WID on Enhancements to Integrated Access and Backhaul, Qualcomm

[2] TS38.213, “NR; Physical layer procedures for control”