3GPP TSG RAN WG1 #108-e			R1-2201528

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.16.10

Source: 	Samsung

Title: 	UE features for NR IAB

Document for:	Discussion and Decision

- Introduction

In RAN1#107-e meeting, Rel-17 UE features for IAB enhancements were endorsed in [1]. In this contribution, we further discuss MT features for Rel-17 IAB based on [1] and provide our view.

- Discussion

Feature 31-1: Guard symbols

- 1) Support [Rel-17 DesiredGuardSymbols] reporting, 2) Support [Rel-17 ProvidedGuardSymbols] reception

It was agreed in RAN1#107-e that Rel-17 Guard symbols are required for the cases of Case#6/#7 MT TX to/from Case#1 DU TX/RX. It means Feature 20-7 (Case 1 OTA timing alignment) is needed as a pre-requisite for Feature 31-1. Therefore, it is proposed to add Feature 20-7 as a pre-requisite for Feature 31-1.

Proposal 1: Feature 20-7 is added as a pre-requisite for Feature 31-1.

Feature 31-4: Case 6 timing alignment

- Support Case 6 timing alignment indication reception

It was agreed in RAN1#107-e that the T_delta range is updated to support Case 6 timing. It means Feature 20-7 (Case 1 OTA timing alignment) is needed as a pre-requisite for Feature 31-4. Therefore, it is proposed to add Feature 20-7 as a pre-requisite for Feature 31-4.

Proposal 2: Feature 20-7 is added as a pre-requisite for Feature 31-4.

- Conclusions

This contribution discusses on MT features for Rel-17 IAB and proposes the followings depending on the discussion:

Proposal 1: Feature 20-7 is added as a pre-requisite for Feature 31-1.

Proposal 2: Feature 20-7 is added as a pre-requisite for Feature 31-4.

References

R1-2112902, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e,” Moderators (AT&T, NTT DOCOMO, INC.).

Appendix: NR_IAB_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support Rel-17 DesiredGuardSymbols reporting 2) Support Rel-17 ProvidedGuardSymbols reception | one or more of {31-4, 31-5} | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-2 | IAB-DU beam restriction indication | Support restricted IAB-DU Beam Indication reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-3 | IAB-MT beam recommendation indication | Support recommended IAB-MT Beam Indication transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended IAB-MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-4 | Case 6 timing alignment | Support Case 6 timing alignment indication reception |  | Yes | N/A | Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. When to perform Case 6 timing at the IAB-node cannot be controlled by the parent node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-5 | Case 7 timing alignment | 1.) Support Case7 timing offset indication reception 2.) Support Case 7 timing at parent-node indication reception |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | 1.) Support Desired DL TX Power Adjustment reporting 2.) Support DL TX Power Adjustment reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting and reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-7 | [Desired] UL TX power adjustment | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | [31-8] | [Dynamic indication of Rel-17 or FDM soft resource availability] | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for Soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-9 | Simultaneous transmission and reception from multiple parent nodes | Support simultaneous transmission and reception from multiple parent nodes |  | Yes | N/A | Simultaneous transmission and reception is not supported in DC scenario | per BC | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |