3GPP TSG RAN WG1 #108-e                                                     R1-2201546

E-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.4.1

Source:	Spreadtrum Communications

Title:	Discussion on timing relationship enhancements for NTN

Document for:	Discussion and decision

Introduction

In this contribution, the remaining issues of timing relationship enhancements for NR NTN are discussed.

K_offset in initial access

In RAN1#107e, it was agreed that MAC CE provides a differential UE specific K_offset value and the full UE specific K_offset value equals the cell specific K_offset value minus the differential UE specific K_offset value [1].

Agreement For determining UE specific K_offset - Option 2: MAC CE provides a differential UE specific K_offset value. The full UE specific K_offset value equals the cell specific K_offset value minus the differential UE specific K_offset value. - FFS: whether/how to resolve ambiguity of which cell-specific K_offset value to use during the SIB modification period

In last meeting, some companies think that if the differential part is chosen, the ambiguity period of the cell-specific K_offset will mount on top of the signalling procedure for the MAC-CE determination. We understand the concern that there is nothing in specifications that forces a UE to read an updated SIB at a specific time when using the system information modification procedure. In our view, during the SIB modification period, both gNB and UE will assume the new cell-specific K-offset is not applied yet and use the old cell-specific K-offset. As cell-specifi K-offset is transmitted in SIB information, it can be treated similar as other SIB contents. Therefore, there is no ambiguity problem.

Proposal 1: There is no ambiguity of which cell-specific K_offset value to use during the SIB modification period.

MAC CE timing relationships

In RAN1#106_e, the following agreement on MAC CE timing relationships has been achieved [2].

Agreement: The information of K_mac is carried in system information.

The K_mac represents the RTT between the RP and gNB. The maximum value of unalignment is equal to the feeder link RTT, where the reference point is configured at the satellite. Since the TA values over service link and feeder link will change with the movement of the satellite, an update mechanism for the value of K_mac need to be supported. In our view, K_mac is cell-specific and can be update by the system information. There is no need to introduce additional mechanism for updating K_mac.

Proposal 2: K_mac can be update based on SIB update mechanism.

Conclusion

As summary, we have the following proposals.

Proposal 1: There is no ambiguity of which cell-specific K_offset value to use during the SIB modification period.

Proposal 2: K_mac can be update based on SIB update mechanism.

Reference

- 3GPP RAN1#107-e Chairman’s Notes, November 11th – 19th, 2021.

- 3GPP RAN1#106_e Chairman’s Notes, August 16th – 27th, 2021.