3GPP TSG RAN WG1 #108-e		 	R1-2201557

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.11.1.1

Source:		Spreadtrum Communications

Title:		Discussion on sidelink resource allocation for power saving

Document for:		Discussion and decision

Introduction

According to NR sidelink enhancement WID [1], for vulnerable road users (VRUs) in V2X use cases and UEs in public safety and commercial use cases, solutions for power saving in Rel-17 are required to minimize  power consumption. Random resource selection and partial sensing are the potential solutions for power saving in RAN1. In the following, we provide our considerations on the two mechanisms, respectively.

Discussion

Random resource selection

In RAN1 #103-e meeting, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof. And, for a resource pool with mixed type of resource allocation, the following agreement was made in RAN1 #106-e meeting.

Agreement For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e - Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed - Note, lower value means higher priority - FFS whether resource pool partitioning can be additionally applied - Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement, - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement. - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking - Option 12: No special consideration

When random resource selection coexists with full sensing or partial sensing in a same resource pool and pre-emption is enabled for full sensing or partial sensing UE, the pre-emption mechanism will not work well. If the interference UE is full sensing or partial sensing UE, it can do resource re-selection to avoid collision after its pre-emption checking. But, if the interference UE is random selection UE, since it has no SL reception capability to perform pre-emption checking, the collision cannot be avoided. So, low priority randomly selected transmission may collide with high priority transmission of full/partial sensing UE.

To solve this issue, several methods are provided in the previous meeting. Among them, that SCI indicates random selection UE or new priority of random selection UE and a priority threshold value is (pre-)configured for the resource pool are the three simplest methods. However, for the former two methods, such indication information cannot be identified by the R16 UE when there are also R16 UEs in the same resource pool, and the conflicts between the power-saving UE and R16 UE cannot be avoided. Considering the compatibility, we think the method that a priority threshold value is (pre-)configured for the resource pool is more suitable.

Proposal 1: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, a priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed.

For UEs having sidelink Rx capability, using random selection may help to further reduce power reduction compared with partial sensing. In order to benefit from random selection and further reduce resource collision, additional information can be provided for UEs performing random selection, i.e., among multiple resource pools pre-configured to power saving UEs with random selection permitted, RSU can instruct such UEs at least one resource pool to be used via sidelink signaling, and the selection of resource pool can be based on RSU’s CBR measurements.

Proposal 2: Assistant information can be provided via sidelink signaling to the UEs performing random selection.

Re-evaluation/pre-emption checking for aperiodic transmission

In the last meeting, the following agreements have been made for re-evaluation/pre-emption checking and contiguous partial sensing.

Agreement When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n, - The candidate resource set (SA) is initialized to the remaining Y’ candidate slots that starts from slot tyiSL and ends at the last slot of the Y’ candidate slots. - tyiSL is the first candidate slot after slot n+T3. - UE may perform PBPS for periodic sensing occasions after the resource (re)selection when sl-MultiReserveResource is enabled for the mode 2 Tx resource pool - It is up to UE implementation - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - FFS: When the minimum M slots for CPS cannot be guaranteed, - All available sensing results not earlier than n–T0 for the resource pool indicated by higher layer are applied for re-evaluation and pre-emption checking procedures Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - For minimum size M of the CPS monitoring window [n+TA, n+TB]: - By default, M is 31 unless (pre-)configured with another value - The range of (pre-)configured M is from 0 (working assumption) to 30

For the value of M in contiguous partial sensing, it has been agreed that M is 31 unless (pre-)configured with another value, and the (pre-)configured M is from a lower bound to 30.

M is the minimum CPS window. So, UE can perform CPS larger than M. We think larger range can include more cases and provide more flexibility. The working assumption in the last meeting should be confirmed.

Proposal 3: The working assumption that the range of (pre-)configured M is from 0 to 30 should be confirmed.

In the RAN1#107-e meeting, for CPS in resource (re-) selection, we have agreed that when the minimum M slots for CPS cannot be guaranteed, support the UE ensures the Y’min criterion is fulfilled (Option A) or UE performs random resource selection (Option B).

For re-evaluation/pre-emption checking, Y’min criterion cannot be always fulfilled and it may be unfair to do random resource selection for already selected/reserved resources. Therefore, neither Option A nor Option B is applicable for re-evaluation/pre-emption checking. The purpose of re-evaluation/pre-emption checking is to improve transmission reliability. So, when the minimum M slots for CPS cannot be guaranteed, UE performs sensing on all available slots before tyiSL-(Tproc,0SL+Tproc,1SL).

Proposal 4: When the minimum M slots for CPS cannot be guaranteed, UE performs sensing on all available slots before tyiSL-(Tproc,0SL+Tproc,1SL).

Impact of sidelink DRX on partial sensing

The following agreement was made in RAN 1#107-e meeting:

Agreement When SL DRX active time of Rx-UE is provided by the higher layer for candidate resource selection (including resource (re)selection and re-evaluation/pre-emption checking), the following working assumption is confirmed with option 2 as agreement (with modification in RED) Working Assumption (RAN1#106bis-e) When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer: - Option 1: PHY layer selects and reports candidate resources only within the indicated active time of the RX UE - Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE - FFS: Details on when the number of subsets of candidate resource is less than the threshold - FFS: The subset of candidate resource outside of the active time should consider each inactive time period - FFS: UE selection of resource selection window to overlap with indicated RX UE active time - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE - Option 3: PHY layer selects and reports an additional candidate resource set of candidate resources within the indicated active time of the RX UE

We have agreed that a subset of the candidate resources reported by the physical layer needs to be in active time of the RX UE. According to the discussion of RAN2, the initial transmission resource must be within active time, and the retransmitted resource can be within inactive time. Therefore, in order to ensure the reliability of the initial transmission, we need to determine the threshold of the proportion of the candidate resources within active time to the number of all remaining candidate resources.

In step 7 of mode2 resource exclusion, the candidate resource set need to satisfy the restriction of X⋅Mtotal. But this condition is for all remaining candidate resources. There may be a case that no candidate resources is within the active time or the number of candidate resources within the active time is lower than the (pre-)configured threshold. Step 7 needs to be enhanced to avoid this problem. So when the remaining candidate resource set within selection window satisfies the restriction of X⋅Mtotal but the number of subsets of candidate resource is less than the threshold, then RSRP threshold is increased and the procedure continues with step 6 for the candidate resources within active time.

Proposal 5: The ratio threshold of the candidate resources within active time to the number of all remaining candidate resources should be determined.

Proposal 6: When the number of subsets of candidate resource is less than the threshold, then RSRP threshold is increased and the procedure continues with step 6 for the candidate resources within active time.

Conclusions

The following conclusions are proposed:

Proposal 1: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, a priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed.

Proposal 2: Assistant information can be provided via sidelink signaling to the UEs performing random selection.

Proposal 3: The working assumption that the range of (pre-)configured M is from 0 to 30 should be confirmed.

Proposal 4: When the minimum M slots for CPS cannot be guaranteed, UE performs sensing on all available slots before tyiSL-(Tproc,0SL+Tproc,1SL).

Proposal 5: The ratio threshold of the candidate resources within active time to the number of all remaining candidate resources should be determined.

Proposal 6: When the number of subsets of candidate resource is less than the threshold, then RSRP threshold is increased and the procedure continues with step 6 for the candidate resources within active time.

References

RAN #90e RP-202846.