3GPP TSG RAN WG1 #108-e			R1-2201558

e-Meeting, February 21st – March 3rd, 2022

Agenda Item: 	8.11.1.2

Source:		Spreadtrum Communications

Title:		Discussion on inter-UE coordination in sidelink resource allocation

Document for:		Discussion and decision

Introduction

In this contribution, we will discuss the feasible mechanism of inter-UE coordination in mode 2 for NR sidelink communication.

Discussion

How to determine inter-UE coordination information in scheme 1

In RAN1 #106b-e, the following working assumption about the determination of inter-UE coordination information were achieved:

Working Assumption For Condition 1-B-1 of Scheme 1, the following two options are supported - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s)

In RAN1 #106b-e, two options were proposed for Condition 1-B-1 of Scheme 1. Option 1 reuses the resource exclusion procedure of Mode 2 to determine the non-preferred resource(s). UE-A will exclude the resource(s) with high interference whose RSRP measurement is larger than a threshold. Obviously, this option will work well similar to the R16 resource allocation of Mode 2.

The intention of Option 2 is for UE-A to protect the transmission between itself and another UE (e.g., UE-C) from interference by UE-B. When the detected RSRP is smaller than the RSRP threshold, the transmission of UE-B may interfere with the communication between UE-A and UE-C, and this option suggests deeming the resources used by UE-C as a non-preferred resource. However, considering that UE-C does not always exist and we are not sure whether UE-A must consider the impact on UE-C, it’s better to leave it to UE implementation to decide whether to adopt option 2.

Proposal 1: Option 1 of Condition 1-B-1 should be supported for scheme 1,

- Whether to support option 2 is up to UE implementation.

The contents of new 2nd SCI format in Scheme 1

In RAN1#107b-e, the following agreement about the containers of explicit request and inter-UE coordination information are made:

Agreement For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - (working assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A - A single format SCI 2-C is used for inter-UE coordination information and request - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information - SCI 2-C is UE RX optional - It is up to UE implementation to additionally use 2nd SCI (for UE-B). - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A

According to the above agreement, we can find that the explicit request and inter-UE coordination information use the same 2nd SCI format, i.e., SCI 2-C. RAN1 should strive for unified design the contents of SCI 2-C for both request and information as much as possible to minimize workload.

Referring to the previous agreement, the parameter fields of explicit request and inter-UE coordination information are summarized as below:

- For explicit request only:

- Priority value to be used for PSCCH/PSSCH transmission

- Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

- Resource reservation interval

- Starting/ending time locations of resource selection window

- For inter-UE coordination information only:

- Time indicator of resource set (TRIV)

- Frequency indicator of resource set (FRIV)

- Reference slot

- For both explicit request and inter-UE coordination information:

- Message type indicator (the SCI 2-C used for explicit request or inter-UE coordination information)

- Resource set type

Besides, according to the FL summary in the last meeting, most companies support that SCI format 2-C includes at least all the SCI fields in a SCI format 2-A as specified in TS 38.212, i.e., HARQ process number, new data indicator, redundancy version, source ID, destination ID, HARQ feedback enabled/ disabled indicator, cast type indicator, and CSI request.

According to the above summary for the parameter fields, the sizes of each parameter field could be further determined.

Proposal 2: The contents and sizes of parameter fields in the new 2nd SCI format 2-C are summarized as below:

Table 1: the contents and sizes of SCI 2-C

| Field | SCI 2-C |  |
| --- | --- | --- |
|  | Number of bits | Comments |
| Source ID | 8 bits | Same as SCI 2-A |
| Destination ID | 16 bits |  |
| Cast type indicator | 2 bits |  |
| Message type indicator | 1 bit | 1 bit in SCI 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information. Besides, 0 could be used to indicate the request. |
| Resource set type | 1bit | 1 bit in SCI 2-C is used to indicate whether the preferred resource set or non-preferred resource set will be transmitted in inter-UE coordination information. Besides, 0 could be used to indicate preferred resource set; 1, otherwise. |
| Priority value to be used for PSCCH/PSSCH transmission | 3 bits | For indicating the priority |
| Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot | Up to 5 bits | Up to 27 sub-channels per resource pool configuration |
| Resource reservation interval | Up to 4 bits / x | If the same indicating method as “resource reservation period” is used, the maximum number of bits is 4; otherwise, it will be further discussed. |
| Starting/ending time locations of resource selection window | Up to 17 bits + y | 10 bits to indicate the current frame from 1024 frames; up to 80 slots per frame; |
| Time indicator of resource set | z | Need to be further determined |
| Frequency indicator of resource set |  |  |
| Reference slot |  |  |

Obviously, considering the limitation of the timeline for inter-UE coordination information transmission, HARQ will not be supported for inter-UE coordination information. Hence, the parameter fields related to HARQ are not needed, unless other data is multiplexed with inter-UE coordination information. However, it is rare for the sidelink transmission to convey both inter-UE coordination and other data, which means that the payload related to HARQ will be wasted if these parameter fields about HARQ are introduced. From the point of our view, the parameter fields related to HARQ should not be included in SCI 2-C. Based on the above, we suggest the explicit request/inter-UE coordination information can be multiplexed with other data only if the container of explicit request/inter-UE coordination is MAC CE.

Proposal 3: The multiplex of explicit request/inter-UE coordination information and other data is allowed only when the container is MAC CE.

Which cast type is considered for inter-UE coordination information in scheme 1

In the last meeting, the following working assumption about the cast type for inter-UE coordination information triggered by a condition is achieved:

Working Assumption For Scheme 1, - Following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported

According to the above working assumption, there are three remaining issues to be solved:

1) Whether broadcast should be supported for transmitting preferred resource set?

2) Under which conditions groupcast/broadcast can be supported?

3) Under which conditions unicast can be supported?

For the first issue, when UE-A transmits preferred resource set in groupcast/broadcast, multiple UE-Bs will receive it. Then, multiple UE-Bs may select the same resource in preferred resource set from UE-A. A serious resource collision may occur. Hence, groupcast/broadcast should not be supported for preferred resource set.

Proposal 4: Groupcast/broadcast is not supported for preferred resource set when inter-UE coordination information transmission is triggered by a condition.

For the second one, since the parameters for determining non-preferred resource set are pre-configured or determined by UE-A’s implementation when triggered by a condition, the selected resource set may be valuable to multiple UE-Bs . Hence, multiple UE-Bs can use it for resource selection. From our point of view, groupcast/broadcast is suitable for non-preferred resource set when inter-UE coordination information transmission is triggered by a condition.

For the last issue, we think at least one condition should be considered, i.e., the inter-UE coordination information is multiplexed with other data which is transmitted by unicast. In this case, unicast should be supported.

Proposal 5: When the inter-UE coordination information triggered by a condition is multiplexed with other data which is transmitted by unicast, unicast can be supported.

PSFCH resource determination in scheme 2

In the previous meeting, the following agreements were achieved about PSFCH resource determination in scheme 2.

Agreement For Scheme 2, - Index of a PSFCH resource for inter-UE coordination information transmission is determined in the same way according to Rel-16 TS 38.213 Section 16.3 with at least following modification - P_ID is L1-Source ID indicated by UE-B’s SCI - M_ID is 0 - FFS: How to set m_CS - FFS: How to set m_0 - FFS: Whether M_ID can be (pre)configured Conclusion - For Scheme 2, there is no consensus to support indication of the following - Condition type of a resource conflict - Time location of a resource conflict Agreement For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

It has been agreed that PSFCH format 0 is used to send coordination information transmission in scheme 2, and the details of the PSFCH design are leftover issues. Considering the remaining time of Rel-17, the design of PSFCH for HARQ feedback should be reused as much as possible. Therefore, m_0 for a resource conflict indication can be derived in the same way as specified for HARQ-ACK information.

Proposal 6: m_0 for a resource conflict indication should be derived in the same way as specified for HARQ-ACK information.

The behavior of the UE-B may be different for conflict in periodic transmission and current TB transmission. For example, for conflict in periodic transmission, the resource reselected by UE-B should be reserved periodically for another TB transmission. But, for conflict in current TB transmission, periodic resource reservation is not mandatory. Therefore, the conflict in periodic transmission should be indicated by UE-A. And, in our view, m_CS can be set different values to distinguish conflict in periodic transmission and current TB transmission. Similar to HARQ-ACK information of R16, the value of m_CS is 0 for conflict in periodic transmission, the value of m_CS is 6 for conflict in current TB transmission.

Proposal 7: The conflict in periodic transmission should be indicated by UE-A, and m_CS can be set different values to distinguish conflict in periodic transmission and current TB transmission.

Conclusions

Proposal 1: Option 1 of Condition 1-B-1 should be supported for scheme 1,

- Whether to support option 2 is up to UE implementation.

Proposal 2: The contents and sizes of parameter fields in the new 2nd SCI format 2-C are summarized as below:

Table 1: the contents and sizes of SCI 2-C

| Field | SCI 2-C |  |
| --- | --- | --- |
|  | Number of bits | Comments |
| Source ID | 8 bits | Same as SCI 2-A |
| Destination ID | 16 bits |  |
| Cast type indicator | 2 bits |  |
| Message type indicator | 1 bit | 1 bit in SCI 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information. Besides, 0 could be used to indicate the request. |
| Resource set type | 1bit | 1 bit in SCI 2-C is used to indicate whether the preferred resource set or non-preferred resource set will be transmitted in inter-UE coordination information. Besides, 0 could be used to indicate preferred resource set; 1, otherwise. |
| Priority value to be used for PSCCH/PSSCH transmission | 3 bits | For indicating the priority |
| Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot | Up to 5 bits | Up to 27 sub-channels per resource pool configuration |
| Resource reservation interval | Up to 4 bits / x | If the same indicating method as “resource reservation period” is used, the maximum number of bits is 4; otherwise, it will be further discussed. |
| Starting/ending time locations of resource selection window | Up to 17 bits + y | 10 bits to indicate the current frame from 1024 frames; up to 80 slots per frame; |
| Time indicator of resource set | z | Need to be further determined |
| Frequency indicator of resource set |  |  |
| Reference slot |  |  |

Proposal 3: The multiplex of explicit request/inter-UE coordination information and other data is allowed only when the container is MAC CE.

Proposal 4: Groupcast/broadcast is not supported for preferred resource set when inter-UE coordination information transmission is triggered by a condition.

Proposal 5: When the inter-UE coordination information triggered by a condition is multiplexed with other data which is transmitted by unicast, unicast can be supported.

Proposal 6: m_0 for a resource conflict indication should be derived in the same way as specified for HARQ-ACK information.

Proposal 7: The conflict in periodic transmission should be indicated by UE-A, and m_CS can be set different values to distinguish conflict in periodic transmission and current TB transmission.