3GPP TSG RAN WG1 #108-e	R1-2201585

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.11.1.2

Source: 	Sony

Title:	Discussion on inter-UE coordination for Mode 2 enhancements

Document for:	Discussion

Introduction

In RAN1#107b-e, we had discussed inter-UE coordination schemes based on FL summary [1] and some details procedure and made agreements [2].

In this contribution, we discuss our view on some details of the inter-UE coordination for mode 2 in order to realise enhanced reliability and reduced latency.

Discussion

Combination(s) of supported features

In scheme 1, it was agreed to support two different types of inter-UE coordination information signalling and two different mechanisms to trigger inter-UE coordination information transmission as follows:

- Types of inter-UE coordination information signaling

  - Option A: Set of resources preferred for UE-B’s transmission

  - Option B: Set of resources non-preferred for UE-B’s transmission

- Mechanisms to trigger inter-UE coordination information transmission

  - Option 1: Triggered by an explicit request

  - Option 2: Triggered by a condition other than explicit request reception

At RAN1#106-e, FL has asked which combinations above will be supported. But we don’t think such down-selection is necessary since all combinations are beneficial for the inter-UE coordination in scheme 1. In addition, UE-A should be able to choose a preferred resource set (Option A) or a non-preferred resource set (Option B) depending on amount of resource set to be reported to UE-B in order to minimise the inter-UE coordination for both Option 1 and 2. Therefore we propose that all combinations of features should be supported for scheme 1. Also we think enabling/disabling for four different combinations should be (pre)configured separately.

Proposal 1: All combinations of information types and triggering mechanisms should be supported for scheme 1.

Proposal 2: Each combination can be enabled or disabled by (pre)configuration.

In scheme 2, since a type of inter-UE coordination information is an expected/potential resource conflict, we think the scheme 2 is basically used when UE-A detects such kind of conflict. Therefore, at least condition-based triggering should be supported for scheme 2.

Proposal 3: At least condition-based triggering should be supported for scheme 2.

Conclusion

Proposal 1: All combinations of information types and triggering mechanisms should be supported for scheme 1.

Proposal 2: Each combination can be enabled or disabled by (pre)configuration.

Proposal 3: At least condition-based triggering should be supported for scheme 2.

Reference

- R1-2200749, Feature lead summary #8 for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements, Moderator (LG Electronics), RAN1#107b-e

- RAN1 chairman’s notes, RAN1#107b-e