3GPP TSG RAN WG1 #108 	R1-2201587

e-Meeting, Feb 21st – Mar 3nd, 2022

Agenda item:		8.14.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues of time and frequency synchronization for NB-IoT/eMTC over NTN

Document for:		Discussion and Decision

# Introduction

In RAN1 #107 meeting, most of the discussion on IoT over NTN in RAN1 has been completed with agreements. Some FFS are remaining for further discussion, which we will discuss and provide our views on these points. Additionally, we also provide our views and proposals for remaining issues, for completion of Rel17.

# Discussion

## GNSS measurement window in IDLE mode

In RAN2 #116bis-e the following was agreed:

UE need to have a valid GNSS fix before going to connected. RAN2 assumes that the UE may need to re-aquire the GNSS fix right before establishing the connection (regardless if previously valid or not), if needed to avoid interruption during the connection.

In order to perform PRACH transmission with time-frequency pre-compensation, the UE needs to obtain valid GNSS information as also observed by RAN2. According to the discussions, a hot start may require a few seconds while a warm start takes considerably longer.

For mobile terminated traffic, the network will first page the UE after which the UE will attempt to set up a connection via Random Access. If the network is not aware that the UE has to obtain the valid GNSS information, i.e. delaying the start of the random access procedure, the network may interpret the lack of UE response as a sign that the UE did not receive the paging. Therefore, the network may repeat the paging, potentially escalating it to a larger area and thus affecting more UEs. Figure 1 illustrates the paging scenario where the UE applies either hot or warm start GNSS before being able to respond with start of random access procedure. Similarly, if (G)WUS is utilized, the network may need to accommodate the time to obtain valid GNSS information either between sending the WUS and the paging message, or after the paging message.

Figure 1 Illustration of GNSS start delay in a paging scenario.

Observation 1: If the network is not aware that a UE requires time to obtain valid GNSS information the network may trigger additional paging before the UE has a chance to initiate the pre-compensated random access procedure.

Acquisition of GNSS position fix during paging procedure is up to UE implementation and network configuration of paging timers considering GNSS measurement duration (e.g. GNSS Time To First Fix with cold start of typically 10 seconds) impact in NTN scenario. These paging timers are not specified in 3GPP in legacy paging procedure (i.e. T3413 / T3415).

In RAN1 #106-bis-e there was a discussion as follows:

However, delaying the paging procedure based on duration of GNSS cold start is not optimal, because different UEs with different capabilities and channel conditions will have varying time to get GNSS position fix.

If the timer to repeat and/or escalate paging is set to 10 s it results in a very slow paging procedure. If the network e.g. follows a paging procedure where it attempts to page the UE in the coverage area of the last known serving cell first and then repeat the paging at least once in that cell (area) if the UE does not respond, the long timer would mean that the network would potentially be paging in different cells due to the satellite movement (recall cell availability time is about 7 s in case of a small earth moving cell). This may not be an issue for the UE, because it would anyway need to perform cell reselection, but it complicates the network’s book keeping. Likewise, if the UE has moved from the area of the last serving cell to a neighbor cell within the tracking area, the delay until the network escalates the paging to that larger area is significantly prolonged if a long timer, based on GNSS cold start time, is used.

Observation 2: Basing paging repetition/escalation on GNSS cold start time value significantly delays the paging procedure.

A simple solution to the issue is to require that the UE always obtains valid GNSS information prior to monitor for paging. However, such an approach is rather energy consuming since the UE is not likely to be paged in every paging occasion. For example, the paging probability may be in the order of 0.1-1 % for IoT devices and thus it is a waste of energy to validate GNSS in 99 % or more of the paging occasions.

Observation 3: If UE validates GNSS before every paging occasion it will waste energy due to low paging probability.

An alternative solution is that the UE, at a prior occasion, provides its GNSS capability including an estimate of how long time is needed to obtain valid GNSS information (i.e.. GNSS measurement gap). Having such knowledge on network side enables the network to postpone further paging after sending the initial paging until after the GNSS measurement gap related to the reported UE capability has expired. The network may also consider the last time the UE was active to distinguish between whether the UE is expected to perform a warm or hot start. Alternatively, it can be specified that the UE shall always have either up-to-date GNSS information or be ready to perform hot start after being paged, i.e. excluding the longer cold and warm start options.

Proposal 1: UE shall report GNSS measurement capability such that network can allocate sufficient time between sending a paging message and when to expect random access procedure initialization from UE.

Proposal 2: Network shall not repeat the paging message for a UE during the UE’s GNSS measurement gap.

The position and duration of the GNSS measurement gap within the paging procedure can be further discussed and should be supported in Rel 17.

Proposal 3: A GNSS measurement gap, corresponding to the time the UE requires to validate GNSS, shall be configured in the paging procedure. The position and duration of the gap can be decided and supported in Rel 17.

## PUSCH/PRACH transmission

In RAN1 #107-e meeting, there were good progress for UL segment related agreements but still there are FFS as

For NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC:

Agreement

UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

- UE may drop / Insert samples / Puncture OFDM symbols

- UE may blank subframes / slots where UE skip a slot or a subframe

The total transmission time is not changed

UE autonomously Drop / insert samples / Puncture OFDM symbols or Blank subframes / slots where UE drops a subframe / slot

The method used for the UE pre-compensation is known to the eNB by a single UE capability

- UE Blank subframes / slots where UE skip a slot or a subframe (slot is based on Sub Carrier Spacing)

FFS Details of method(s) to drop / insert samples, blanking subframes / slots (slot is based on Sub Carrier Spacing)

For NPRACH for NB-IoT and PRACH for eMTC:

Agreement

For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

- UE may drop / Insert samples

- UE may blank subframe / repetition unit where UE drops a subframe / repetition unit

The total transmission time is not changed

FFS Details of method(s) to drop / insert samples / blank subframe / repetition unit

FFS Specification impact

Agreement

For eMTC, UE pre-compensation per segment of PRACH is applied from one segment to the next segment by drop / insert samples in Guard Period of PRACH preamble.

- The total transmission time is not changed

- FFS Details of method(s) to drop / insert samples

Agreement

At least UL transmission segment duration with one value X for NPRACH for NB-IoT and PRACH for eMTC may be indicated on SIB

- For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPRACH/PRACH

- FFS One value X, one or more values Xi

### Detail method for dropping

In WI phase, there were discussion and agreements on dropping/inserting will be processed between two adjacent segments considering the TA changing between them. Then one important point is the time length for the dropping/insertion.

As we discussed in [1], considering the distance changing between UE and satellite, betetween satellite and gateway, the largest TA change is smaller than 25 μs, as shown in Figure 2 for the minimum elevation angle 10 degree in period of 256ms.

Figure 2. TA change during the transmission period of 256 ms.

The processing of dropping will impact the performance for the dropped part. While the more to drop, the more performance loss. The more ratio of dropped part over the entire segment, the more performance loss, especially when segment sise is small, e.g. 2/4/8ms segment size. When dropping the entire subframe, for 2/4/8ms segment size cases, it means 1/2, 1/4, 1/8 of all repetitions will be dropped. This results in unnecessary performance loss as there is no need to drop the entire subframe but dropping some samples or one symbol will cover the TA changing i.e. 25us as maximum. Actually what needs to be dropped depends on the length of the TA difference between the two adjacent segments. Low TA changing rate can be seen in large elevation angle cases, while for small elevation angle cases the actual TA changing per segment will be much smaller than 25us e.g. 25 us / (256 ms /16 -1) = 1.66us for 16ms segment size case, which is much smaller than CP of the symbol, especially considering the large CP for the first symbol of the subframe. Thus, all cases can be covered by dropping one symbol or part of CP of the symbol, no need to drop more for unnecessary performance loss. Additionally, considering the longer CP of first symbol, it will be good to dropping starting from first symbol of the segment instead of the last symbol of the previous segment.

Observation 4: The more dropping, the more performance loss for the dropped symbol/subframe.

Observation 5: Dropping of entire subframe in each segment will cause large performance reduction, especially for the case when segment size is small, e.g. segment size as 2/4/8 subframe.

Observation 6: considering the longer CP of first symbol, it will be good to dropping starting from first symbol of the segment.

Proposal 4: For PUSCH, when dropping is not exceeding the CP, samples dropping of partial CP can be considered, otherwise one-symbol to be dropped should be utilized.

When droppng is done, it can be expected the error rate for the dropped symbol will be lower than the symbol not dropped, which will also cause the increasing of packet error rate for the entire PUSCH. How to reduce the loss because of dropping should be considered, e.g. changing the symbol to be dropped in the segments.

Observation 7: If always dropping the same symbol, then the receiving error rate of that symbol will be larger than other symbols, finally impact the BLER of the packet.

Proposal 5: How to mitigate the performance loss because of dropping in segments should be considered.

Similarly, for PRACH, as the dropping requested for each segment is very small as mentioned above, also considering the long CP of PRACH, the dropping should be to drop part of CP if overlapping part not exceeding the CP, or to drop CP + one sequence can be considered.

Proposal 6: For PRACH, when dropping is not exceeding the CP, samples dropping of parital CP can be considered, otherwise dropping of CP + one sequence should be utilized.

### TA error in segment

RAN1 has agreed TA processing before each segment and the TA will be not changed in the segment. This will provide TA update for each segment along with the satellite movement.. However, as the TA is not changed in the segment, the TA used can be aligned with the TA requested for one subframe of the segment but there will be still TA error for the other subframe of the segment. It is not discussed which repetition the TA for the segment is adjusted for. A general way is like UE processing in TN, TA error for the first subframe will be 0us, then the TA error for the following subframe will increase along with time. For the last subframe the TA error may be as ∆TA * (m-1), where ∆TA is the TA drifting in time of one subframe.

The TA error will decide the performance of the repetitions. The less the TA error, the better performance. How to provide reduce the TA error for repetitions should be considered and discussed.

Proposal 7: How to reduce the TA error for repetitions in the segment for good performance should be considered and discussed.

### Supporting of X value for segment size of PRACH

In the previous RAN1 meeting there was an FFS on whether more than one segment duration can be provided in SIB, i.e. X>1.

The benefit of configuring more than one segment duration is that the UE can select a value, which suits the current and future elevation angle of that UE. This would effectively optimize the number of segments, such that the UE can limit the processing it needs to perform at each segment switch to evaluate whether to insert/drop samples. If only one short segment duration is configured the UEs with high elevation angles would perform such evaluation too frequently.

Observation 8: A UE with a high elevation angle will benefit from using a longer segment duration than a minimum value applicable to all angles.

The eNB receiver will need to know which segment duration the UE is applying such that the receiver knows when a modification of one or more samples has occurred. However, for transmission on PRACH the network is not aware that the UE is about to transmit the preamble and thus the network cannot perform a dedicated UE configuration (at least not in initial access).

Observation 9: The eNB receiver needs to know which segment duration a UE is applying in order to determine when samples will be modified.

Observation 10: For initial access using Random Access the eNB receiver cannot perform UE-specific segment duration configuration.

Thus, if the network configures more than one segment duration, the UE needs to inform the eNB receiver, which segment duration it is using. However, since the segment duration is applied already in the preamble transmission the UE cannot provide the information as part of later signaling such as message 3 of the Random Access procedure.

If RAN1 agrees the network is allowed to configure more than one segment duration there is need for a solution by which the UE can indicate the selection of a segment duration from a set of durations as part of the preamble transmission.

Observation 11: UE needs to notify which segment duration it is applying in the preamble transmission if network has configured more than one segment duration.

The indication must be detectable by the network before it has received all repetitions of the preamble, because the UE will have applied the segment duration and thus sample modification during the repetitions.

Proposal 8: RAN1 to discuss configuration of more than one segment duration X for PRACH and how the UE can indicate the selected segment duration.

If RAN1 does not agree to configure more than one segment duration for formats 0,1 and format 2, the UE does not need to perform the indication of the selected segment duration, but it will suffer additional complexity of sometimes too frequently evaluating whether to modify samples.

A simple solution would be to define one segment duration for format 0,1 and one segment duration for format 2.

Observation 12: A simple solution is to define one segment duration for the Random Access preamble formats, but it may cause additional UE complexity for processing too many short segments.

### Configuration of segment for long uplink transmission

We have discussed the TA error compensation by configuring a transmission segment within a transmission block of 256 ms. Next, it is necessary to consider the case where multiple blocks of 256 ms are used for uplink. During the transmission period of the multiple blocks, TA drift rate will dramatically change due to the wide range of elevation angle of the satellite as the satellite moves along the orbit. Therefore, it can be needed to update the segment configuration every block of 256 ms. That will however increase signaling overhead with additional delay and some risk of link failure during a long UL transmission. Also, to avoid such segment configuration update, a single segment can be set to the shortest segment duration that is commonly applicable to UE in any elevation angle. However, it will also result in more frequent TA adjustments than necessary, thus adding to the UE processing complexity. Moreover, half-duplex UE transmitting in uplink cannot receive the downlink data. Therefore, half-duplex UE needs to complete all current uplink repetitions before changing uplink segment size.

Observation 13: Over a long uplink transmission the elevation angle change will cause large variation of TA drift rate.

Observation 14: Different segment sizes may be needed depending on the TA drift rate.

One approach to handle the issue of need for different segment sizes during a long uplink transmission is to configure multiple segment sizes, such that UE can switch from one segment size to the next during a single transmission.

Proposal 9: RAN1 to discuss how to configure multiple segment size for an uplink transmission.

## Validity timer for LEO/GEO

For validity timer, there were agreement in RAN1 #107-e meeting with FFS:

Agreement

Validity timer duration is configured per cell and indicated to the UE in X bits with:

- Value range {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240}

- Unit is second

- FFS Additional values for GEO

It seems possible to reuse the solution, which will be agreed in NR NTN.

Proposal 10: Regarding validity timer for GEO scenario reuse the NR NTN solution.

As half-duplex mode will be used for most of the NB-IoT and eMTC UE, when these UE are in the UL repetitions, UE can not read the NTN SIB at the same time. From this PoV, in order for UE to always read the NTN SIB to have updated ephemeris, there will be a limitation on UL scheduling, i.e. it is necessary to avoid UL transmission when UE need to update the NTN SIB. This will make the system efficiency lower. How to remove the limitation and keep effectiveness of the IoT NTN should be considered and discussed.

Observation 15: IoT UE with long UL repetition may miss the NTN SIB.

Proposal 11: How to remove limitation of UL scheduling for reading NTN SIB and keep effectiveness of IoT NTN system should be considered.

# Conclusion

In this contribution, we discussed time and frequency synchronization for NB-IoT/eMTC over NTN, our observations and proposals are presented as following:

Observation 1: If the network is not aware that a UE requires time to obtain valid GNSS information the network may trigger additional paging before the UE has a chance to initiate the pre-compensated random access procedure.

Observation 2: Basing paging repetition/escalation on GNSS cold start time value significantly delays the paging procedure.

Observation 3: If UE validates GNSS before every paging occasion it will waste energy due to low paging probability.

Observation 4: The more dropping, the more performance loss for the dropped symbol/subframe.

Observation 5: Dropping of entire subframe in each segment will cause large performance reduction, especially for the case when segment size is small, e.g. segment size as 2/4/8 subframe.

Observation 6: considering the longer CP of first symbol, it will be good to dropping starting from first symbol of the segment.

Observation 7: If always dropping the same symbol, then the receiving error rate of that symbol will be larger than other symbols, finally impact the BLER of the packet.

Observation 8: A UE with a high elevation angle will benefit from using a longer segment duration than a minimum value applicable to all angles.

Observation 9: The eNB receiver needs to know which segment duration a UE is applying in order to determine when samples will be modified.

Observation 10: For initial access using Random Access the eNB receiver cannot perform UE-specific segment duration configuration.

Observation 11: UE needs to notify which segment duration it is applying in the preamble transmission if network has configured more than one segment duration.

Observation 12: A simple solution is to define one segment duration for the Random Access preamble formats, but it may cause additional UE complexity for processing too many short segments.

Observation 13: Over a long uplink transmission the elevation angle change will cause large variation of TA drift rate.

Observation 14: Different segment sizes may be needed depending on the TA drift rate.

Observation 15: IoT UE with long UL repetition may miss the NTN SIB.

Proposal 1: UE shall report GNSS measurement capability such that network can allocate sufficient time between sending a paging message and when to expect random access procedure initialization from UE.

Proposal 2: Network shall not repeat the paging message for a UE during the UE’s GNSS measurement gap.

Proposal 3: A GNSS measurement gap, corresponding to the time the UE requires to validate GNSS, shall be configured in the paging procedure. The position and duration of the gap can be decided and supported in Rel 17.

Proposal 4: For PUSCH, when dropping is not exceeding the CP, samples dropping of partial CP can be considered, otherwise one-symbol to be dropped should be utilized.

Proposal 5: How to mitigate the performance loss because of dropping in segments should be considered.

Proposal 6: For PRACH, when dropping is not exceeding the CP, samples dropping of parital CP can be considered, otherwise dropping of CP + one sequence should be utilized.

Proposal 7: How to reduce the TA error for repetitions in the segment for good performance should be considered and discussed.

Proposal 8: RAN1 to discuss configuration of more than one segment duration X for PRACH and how the UE can indicate the selected segment duration.

Proposal 9: RAN1 to discuss how to configure multiple segment size for an uplink transmission.

Proposal 10: Regarding validity timer for GEO scenario reuse the NR NTN solution.

Proposal 11: How to remove limitation of UL scheduling for reading NTN SIB and keep effectiveness of IoT NTN system should be considered.

# References

- R1-2111276, “Enhancement to time and frequency synchronization for NB-IoT/eMTC over NTN”, 3GPP RAN1 #107-e, 2021