3GPP TSG RAN WG1 #108-e	R1-2201606

e-Meeting, February 21 – March 3, 2022

Agenda item:	8.16.6

Source:	NEC

Title: 	Discussion on UE feature for RedCap

Document for:	Discussion and Decision

Introduction

The following agreements were made in RAN1#107-e. RedCap specific UE feature is agreed as follows [1]:

Agreement

- FG 28-5 is removed

- Add a note in FG 1-5 (256QAM for PUSCH) that “For RedCap UEs, the 256QAM MCS table for PUSCH is only supported if the UE supports 256QAM for PUSCH”

Agreement

- Following features are not added into FG 28-1

  - Supported Rx branches and corresponding maximum DL MIMO layers

    - RedCap UE must indicate this capability from RAN1 perspective

  - Supported FDD operation

  - Supported maximum DL modulation order

Agreement

- Inform RAN2 that “From RAN1 perspective, it would be enough to indicate the maximum number of PDSCH MIMO layers per band for RedCap UEs, but RAN1 notes that the type of FG2-3 (maxNumberMIMO-LayersPDSCH) is currently per FSPC and that it is up to RAN2 whether to signal per band or per FSPC”

  - Note: If RAN2 decides to reuse the existing signaling (FG2-3) with modification for RedCap, then FG 28-2 is not needed from RAN1 perspective. If RAN2 decides to keep FG28-2, a RedCap UE must indicate FG28-2 from RAN1 perspective.

  - 1st bullet is captured in the LS to RAN2 being discussed in [107-e-R17-UE-features-REDCAP-02]

Agreement

- For early indication of RedCap UE,

  - The capability of early indication of RedCap UE in Msg.1 for 4-step RACH is added as a component in FG 28-1

  - FFS other early indication schemes

And in response LS on capability related RAN2 agreements for RedCap, RAN1 sent RAN1 agreements on RedCap UE capability to RAN2 [3].

In this contribution, we discuss remaining details on UE features for RedCap UE.

Discussion

On a UE feature on BWP operation, existing feature group FG 6-1 would be applicable for RedCap UE. Applicability of FG 6-1a would depend on discussion in agenda 8.6.1.1 in Rel-17. These FG are defined as follows [2]:

| 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |
| --- | --- | --- |
| 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell |

Regarding FG 6-1, basic BWP operation for RedCap UE supports a case where BW of a UE-specific RRC configured BWP does not include BW of CORESET#0 and SSB for PCell/PSCell (i.e. CD-SSB) but includes NCD-SSB and a common CORESET (if configured). To accommodate the case for RedCap UE, FG 6-1 would need updates.

Proposal:

- FG 6-1 is proposed to be updated as follows:

| 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell; for RedCap, BW of a UE-specific RRC configured BWP includes BW of CORESET#0 and SSB for PCell, or alternatively a common CORESET (if configured) and NCD-SSB |
| --- | --- | --- |
| 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell |

Summary

Proposal:

- FG 6-1 is proposed to be updated as follows:

| 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell; for RedCap, BW of a UE-specific RRC configured BWP includes BW of CORESET#0 and SSB for PCell, or alternatively a common CORESET (if configured) and NCD-SSB |
| --- | --- | --- |
| 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell |

Reference

[1] R1-2112137	Summary on UE features for REDCAP	Moderator (NTT DOCOMO, INC.)

[2] TR 38.822

[3] R1-2112754	Reply LS on capability related RAN2 agreements for RedCap	RAN1, Ericsson