3GPP TSG RAN WG1 #108-e		R1- 2201607

e-Meeting, February 21st – March 3rd, 2021

Agenda Item:	8.12.1

Source:	ASUSTeK

Title:	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs

Document for:		Discussion and Decision

- Introduction

The support of NR Multicast and Broadcast Services (MBS) is introduced in Rel-17 as a work item [1] to achieve group-scheduling. The objectives of NR MBS are specified in [1] and some agreements on the mechanisms to support group scheduling for RRC_CONNECTED UEs were reached at the RAN1#102-e through RAN1#106b-e meeting [2]~[8]. In this contribution, we discuss the issues about BWP switching handling, SPS multicast handling when switching BWPs, and possible solutions for supporting group-scheduling for the MBS WI and some proposals are given.

- Discussion

2-1. BWP switch handling for multicast

It was agreed that for multicast of RRC-CONNECTED UEs, a common frequency resource (CFR) for group-common PDCCHs and PDSCHs is confined within the frequency resource of a dedicated unicast BWP configured for a UE to support simultaneous receptions of unicast and multicast PDCCHs/PDSCHs in the same slot. Also, it was agreed that the common frequency resource for multicast of RRC_CONNECTED UEs includes one PDCCH-config and one PDSCH-config separate from the PDCCH-config and PDSCH-config of the dedicated unicast BWP.

For simplicity, here we denote a dedicated unicast BWP in which a common frequency resource is configured/confined and in which a UE is capable of receiving multicast PDCCHs/PDSCHs as an “MBS-capable BWP”. On the other hand, we denote a dedicated unicast BWP which does not contain a common frequency resource or in which a UE is incapable of receiving multicast PDCCHs/PDSCHs as an “MBS-incapable BWP”.

A UE may be configured with an MBS-capable BWP and an MBS-incapable BWP simultaneously. If active BWP is the MBS-capable BWP, the UE is able to receive multicast PDCCHs/PDSCHs. However, if at a moment the active BWP is switched to the MBS-incapable BWP, the UE is not able to receive the multicast PDCCHs/PDSCHs. In this case, it needs some studies on how to resume multicast PDCCH/PDSCH receptions, e.g. the may automatically switches back to the MBS-capable BWP after a certain time duration for multicast PDCCH/PDSCH receptions.

Observation 1: A UE is not able to receive multicast PDCCHs/PDSCHs if the UE’s active BWP is switched to an MBS-incapable BWP.

Proposal 1: If a UE’s active BWP is switched from an MBS-capable BWP to an MBS-incapable BWP, it needs some studies for the UE to resume multicast PDCCH/PDSCH receptions, e.g. the UE automatically switches back to the MBS-capable BWP after a certain time duration.

2-2. CFR sharing between BWPs

Based on the current agreements, the CFR is configured within a UE’s dedicated BWP for multicast receptions. Consider the scenario that a UE has two dedicated BWPs (e.g. BWP 1 and BWP 2) where one of the two BWPs (e.g. BWP 1) is configured with a CFR. If the UE’s active BWP is BWP 1, the UE is able to receive multicast PDCCH/PDSCH in the CFR in BWP 1. However, if the frequency resource of BWP 2 also fully covers the CFR, the UE is unable to receive multicast PDCCH/PDSCH in the CFR in BWP 2, which limits the multicast capabilities/throughputs for the UE. The design of the configuration of a CFR can be further studied or some mechanisms can be specified to better improve the multicast scheduling capability.

Proposal 2: CFR sharing mechanisms should be further studied to improve the multicast scheduling capability.

2-3. Semi-persistent scheduling (SPS) for multicast - SPS handling when switching BWPs

In NR Rel-15/16, an SPS configuration is configured per BWP (as in BWP-DownlinkDedicated RRC IE for a BWP). When a UE’s active BWP is switched from BWP 1 to BWP 2, any activated SPS configuration on BWP 1 would be terminated (no matter whether BWP 2 comprises frequency resources of the activated SPS or not), which is specified in 38.321.

This situation can be improved for the multicast reception scenario. When BWP 1 and BWP 2 both fully covers a CFR and if the CFR can be shared between the two BWPs (based on some mechanisms), the SPS multicast PDSCH receptions which are activated on BWP 1 may not be interrupted if the UE’s active BWP is switched to BWP 2. In this case, no (re-)activation PDCCH is needed on BWP 2 to resume the SPS multicast PDSCH receptions.

Proposal 3: SPS multicast PDSCH receptions are not interrupted in a CFR when switching between two BWPs if the CFR can be shared between the two BWPs.

2-4. The support of PTM transmission scheme 2

PTM transmission scheme 2 uses a UE-specific PDCCH to schedule a group-common PDSCH. PTM scheme 2 may bring respective benefits to control channel and data channel like better PDCCH reliability due to UE-specific beam selection and reducing data channel congestion by using a group-common PDSCH, instead of many UE-specific PDSCHs, to a group of UE. Therefore, we think introducing PTM scheme 2 for initial transmissions and retransmissions for multicast may be beneficial. Also, the design or fields of a PTM scheme 2 UE-specific PDCCH may be similar to the existing DCI format 1_0, 1_1 with CRC-scrambled by a C-RNTI, which may not take as much effort as the PTM scheme 1 PDCCH design does.

Also, we found that using a UE-specific PDCCH to activate an SPS multicast PDSCH configuration is very similar to PTM transmission scheme 2, where both of them use a UE-specific PDCCH for scheduling group-common PDSCHs. Therefore, if using a UE-specific PDCCH to activate an SPS multicast PDSCH configuration is supported, PTM transmission scheme 2 should be supported as well.

Proposal 4: PTM transmission scheme 2 for initial transmissions and retransmissions is supported for multicast.

2-5. Simultaneous reception of multicast and unicast PDSCH

In the latest feature lead summary in RAN1#107bis-e, there was a discussion on collision of multicast PDSCH and unicast PDSCH and a discussion on finalizing FDM reception between unicast PDSCH and multicast PDSCH. During the offline discussion, some company proposed to consider collision between DG PDSCH and resultant SPS PDSCH, but from our view, gNB has responsibility to avoid collision of DG PDSCH and resultant PDSCH no matter the DG PDSCH is unicast or multicast, and then we think the remaining issue should focus on collision between multicast SPS PDSCH and unicast SPS PDSCH.

In Rel-16, there is a procedure for UE to determine a number of TDM survivor SPS PDSCH among a set of SPS PDSCHs in a slot by ascending order of SPS configuration index. When it comes to Rel-17 MBS, the set of SPS PDSCHs in the slot may include multicast and unicast SPS PDSCH, and it’s necessary to discuss on how to extend Rel-16 procedure if UE has capability for FDM reception of multicast and unicast PDSCH. During offline discussion in RAN1 #107bis-e, proposal from feature lead consider extending Rel-16 procedure for determining only one resultant unicast SPS PDSCH and only one resultant multicast SPS PDSCH in FDM manner. From our perspective, it may have latency issue for unicast service since there is no other unicast SPS PDSCHs for URLLC. Thus, our first preference is to support case 6 (i.e., case6: FDM between multiple TDMed unicast PDSCHs and one GC-PDSCH in a slot), but for progress, FDM between only one resultant unicast SPS PDSCH and only one resultant multicast SPS PDSCH could be our second preference.

Proposal 5: If UE has capability for FDM reception of multicast and unicast PDSCH, and if more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, down-selecting following one alternative:

- Alt1: Extending Rel-16 procedure for determining one resultant multicast SPS PDSCH and one or more resultant (TDM) unicast SPS PDSCH in a slot, which resultant multicast SPS PDSCH and one or more resultant unicast SPS PDSCH could be received simultaneously.

- Alt2: Extending Rel-16 procedure for determining only one resultant multicast SPS PDSCH and only one resultant unicast SPS PDSCH in a slot, which could be received simultaneously.

- Conclusion

According to the discussion above, some observations and proposals are given.

Observation 1: A UE is not able to receive multicast PDCCHs/PDSCHs if the UE’s active BWP is switched to an MBS-incapable BWP.

Proposal 1: If a UE’s active BWP is switched from an MBS-capable BWP to an MBS-incapable BWP, it needs some studies for the UE to resume multicast PDCCH/PDSCH receptions, e.g. the UE automatically switches back to the MBS-capable BWP after a certain time duration.

Proposal 2: CFR sharing mechanisms should be further studied to improve the multicast scheduling capability.

Proposal 3: SPS multicast PDSCH receptions are not interrupted in a CFR when switching between two BWPs if the CFR can be shared between the two BWPs.

Proposal 4: PTM transmission scheme 2 for initial transmissions and retransmissions is supported for multicast.

Proposal 5: If UE has capability for FDM reception of multicast and unicast PDSCH, and if more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, down-selecting following one alternative:

- Alt1: Extending Rel-16 procedure for determining one resultant multicast SPS PDSCH and one or more resultant (TDM) unicast SPS PDSCH in a slot, which resultant multicast SPS PDSCH and one or more resultant unicast SPS PDSCH could be received simultaneously.

- Alt2: Extending Rel-16 procedure for determining only one resultant multicast SPS PDSCH and only one resultant unicast SPS PDSCH in a slot, which could be received simultaneously.

Reference

- RP-201038, ”WID revision: NR Multicast and Broadcast Services”, Huawei, HiSilicon.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #102-e, e-Meeting, August 17th - 28th, 2020.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #103-e, e-Meeting, October 26th - November 13th, 2020.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #104-e, e-Meeting, January 25th - February 5th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #104b-e, e-Meeting, April 12th – 20th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #105-e, e-Meeting, May 10th – 27th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #106-e, e-Meeting, August 16th – 27th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #106bis-e, e-Meeting, October 11th – 19th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #107-e, e-Meeting, November 11th – 19th, 2021.