3GPP TSG-RAN WG1 Meeting #108-e	R1-2201619

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on LS related to CORESET#0 impact of CBW narrower than 40MHz of n79

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has sent RAN1 an LS on CORESET#0 impact of CBW narrower than 40MHz of n79 [1]. For reference, the overall description is repeated below:

In RAN#94-e, the updated channel bandwidth (CBW) support to the band n79 was approved for the new scenarios (e.g., RedCap) based on RAN4’s agreements. Since the band n79 newly introduces {10, 20, 30, 70, 90} MHz to existing channel bandwidths {40, 50, 60, 80, 100} MHz, the minumum channel bandwidth of n79 has been changed from 40 MHz to 10MHz. As updated in TS 38.101-1 [1], most of the issues has been resolved for the band n79 by adding a step size of the n79 sync raster for the narrower channel bandwidths from RAN4’s perspective.

However, after RAN#94-e, RAN4 also identified a non-backward compatible issue due to the minimum bandwidth change of n79. Also, this issue shall be fixed in Rel-17. In TS 38.213, upon detection of SS/PBCH block, UE determines CORESET#0 from MIB by looking up the table for controlResourceSetZero. The table is determined according to subcarrier spacing of SSB, subcarrier spacing of PDCCH, and minimum channel bandwidth of the frequency band where UE located. For example, a new UE supporting 10 MHz the minimum channel bandwidth for n79 will have a different table (table 13.4 in TS 38.213) for CORESET#0 configuration with a legacy UE still supporting 40 MHz (table 13.6 in TS 38.213). Then, how to determine the table in this scenario can be an issue that needs further clarifications. RAN4 has come up with four potential solutions as follows:

- Alt-1: Add narrower channel bandwidth to n79, and indicate different table to legacy UE and new UE for CORESET#0 configuration.

- Alt-1a: Add narrower channel bandwidth to n79, and indicate the same table 13.6 to legacy UE and new UE for CORESET#0 configuration.

  - Rationale: Legacy UE not supporting channel bandwidth lower than 40MHz will always look at table 13.6, this table shall then be the common one.

- Alt-1b: Add narrower channel bandwidth to n79, and indicate:

  - A UE synchronizing on a GSCN with step 4 shall look at the table 13.6.

    - Rationale: This enables new and legacy to connect in band n79 using the same CORESET#0 configuration.

  - A UE synchronizing on a GSCN with step 1 but not step 4 shall look at the table 13.4.

    - Rationale: Table 3.4 offers more flexibility on CORESET#0 configuration

- Alt-2: Add narrower channel bandwidth to new band nX instead of n79.

|  | Pros | Cons | Specification impact |
| --- | --- | --- | --- |
| Alternative 1 | - Limited specification effort (Rel-17) | - Only one CORESET#0 configuration can be used for n79 | - Add clarification on restriction of CORESET#0 configuration in RAN1 |
| Alternative 1a | - Limited specification (Rel-17) - More configuration flexibility comparing to Alt.1 | - Lose some configuration flexibility compared with 10MHz table. | - Add clarification on CORESET #0 configuration table determination in RAN1 |
| Alternative 1b | - Limited specification (Rel-17) - More configuration flexibility comparing to Alt.1a for the new Rel-17 UEs | - UE should check on which GSCN it has synchronized | - Add clarification on CORESET #0 configuration table determination in RAN1 |
| Alternative 2 | - No impact on RAN1 | - New band shall be introduced (Rel-18) - Potential increase in UE complexity to support both n79 and nX | - Previous RAN4 agreements on n79 is reverted - Add new band in RAN4 |

After the discussion with options above, some RAN4 companies have preference for Alternative 1a/1b based on the table above, and would kindly ask RAN1 to consider this input to solve this n79 non-backward compatibility issue for Rel-17.

In this contribution we discuss an answer to RAN2’s question.

# 2	Discussion

New channel bandwidth was added for band n79 based on market demand in Korea and the need of narrower channel bandwidths than 40 MHz for RedCap devices.

Narrower channel bandwidths than 40 MHz are needed for RedCap devices for band n79.

Besides the new channel BWs, new raster points for n79 have been added to support finer granularity of SSB allocation in Rel-17. The step for raster points is decreased from 16 to 1.

Raster update in Rel-17:

Table 1: Proposed SS raster entries for n79

| NR operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n79 | 30 kHz | Case C | 8480 – <16> – 8880 (1) |
|  |  |  | 8475 – <1> – 8884 (2) |
| NOTE 1: The SS raster entries apply for channel bandwidths larger than or equal to 40 MHz NOTE 2: The SS raster entries apply for channel bandwidths smaller than 40 MHz |  |  |  |

The new added channel BWs to n79 leads to non-backward compatible issue because the added channel BWs (10, 20, 30, 70, 90) include values which are smaller than the minimum supported channel supported in Rel-18, i.e. 40 MHz, thus the SSB Index interpretation for band n79 in Rel-17 uses a different look up table to find the CORESET#0 than Rel-15.

Rel-15 UE shall apply table 13-6 (minimum channel BW is equal to 40MHz in Rel-15) from TS 38.213 if it supports band n79.

Table 13-6: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 40MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 24 | 3 | 0 |
| 3 | 1 | 24 | 3 | 4 |
| 4 | 1 | 48 | 1 | 0 |
| 5 | 1 | 48 | 1 | 28 |
| 6 | 1 | 48 | 2 | 0 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 48 | 3 | 0 |
| 9 | 1 | 48 | 3 | 28 |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

While a Rel-17 UE shall apply table 13-4 (minimum channel BW is equal to 10MHz in Rel-17) from TS 38.213 if it supports band n79 for the corresponding SS/PBCH and PDCCH SCS.

Table 13-4: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 1 |
| 2 | 1 | 24 | 2 | 2 |
| 3 | 1 | 24 | 2 | 3 |
| 4 | 1 | 24 | 2 | 4 |
| 5 | 1 | 24 | 3 | 0 |
| 6 | 1 | 24 | 3 | 1 |
| 7 | 1 | 24 | 3 | 2 |
| 8 | 1 | 24 | 3 | 3 |
| 9 | 1 | 24 | 3 | 4 |
| 10 | 1 | 48 | 1 | 12 |
| 11 | 1 | 48 | 1 | 14 |
| 12 | 1 | 48 | 1 | 16 |
| 13 | 1 | 48 | 2 | 12 |
| 14 | 1 | 48 | 2 | 14 |
| 15 | 1 | 48 | 2 | 16 |

Comparing the alternatives listed in LS, with Alt-1 only index 0 from table 13-4 and table 13-6 can be used after Rel-17.

Observation 1    Alt-1 supports only one entry of CORESET#0 configuration.

If we adopt Alt-1 as final solution for band n79, it means network can never use other configurations other than index 0 after Rel-17. This alternative put restrictions on the usage for band n79 which is against the Rel-17 purpose to broader the usage of this band.

Exclude Alt-1 in RAN1 discussion.

On remaining alternatives, Alt-2 would require UE to try both tables after resolved SSB Index to detect the PDCCH. Alt-2 is friendly to network implementation, however as stated in the LS, it adds complexity to UE implementation.

Observation 2    Alt-2 adds complexity to UE implementation.

Between Alt-1a and Alt-1b, both have limited specification impact in RAN1. Alt-1a reuses table 13-6 that provides 9 valid entries to indicate CORESET#0 configuration, where 5 of the entries are used for CORESET configuration of 48 RBs, which means for 10 MHz bandwidth, only 4 of configurations can be used. With Alt-1b, table 13-4 provides 16 valid entries, where 10 out of these 15 entries can be used for 10 MHz bandwidth. More configuration flexibility comparing to Alt-1a can be provided by Alt-1b. Therefore, Alt-1b fulfills the new market demand of narrower bandwidths and provides broader usage for band n79. Alt-1b shall be supported for n79.

Observation 3    Between Alt-1b and Alt-1a, Alt-1b provides more flexible SSB to CORESET#0 configurations (16:10), especially for narrower bandwidth of 10 MHz (10:4).

Support Alt-1b for n79.

# Conclusion

In this contribution we discussed CORESET#0 impact of CBW narrower than 40MHz of n79.

In the previous sections we made the following observations:

Narrower channel bandwidths than 40 MHz are needed for RedCap devices for band n79.

Observation 1    Alt-1 supports only one entry of CORESET#0 configuration.

Observation 2    Alt-2 adds complexity to UE implementation.

Observation 3    Between Alt-1b and Alt-1a, Alt-1b provides more flexible SSB to CORESET#0 configurations (16:10), especially for narrower bandwidth of 10 MHz (10:4).

Based on the discussion in the previous sections we propose the following:

Proposal 1	Exclude Alt-1 in RAN1 discussion.

Proposal 2	Support Alt-1b for n79.

A reply LS can be sent to RAN2 with this conclusion.

# References

R1-2200907, “LS on CORESET#0 impact of CBW narrower than 40MHz of n79”, RAN2, Samsung