3GPP TSG RAN WG1 Meeting #108-e	R1-2201627

e-Meeting, February 21 – March 3, 2022

Agenda Item:	7.2.5

Source:	Huawei, HiSilicon

Title:	Remaining issues on UL prioritization cases related to SP-CSI

Document for:	Discussion and Decision

# Introduction

In the RAN1#101-e meeting, cancellation handling with configured high priority transmission was discussed in [1], but it only focused on the UL prioritization case between PUCCH and PUCCH/PUSCH. The UL prioritization cases between PUSCH with SP-CSI and other PUSCHs with different priorities were omitted. This paper shows our views on this issue.

# Discussion

According to current spec [2], the agreements from the RAN1#101-e meeting are captured as follows:

… If a UE would transmit the following channels, including repetitions if any, that would overlap in time - a first PUCCH of larger priority index with SR and a second PUCCH or PUSCH of smaller priority index, or - a configured grant PUSCH of larger priority index and a PUCCH of smaller priority index, or - a first PUCCH of larger priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s) and a second PUCCH of smaller priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or a second PUCCH of smaller priority index with SR and/or CSI, or a configured grant PUSCH with smaller priority index, or a PUSCH of smaller priority index with SP-CSI report(s) without a corresponding PDCCH, or - a PUSCH of larger priority index with SP-CSI reports(s) without a corresponding PDCCH and a PUCCH of smaller priority index with SR, or CSI, or HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or - a configured grant PUSCH of larger priority index and a configured PUSCH of smaller priority index on a same serving cell the UE is expected to cancel a repetition of the PUCCH/PUSCH transmissions of smaller priority index before the first symbol overlapping with the PUCCH/PUSCH transmission of larger priority index if the repetition of the PUCCH/PUSCH transmissions of smaller priority index overlaps in time with the PUCCH/PUSCH transmissions of larger priority index. … A UE does not expect to be scheduled to transmit a PUCCH or a PUSCH with smaller priority index that would overlap in time with a PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH. A UE does not expect to be scheduled to transmit a PUCCH of smaller priority index that would overlap in time with a PUSCH of larger priority index with SP-CSI report(s) without a corresponding PDCCH. …

For the DCI scheduling a high priority transmission, the collision handling is captured as follow:

… When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1 and clause 11.1.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where …

The different combinations of UL transmissions with different priorities can be summarized in the table below. And based on the above paragraphs from the current spec, a UE behavior is defined for most of the cases except those that we mark with “Unknown1” or “Unknown2”.

Table 1 – UE behavior for different combinations of HP/LP overlap

|  | HP PUCCH with DCI | HP PUCCH with SPS AN without DCI | HP PUCCH with SR | HP PUSCH with DCI | HP CG PUSCH | HP PUSCH with SP-CSI without DCI |
| --- | --- | --- | --- | --- | --- | --- |
| LP PUCCH with DCI | HP | Error case | HP | HP | HP | Error case |
| LP PUCCH with SPS AN without DCI | HP | HP | HP | HP | HP | HP |
| LP PUCCH with SR | HP | HP | HP | HP | HP | HP |
| LP PUCCH with CSI | HP | HP | HP | HP | HP | HP |
| LP PUSCH with DCI | HP | Error case | HP | Error case | DG | Unknown2 |
| LP CG PUSCH | HP | HP | HP | DG | HP | Unknown2 |
| LP PUSCH with SP-CSI without DCI | HP | HP | HP | Unknown1 | HP | Unknown2 |

For all these “Unknown1” or “Unknown2” cases, the UE behavior should be defined to avoid misunderstanding between the UE and the gNB. For simplicity, we suggest that the PUSCH with smaller priority index can be cancelled for all these cases.

Proposal 1: For the overlap between LP PUSCH and HP PUSCH for the following cases, the LP PUSCH is cancelled and the TPs from Appendix 1 are adopted in 38.213

- TP 1 from Appendix 1 for TS 38.213 to define the UE behavior for the overlap between

  - HP PUSCH with DCI and LP PUSCH with SP-CSI without DCI;

- TP 2 from Appendix 1 for TS 38.213 to define the UE behavior for the overlap between

  - HP PUSCH with SP-CSI without DCI and LP PUSCH with DCI;

  - HP PUSCH with SP-CSI without DCI and LP CG PUSCH;

  - HP PUSCH with SP-CSI without DCI and LP PUSCH with SP-CSI without DCI.

# Conclusion

According to the discussion, following proposal is provided:

Proposal 1: For the overlap between LP PUSCH and HP PUSCH for the following cases, the LP PUSCH is cancelled and the TPs from Appendix 1 are adopted in 38.213

- TP 1 from Appendix 1 for TS 38.213 to define the UE behavior for the overlap between

  - HP PUSCH with DCI and LP PUSCH with SP-CSI without DCI;

- TP 2 from Appendix 1 for TS 38.213 to define the UE behavior for the overlap between

  - HP PUSCH with SP-CSI without DCI and LP PUSCH with DCI;

  - HP PUSCH with SP-CSI without DCI and LP CG PUSCH;

  - HP PUSCH with SP-CSI without DCI and LP PUSCH with SP-CSI without DCI.

# References

- R1-2005074, Summary #1 of email discussion [101-e-NR-L1enh-URLLC-HARQ&Scheduling-02], 	Moderator (Qualcomm), RAN1 #101-e

- 3GPP TS 38.213: "NR; Physical layer procedures for control", V16.8.0

# Appendix 1 – Text Proposals for RAN1#108-e

TP 1 for TS 38.213

To define the UE behavior for the overlap between HP PUSCH with DCI and LP PUSCH with SP-CSI without DCI

9 UE procedure for reporting control information <unchanged text omitted> When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1 and clause 11.1.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a second PUSCH of smaller priority index with SP-CSI reports(s) without a corresponding PDCCH on a same serving cell, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where <unchanged text omitted>

TP 2 for TS 38.213

To define the UE behavior for overlap between LP PUSCH and HP PUSCH for

- HP PUSCH with SP-CSI without DCI and LP PUSCH with DCI

- HP PUSCH with SP-CSI without DCI and LP CG PUSCH

- HP PUSCH with SP-CSI without DCI and LP PUSCH with SP-CSI without DCI

9 UE procedure for reporting control information <unchanged text omitted> If a UE would transmit the following channels, including repetitions if any, that would overlap in time - a first PUCCH of larger priority index with SR and a second PUCCH or PUSCH of smaller priority index, or - a configured grant PUSCH of larger priority index and a PUCCH of smaller priority index, or - a first PUCCH of larger priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s) and a second PUCCH of smaller priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or a second PUCCH of smaller priority index with SR and/or CSI, or a configured grant PUSCH with smaller priority index, or a PUSCH of smaller priority index with SP-CSI report(s) without a corresponding PDCCH, or - a PUSCH of larger priority index with SP-CSI reports(s) without a corresponding PDCCH and a PUCCH of smaller priority index with SR, or CSI, or HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or a PUSCH of smaller priority index on a same serving cell, or - a configured grant PUSCH of larger priority index and a configured PUSCH of smaller priority index on a same serving cell - a PUSCH with a PUSCH of smaller priority index with SP-CSI reports(s) without a corresponding PDCCH and the UE is expected to cancel a repetition of the PUCCH/PUSCH transmissions of smaller priority index before the first symbol overlapping with the PUCCH/PUSCH transmission of larger priority index if the repetition of the PUCCH/PUSCH transmissions of smaller priority index overlaps in time with the PUCCH/PUSCH transmissions of larger priority index. <unchanged text omitted>

# Appendix 2 – Agreements from previous meetings

- Agreements made in #98b meeting

Agreements: For intra-UE collision handling at the PHY layer, in case a high-priority UL transmission overlaps with a low-priority UL transmission, drop the low-priority UL transmission under certain constraint (particularly timeline). - The UL transmission is a positive SR, HARQ-ACK, PUSCH or P/SP-CSI on PUCCH. - FFS: for other types of UL transmission, e.g. SRS, PRACH, PUCCH-BFR, etc. - FFS details of dropping behaviours. - FFS details of processing timeline issues, e.g. - How to handle the case where the timeline condition is not satisfied. - Necessity of a new timeline.

- Agreements made in #101-e meeting

Agreement At least for handling collision between a high priority configured UL transmission and low priority channels in the following cases, it is up to UE implementation to ensure that the low priority UL transmission is cancelled, at the latest, from the first symbol that is overlapping with the high priority UL transmission: - Case 1: Collision between a high priority SR PUCCH and any low priority channels - Case 2: Collision between a high priority CG-PUSCH and a low priority PUCCH - Case 3: Collision between a high priority PUCCH carrying only HARQ-ACK corresponding to PDSCH without corresponding PDCCH and any low priority configured uplink transmission. - Case 4: Collision between a high priority PUSCH carrying SP-CSI, except the first PUSCH after the activation DCI, and a low priority PUCCH Agreement - A UE is not expected to be dynamically scheduled with a PUCCH or PUSCH with a lower priority overlapping with a higher priority PUCCH carrying only HARQ-ACK for PDSCH without corresponding PDCCH. - A UE is not expected to be dynamically scheduled with a PUCCH with a lower priority overlapping with a higher priority PUSCH carrying SP-CSI excluding the first PUSCH activated by a DCI.