3GPP TSG RAN WG1 #108-e	R1- 2201637

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		8.5.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 		Maintenance of PHY Latency Reductions

Document for: 	Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized. At RAN#90 a new WI was approved on enhancements in Rel-17 to positioning [1]. This contribution discusses our views on the enhancements related to on-demand PRS. Our companion contributions discuss our others views [2-6]. The objective from the WID is to

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

# Discussion

## Measurement gap enhancements

The UE is now able to request a MG activation and deactivation using UL MAC CE. Similarly a DL MAC CE will be introduced to active the PRS processing window (PPW) when the UE is making PRS measurements outside of a MG. What is still unclear is if a UE is allowed to make PRS measurements both inside the MG and outside a MG within one measurement report. RAN1 should discuss if this type of measurement is possible.

Proposal 1: RAN1 to discuss if a UE should make measurements both inside the MG and outside a MG in the same measurement report.

If some UE are restricted from making PRS measurement both inside the MG and outside a MG then it is not necessary for the UE to have both the PPW and the MG active. It is wasted overhead if the UE has to request the MG, the gNB needs to activate it, and the gNB needs to deactivate the PPW (and vice versa). One solution is to use an implicit deactivation of the MG (or PPW) when the UE can only make PRS measurement either inside or outside the MG.

Proposal 2: Consider implicit deactivation of the MG (or PPW) when the UE can only make PRS measurement either inside or outside the MG.

## PRS reception outside of a measurement gap

At RAN1#106 the following working assumption was reached:

Working assumption:

Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP.

- Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities:

  - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window.

    - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected.

    - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected.

      - FFS: band or CC

  - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window

  - A UE shall be able to declare a PRS processing capability outside MG.

    - FFS: Details of capability signalling (e.g., per UE or per band, etc.)

- For the purpose of this feature, PRS-related conditions are expected to be specified, with the following to be down-selected:

  - Alt. 1: Applicable to serving cell PRS only

  - Alt. 2: Applicable to all PRS under conditions to PRS of non-serving cell.

- Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options.

- Further study

  - Further details of which other DL signals/channels to be prioritized

  - How the UE determines DL PRS’s priority based on one or more of the following:

    - Opt. 1: Based on indication/configuration from serving gNB

    - Opt. 2: Other options (e.g., implicit, signalling from LMF, etc)

  - Whether UE can do the measurement for both inside MG (if MG is configured) and outside MG in a measurement period

  - How to do the PRS measurement when the conditions cannot be satisfied, e.g. when BWP switching happens

  - Prioritization conditions of processing PRS over other DL channels/signals or vice versa.

- Send an LS to RAN2, RAN3 and RAN4 informing them of this working assumption and requesting feedback in case they have concerns.

Now that reception of PRS outside a MG has been agreed there should be some RAN1 discussion on PRS priority and UE behaviour without a MG leads to PRS dropping. In particular if the UE is attempting to receive the PRS outside of a MG but is dropping the PRS multiple times (due to other DL signals and channels) then there should be some fallback mechanism in place to ensure the latency does not explode. One way to ensure this is to define a way for the UE to switch from MG-less to MG-based. For example, if a UE drops X PRS occasions in a row then it should request a MG or switch to MG-based mode.

Proposal 3: Specify a fallback method for the UE to switch from PRS measurement outside of MG to MG-based if the UE drops enough PRS.

At RAN1#107 the following agreements were reached for PRS reception outside a MG.

Agreement(RAN1#107-e)

For the purpose of determining conditions for measuring the PRS outside of a MG, the expected Rx timing difference between the PRS from the non-serving cell and that from the serving cell is determined by expected RSTD and expected RSTD uncertainty in the assistance data.

Send an LS to request RAN4 study and determine the threshold, which is used to be compared against with the Rx timing difference to determine whether the PRS from the non-serving cell satisfy the condition of PRS measurement outside MG.

  - Examples for the threshold: CP length, 50% of the OFDM symbol, 1ms

  - Other options can also be considered by RAN4

  - Note: the requirement on whether UE needs to calculate the expected Rx time difference and/or compare against the threshold is also a part of the study request

Further in TS 38.214, Clause 5.1.6.5 we have the following:

[The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter [PRSProcessingWindow].] The UE is not expected to measure the DL PRS outside the measurement gap if the received timing difference between PRS from the non-serving cell and that from the serving cell as determined [by assistance data] is larger than a threshold as determined by higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty.

So one issue left is the determination of the received timing difference between the PRS from the non-serving cell and that from the serving cell. If determined solely based on the assistance data then we feel that the PRS measurement outside of MG feature will not be very useful. The issue is that the granularity of expected-RSTD-uncertainty is pretty high (4*Ts for FR1 ~= 130 ns). Meaning that the value range is fairly large and not easy for the LMF to estimate/control when the UE will receive some PRS outside the MG. Especially when we consider the InF scenario where the UE may be close to many TRPs but the assistance data may give a wider range causing the UE not measure outside the MG for some TRPs.

Observation 1: The Expected RSTD granularity is fairly large and may cause limited numbers of TRPs to be within the threshold for measurement outside of MG.

In fact a UE may have a much better estimate of the Expected RSTD, and uncertainty, at a given time e.g., based on prior measurements or measurements on other signals. So we propose to take advantage of this fact by allowing the UE to use a local estimate of Expected RSTD to compare against the threshold for measurement outside of MG.

Proposal 4: Enable UE to use local estimate of ExpectedRSTD for comparing the received time difference with the threshold for measurement outside of MG.

There was additionally progress at RAN1#107-e on the topic of PRS priority within the PRS processing window with the following agreement reached:

Agreement(RAN1#107-e)

The following options are supported subject to UE capability for priority handling of PRS when PRS measurement is outside MG.

  - Option 1: UE may indicates support of two priority states.

    - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS

    - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS

  - Option 2: UE may indicate support of three priority states

    - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS

    - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS

      - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority.

    - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS

  - Option 3: UE may indicate support of single priority state

    - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS

Note: SSB is a separate issue.

UEs measuring PRS outside the MG that are capability 2 only prioritized PRS over other DL signals/channels only in the PRS symbols inside the window. For Option 2 of the PRS priority the UE supports 3 states and in state 2 and state 3 the PRS is lower priority than at least URLLC PDSCH. However, for capability 2 and state 2 of option 2 priority the UE behavior seems not fully possible. It is possible that the PDCCH is scheduled in the symbol immediately prior to the PRS transmission. In that case, the UE may not have sufficient time to decode PDCCH, in order to interprete any URLLC PDSCH in the follow-up symbols (especially when K0=0). Hence, the UE may not be fully ready to receive the PRS in the next symbol.

Observation 2: The UE may not be able to receive the PRS properly outside the MG for capability 2, state 2 of option 2 priority, when a PDCCH is in the symbol prior to the PRS.

To deal with this the UE may need some buffer between when the PDCCH and the PRS can be received. The UE could drop the PRS within the buffer as one option.

Proposal 5: Add a buffer between the PDCCH and PRS in some cases of UE measurement of PRS outside the MG (e.g., for capability 2, state 2 of option 2 priority).

The target PRS transmission by on-demand PRS request should consider the UE's preference on PRS processing outside of MG or MG-based mode. If the UE needs to keep receiving the data, but if it needs switching over to MG-mode to receive the target PRSs by UE/LMF-initiated on-demand request, the UE may try to repeatedly request another PRS transmission to receive it without MG configuration. Consequently, it may lead to unnecessary latency and resource inefficiency. Thus, the UE needs to send PRS processing outside of MG indicator to inform LMF that it can receive a PRS in outside of MG and another PRS in MG-based mode at least for the preconfigured PRS. This also helps to reduce the latency of the overall positioning session and makes the PRS processing outside of MG feature more useful.

Proposal 6: RAN1 to support PRS processing outside of MG indicator as an additional parameter for UE-initiated on-demand DL PRS request.

Another agreement relevant from RAN1#107 to measuring the PRS outside a MG is:

Agreement(RAN1#107-e)

At least the following parameters for PRS processing window from the gNB to the UE are supported.

  - Starting slot

  - Periodicity

  - Duration/length

  - Cell and SCS information associated with the above parameters

Discuss during the maintenance phase on the necessity of other parameters including but not limited to

  - Processing type (associated with the corresponding UE capability 1A/1B/2)

  - Band/CC-ID as needed depending on each scenario on which the PRS processing window is applied

  - The above cell and SCS information to determine where/when the PRS processing window is applied

Note: Indication of processing type does not suggest UE indication of multiple capabilities among (1A/1B/2) is already supported, which is a separate discussion.

Note: Some of the parameters above may not be mandatory for a PRS processing window

In our view the additional parameters for the PPW listed in the FFS are not essential.

Proposal 7: Do not support processing type, band/CC-ID, or additional cell and SCS information as parameters for the PPW.

# Conclusion

We made the following observations and proposals in this paper:

Proposal 1: RAN1 to discuss if a UE should make measurements both inside the MG and outside a MG in the same measurement report.

Proposal 2: Consider implicit deactivation of the MG (or PPW) when the UE can only make PRS measurement either inside or outside the MG.

Proposal 3: Specify a fallback method for the UE to switch from PRS measurement outside of MG to MG-based if the UE drops enough PRS.

Observation 1: The Expected RSTD granularity is fairly large and may cause limited numbers of TRPs to be within the threshold for measurement outside of MG.

Proposal 4: Enable UE to use local estimate of ExpectedRSTD for comparing the received time difference with the threshold for measurement outside of MG.

Observation 2: The UE may not be able to receive the PRS properly outside the MG for capability 2, state 2 of option 2 priority, when a PDCCH is in the symbol prior to the PRS.

Proposal 5: Add a buffer between the PDCCH and PRS in some cases of UE measurement of PRS outside the MG (e.g., for capability 2, state 2 of option 2 priority).

Proposal 6: RAN1 to support PRS processing outside of MG indicator as an additional parameter for UE-initiated on-demand DL PRS request.

Proposal 7: Do not support processing type, band/CC-ID, or additional cell and SCS information as parameters for the PPW.

# References

[1] RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson.

[2] R1-2201634, Maintenance of mitigating UE and gNB Rx/Tx timing errors, Nokia, Nokia Shanghai Bell.

[3] R1-2201635, Maintenance of enhancing UL AoA, Nokia, Nokia Shanghai Bell.

[4] R1-2201636, Maintenance of enhancing DL AoD, Nokia, Nokia Shanghai Bell.

[5] R1-2201638, Maintenance of LoS/NLoS Identification and Mitigation, Nokia, Nokia Shanghai Bell.

[6] R1-2201639, Maintenance of Inactive Mode Positioning and on-demand PRS, Nokia, Nokia Shanghai Bell.