3GPP TSG RAN WG1 #108-e	Tdoc R1-2201656

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	7.2.5

Source:	Ericsson

Title:	Clarification on HARQ-ACK for SPS PDSCH

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss the issue of HARQ-ACK for the first SPS PDSCH after activation vs HARQ-ACK for subsequent SPS PDSCHs.

# 2	HARQ-ACK for SPS

For DL SPS and HARQ-ACK, there are three aspects:

- For the activation DCI: there is no HARQ-ACK for activation DCI. The first HARQ-ACK associated with a SPS configuration corresponds to the first SPS PDSCH after activation.

- For SPS PDSCHs: each SPS PDSCH has a corresponding HARQ-ACK on the uplink.

  - In Rel-15, there is at most a single active SPS configuration. Also the minimum SPS periodicity is 10ms. Thus the HARQ-ACK transmission is not frequent.

  - In Rel-16, there can be up to 8 DL SPS configurations. Furthermore, the minimum SPS periodicity is lowered to 1 slot. Thus the HARQ-ACK transmission in Rel-16 can be tens or hundreds of times more frequent than that of Rel-15.

- For the deactivation (or release) DCI, there is a HARQ-ACK corresponding to the DCI, and there is no associated PDSCH transmission.

Recently, there has been debate how the PUCCH resource for SPS PDSCH is determined. Specifically, if the HARQ-ACK codebook construction and PUCCH resources are determined differently for the following two cases:

- the first SPS PDSCH after activation DCI;

- the subsequent SPS PDSCHs starting with the 2nd SPS PDSCH after activation DCI.

For instance, under AI 8.8.2, the moderator made the proposal in RAN1#106bis for Rel-17 coverage enhancement:

Updated FL proposed conclusion 0: In NR Rel-17, the dynamic PUCCH repetition factor indication mechanism agreed in RAN1 106e does not apply to HARQ-ACK for SPS PDSCH except for the following two cases

  - HARQ-ACK for the first SPS PDSCH associated with the activation DCI.

  - HARQ-ACK corresponding to the SPS release DCI

Note: HARQ-ACK for the first SPS PDSCH associated with the activation DCI and HARQ-ACK corresponding to the SPS release DCI are categorized as PUCCH with associated scheduling DCI

Also, the following agreement was made under AI 8.3.1.1 in RAN1#106 for Rel-17:

Agreement

In addition to HARQ-Ack of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier, the dynamic target carrier indication also applies to:

- HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI

- HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI

- triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI

- FFS: Additional cases

Thus there is a need to clarify the HARQ-ACK codebook construction and PUCCH resource determination for first SPS PDSCH vs subsequent SPS PDSCHs.

When constructing a HARQ-ACK codebook in a slot or sub-slot, there are two cases for SPS HARQ-ACK, depending on if there are HARQ-ACK bits for dynamic PDSCH that map to the slot:

- In a given (sub-)slot, there are only HARQ-ACK(s) for SPS PDSCH(s). No HARQ-ACK for dynamic PDSCH map to this (sub-)slot.

- In a given (sub-)slot, there are HARQ-ACK bits for dynamic PDSCH that map to this (sub-)slot, in addition to HARQ-ACK for SPS PDSCH(s).

Rel-16 agreements and specification texts (TS 38.213) are made for Case (A) and Case (B). This is demonstrated in the sub-sections below.

## 2.1	Case (A). When a (sub-)Slot Only Has HARQ-ACK for SPS PDSCH

For Case (A), the following agreement was made in RAN1#100e.  It clearly states that the agreement covers all SPS PDSCH, i.e., as long as there is no HARQ-ACK for dynamically scheduled PDSCH. There is no differentiation of first SPS PDSCH vs subsequent SPS PDSCH.

Agreements: (RAN1#100e) For HARQ-ACK of SPS PDSCH (without dynamic scheduled PDSCH), the PUCCH resource is determined based on SPS-PUCCH-AN-List once it is configured, regardless of the number of active SPS configurations.

Correspondingly, the specification text in 38.213 uses phrase “SPS PDSCH reception”. This phrase does not differentiate first SPS PDSCH vs subsequent SPS PDSCH.

For HARQ-ACK codebook construction and PUCCH resource determination of Case (A), there is no differentiation of first SPS PDSCH after activation DCI and subsequent SPS PDSCH.

TS 36.213v16.8.0 ### 9.2.1 PUCCH Resource Sets … If the UE is provided SPS-PUCCH-AN-List and transmits OUCI UCI information bits that include only HARQ-ACK information bits in response to one or more SPS PDSCH receptions and SR, if any, the UE determines a PUCCH resource to be - a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the first entry in sps-PUCCH-AN-List if OUCI≤2 including 1 or 2 HARQ-ACK information bits and a positive or negative SR on one SR transmission occasion if transmission of HARQ-ACK information and SR occurs simultaneously, or - a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the second entry in sps-PUCCH-AN-List, if provided, if 2<OUCI≤N1,SPS where N1,SPS is either provided by maxPayloadSize obtained from the second entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or - a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the third entry in sps-PUCCH-AN-List, if provided, if N1,SPS<OUCI≤N2,SPS where N2,SPS is either provided by maxPayloadSize obtained from the third entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or - a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the fourth entry in sps-PUCCH-AN-List, if provided, if N2,SPS<OUCI≤N3,SPS where N3,SPS is equal to 1706.

TS 36.213v16.8.0 ### 9.1.2 Type-1 HARQ-ACK codebook determination ... If a UE reports HARQ-ACK information in a PUCCH only for - a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1, or - a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, or - SPS PDSCH reception(s) within the occasions for candidate PDSCH receptions as determined in clause 9.1.2.1, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only for the PDSCH reception or only for one SPS PDSCH reception according to corresponding occasion(s) on respective serving cell(s), where the value of counter DAI in DCI format 1_0 is according to Table 9.1.3-1 and HARQ-ACK information bits in response to more than one SPS PDSCH receptions that the UE is configured to receive are ordered according to the following pseudo-code; otherwise, the procedures in clause 9.1.2.1 and clause 9.1.2.2 for a HARQ-ACK codebook determination apply. Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configuration configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH reception on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated where NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in sps-Config or, if pdsch-AggregationFactor-r16 is not included in sps-Config, by pdsch-AggregationFactor in pdsch-config, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; end while

## 2.2	Case (B). When a (sub-)Slot Has HARQ-ACK for SPS PDSCH and HARQ-ACK for Dynamic PDSCH

For Type-2 HARQ-ACK codebook, the HARQ-ACK(s) for SPS PDSCH(s) are generated and appended to the HARQ-ACK bits for dynamically scheduled PDSCH. See the quoted text in TS 38.213 section 9.1.3.1 below.

Thus, for Case (B), the specification also uses phrase “SPS PDSCH reception” (or “receive SPS PDSCH”). This phrase also does not differentiate first SPS PDSCH vs subsequent SPS PDSCH.

For HARQ-ACK codebook construction and PUCCH resource determination of Case (B), there is no differentiation of first SPS PDSCH after activation DCI and subsequent SPS PDSCH.

TS 36.213v16.8.0 #### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel … If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits. If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits. …

## 2.3	Summary

Based on the discussion above, it is clear that the agreements and specification do not differentiate (2a) and (2b) in HARQ-ACK construction and PUCCH resource determination. Thus we propose that RAN1 endorses the following conclusion to avoid future confusion.

Proposed Conclusion:

For HARQ-ACK codebook construction and PUCCH resource determination, there is no differentiation of first SPS PDSCH after activation DCI and subsequent SPS PDSCH, regardless of if there are HARQ-ACK bits for dynamically scheduled PDSCH in the same (sub-)slot.

# 3	Conclusion

Based on the discussion in the previous sections we have the following observations:

Observation 1	For HARQ-ACK codebook construction and PUCCH resource determination of Case (A), there is no differentiation of first SPS PDSCH after activation DCI and subsequent SPS PDSCH.

Observation 2	For HARQ-ACK codebook construction and PUCCH resource determination of Case (B), there is no differentiation of first SPS PDSCH after activation DCI and subsequent SPS PDSCH.

Correspondingly, we propose that RAN1 endorses the following conclusion for avoid future confusion.

Proposed Conclusion:

For HARQ-ACK codebook construction and PUCCH resource determination, there is no differentiation of first SPS PDSCH after activation DCI and subsequent SPS PDSCH, regardless of if there are HARQ-ACK bits for dynamically scheduled PDSCH in the same (sub-)slot.

# References

TS 38.213 V16.8.0.

# Appendix. Previous RAN1 Agreements

Agreements: (RAN1#100e)

For HARQ-ACK of SPS PDSCH (without dynamic scheduled PDSCH), the PUCCH resource is determined based on SPS-PUCCH-AN-List once it is configured, regardless of the number of active SPS configurations.