3GPP TSG RAN WG1 #108-e                                                        R1-2201698

e-Meeting, February 21st – March 3rd, 2022

Source: 	Intel Corporation

Title: 	Maintenance of Rel.17 NR Positioning Solutions for UL-AoA Method

Agenda item:	8.5.2

Document for: 	Discussion and Decision

# Introduction

The RAN WG1 agreed to support the UL SRS-RSRPP measurements for the first for the UL-AOA positioning method and additional paths in case of the hybrid positioning methods where the UL-TDOA and Multi-RTT are used in addition to the UL-AOA.

In this contribution we propose the information reporting formats for the first path UL SRS-RSRPP in case of the UL-AOA positioning method. The agreed RX diversity options are considered.

In addition to that, we propose to clarify the UL-AOA definition for the case of additional paths reporting.

Accuracy Improvements for UL-AOA Positioning Solutions

## UL SRS-RSRPP First Path Reporting

The definition of the UL SRS-RSRPP was discussed at the RAN WG1#107-e meeting and the following agreement has been captured in the chairman’s notes, [1]:

Agreement (RAN1#107-e): Apply the following changes to the definition for UL SRS-RSRPP in the previous agreement: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. - Working assumption: For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. Note: The following two options are supported by gNB to LMF: - Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches - Option 2 (RX diversity for UL SRS-RSRP) - The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided)

In addition to the UL SRS-RSRPP definition two options for UL SRS-RSRPP reporting with regards to the different types of RX diversity have been agreed.

With respect to the first path UL SRS-RSRPP reporting, we propose to consider two options.

Option 1:

In option 1, if the UL SRS-RSRPP of the first path is reported only, then the UL SRS-RSRPP of the first path is reported using the absolute values in the range (-156 dBm, -31 dBm).

Option 2:

In option 2, if both UL SRS-RSRP and UL SRS-RSRPP of the first path are reported, then the UL SRS-RSRP is reported using the absolute values in the range (-156 dBm, -31 dBm) and the UL SRS-RSRPP of the first path is reported using the differential values in the range (-30 dB, 0 dB), where the UL SRS-RSRP is selected as a reference measurement.

Based on the provided considerations, we have the following proposal:

  - For the UL-AOA positioning, the gNB can be requested to measure and report to the LMF the UL SRS-RSRPP of the first path using the following options:

    - Option 1: if the UL SRS-RSRPP of the first path is reported only, then the UL SRS-RSRPP of the first path is reported using the absolute values

    - Option 2: if both UL SRS-RSRP and UL SRS-RSRPP of the first path are reported, then the UL SRS-RSRP is reported using the absolute values and the UL SRS-RSRPP of the first path is reported using the differential values, where the UL SRS-RSRP is selected as a reference measurement

    - The absolute values are reported in the range [-156 dBm, -31 dBm] with 1 dB resolution

    - The differential values are reported in the range [-30 dB, 0 dB] with 1 dB resolution

## UL-AOA Measurement Definition

The UL-AOA measurements reporting for additional paths in case of the hybrid positioning, where UL-TDOA and Multi-RTT are used in addition to the UL-AOA have been discussed under the AI 8.5.5 and the following agreement has been captured in the chairman’s notes, [2]:

Agreement: For hybrid positioning methods where UL TDOA and multi-RTT are used in addition to UL AoA, support reporting of up to M=8 UL-AoA values per additional path

It should be noted that the current definition for the UL-AOA measurement in the TS 38.215 is not completely correct with respect to the additional path measurements, [1]:

UL Angle of Arrival (UL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, down tilt and slant angles of LCS are defined according to TS 38.901 [15]. The UL AoA is determined at the gNB antenna for an UL channel corresponding to this UE.

In case if the UL-AOA is measured for the additional path, then the UL-AOA does not define the azimuth and vertical angle of a UE. The UL-AOA angle corresponds to the reflected path with respect to a reference direction, which may not correspond to the direction of a UE.

In this contribution we propose to clarify the UL-AOA definition for the case of additional paths reporting.

  - Update the UL-AOA measurement definition in the TS 38.215, adding the following:

    - In case if the UL Angle of Arrival (UL-AOA) is measured for the additional path, it is defined as the estimated azimuth and vertical angle of a reflected path with respect to a reference direction, which may not correspond to the direction of a UE

    - Note: Multiple UL-AOA pairs can be reported per single channel path

## Use of R15 SRS for NR Positioning

RAN1 has received LS from RAN4 asking to clarify the potential usage of the R15 SRS for NR positioning:

R1-2200900 LS on SRS for multi-RTT positioning RAN4 discussed whether Rel-15 SRS is applicable for multi-RTT positioning, i.e., whether it can be used in UE Rx-Tx time difference and gNB Rx-Tx time difference measurements. The UE Rx-Tx time difference measurement requirements in clause 9.9.4, TS 38.133 and gNB Rx-Tx time difference accuracy requirements in clause 10.1.25, TS 38.133 are applicable provided that the UE is configured with SRS. RAN4 has observed that some legacy gNB may not support Rel-16 positioning SRS. Furthermore, same gNB may be configured to perform gNB Rx-Tx time difference measurement and UL RTOA for the same UE. RAN4 kindly requests RAN1 to confirm whether Rel-15 SRS is applicable for UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement, and if so, from which release.

In our understanding, the RAN4 question may be further divided into two questions:

Q1: Whether Rel-15 SRS is applicable for Multi-RTT positioning?

Q2: Whether Rel-15 SRS is applicable for UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement, and if so, from which release?

Regarding the first question (Q1), our understanding is that there is no explicit agreement that Rel-15 SRS are applicable for Multi-RTT positioning. The use of Rel-15 SRS was not agreed for NR multi-RTT positioning by RAN1, but Rel-15 SRS can be used for NR UL-TDOA and NR UL AOA positioning techniques.

Regarding the second question (Q2), our understanding is that RAN1 agreed to use SRS for RTT-based PDC (propagation delay compensation) in the context of URLLC discussions. This functionality is limited to serving cell measurements only:

Agreement (RAN1#106-e) SRS can be used for Rx – Tx time difference estimation at gNB side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. Conclusion (RAN1#107-e) SRS for positioning is not supported for RTT-based PDC, regardless of whether TRS or PRS is used for RTT-based PDC.

Based on above discussion, we have the following proposal:

Proposal #3:

  - Send reply to LS from RAN4 with the following content:

    - Rel-15 SRS is not supported for NR Multi-RTT positioning

    - Rel-15 SRS can be supported for NR UL-TDOA and UL-AOA positioning starting from Rel.16

    - Rel-15 SRS is supported for serving cell UE/gNB Rx-Tx time difference measurements in context of NR RTT-based PDC

      - This functionality is supported starting from the Rel.17

Conclusions

In this contribution, we have provided further details to support UL-AOA enhancements. In summary, we have the following list of proposals:

Proposal #1:

  - For the UL-AOA positioning, the gNB can be requested to measure and report to the LMF the UL SRS-RSRPP of the first path using the following options:

    - Option 1: if the UL SRS-RSRPP of the first path is reported only, then the UL SRS-RSRPP of the first path is reported using the absolute values

    - Option 2: if both UL SRS-RSRP and UL SRS-RSRPP of the first path are reported, then the UL SRS-RSRP is reported using the absolute values and the UL SRS-RSRPP of the first path is reported using the differential values, where the UL SRS-RSRP is selected as a reference measurement

    - The absolute values are reported in the range [-156 dBm, -31 dBm] with 1 dB resolution

    - The differential values are reported in the range [-30 dB, 0 dB] with 1 dB resolution

Proposal #2:

  - Update the UL-AOA measurement definition in the TS 38.215, adding the following:

    - In case if the UL Angle of Arrival (UL-AOA) is measured for the additional path, it is defined as the estimated azimuth and vertical angle of a reflected path with respect to a reference direction, which may not correspond to the direction of a UE

    - Note: Multiple UL-AOA pairs can be reported per single channel path

Proposal #3:

  - Send reply to LS from RAN4 with the following content:

    - Rel-15 SRS is not supported for NR Multi-RTT positioning

    - Rel-15 SRS can be supported for NR UL-TDOA and UL-AOA positioning starting from Rel.16

    - Rel-15 SRS is supported for serving cell UE/gNB Rx-Tx time difference measurements in context of NR RTT-based PDC

      - This functionality is supported starting from the Rel.17

References

| - [1] | - 3GPP TSG RAN WG1 #107e “RAN1 Chair’s Notes,” November 11 – 19, 2021 |
| --- | --- |
| - [2] | - 3GPP TSG RAN WG1 #106bis-e “RAN1 Chair’s Notes,” October 11 – 19, 2021 |
| - [3] | - TS 38.215 “Physical Layer Measurements,” December 2021. |