3GPP TSG RAN WG1 Meeting #108-e                                                                R1-2201703

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:		8.6.1.2

Source: 	Intel Corporation

Title:	On other aspects of complexity reduction for RedCap UEs

Document for: 	Discussion/Decision

# Introduction

In this contribution, we provide our further views on the remaining issues on HD-FDD operations of RedCap UE.

# Handling overlap between a DL reception and a UL transmission

A FDD cell consists of a DL carrier and a UL carrier. While the FDD gNB can do full duplex operation on both DL/UL carriers, a HD-FDD UE can only perform either DL reception on the DL carrier or UL transmission on the UL carrier at a time. Further, a gap is required between a DL reception and a UL transmission at UE, i.e., DL-to-UL switching time, and between a UL transmission and a DL reception, i.e., UL-to-DL switching time. Except that DL reception and UL transmission are performed on DL carrier and UL carrier respectively, HD-FDD UE operation is similar to the operation of a TDD UE.

The overlap handling scheme between a DL channel/signal and a UL channel/signal for NR TDD is summarized in Table 1. A deprioritized channel/signal may be fully cancelled or partly cancelled on the overlapped symbols.

Table 1.  Overlap handling between DL and UL channels/signals of NR TDD UE

| Case# | DL channel/signal | UL channel/signal | Rel-15/16 UE behavior |
| --- | --- | --- | --- |
| 1 | Scheduled DL, e.g. PDSCH or CSI-RS | Configured UL, e.g. SRS, or PUCCH, or PUSCH, or PRACH | Prioritize scheduled DL subjected to cancellation timeline checking PUCCH, PUSCH, PRACH: cancel all symbols (not capable of [partialCancellation]), or cancel overlapped symbols (capable of [partialCancellation]) SRS: cancel the overlapped symbols |
| 2 | Configured DL, e.g. PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS | Scheduled UL, e.g. PUSCH, a PUCCH, a PRACH, or a SRS | Prioritize scheduled UL Cancel all DL symbols |
| 3 | Configured DL | Configured UL | Error case |
| 4 | Scheduled DL | Scheduled UL | Error case |
| 5 | SSB | Scheduled/configured UL, e.g. PUSCH, PUCCH, PRACH, SRS | Prioritize SSB PUSCH, PUCCH, PRACH: Cancel all UL symbols SRS: cancel the overlapped symbols |
| 8 | Scheduled/configured DL | Valid RO | Prioritize PRACH Cancel all DL symbols |

Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

In NR TDD, for the overlap between SSB and scheduled/configured UL channel/signal, the SSB is prioritized. All symbols of PUSCH, PUCCH, PRACH are dropped. Regarding SRS, only the overlapped SRS symbols are dropped. In RAN1#106-e meeting, it was agreed that when a configured SSB overlaps with configured UL transmission, the SSB is prioritized. Then, when a configured SSB overlaps with dynamically scheduled UL transmission, it was further agreed in RAN1#107-e that SSB is prioritized at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4. It is preferred to define a unified solution for all UL channel/signals including Msg3 (re)transmission and PUCCH for Msg4. Since gNB has the full control on the timing of dynamically scheduled UL channel/signal, it is up to gNB implementation to avoid the overlap between SSB and Msg3 (re)transmission and PUCCH for Msg4.

Proposal 1: For Case 5, if a SSB overlaps with Msg3 (re)transmission and PUCCH for Msg4, the existing NR TDD behaviour is reused, i.e., SSB is prioritized.

- No TP is needed since the existing specification already reflect the proposed behaviour.

The operation related to Case 5 is currently captured by the following text in section 17.2 in TS38.213 [2].

If a HD-UE would transmit a PUSCH, or PUCCH, or SRS based on a configuration by higher layers and the HD-UE is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, the HD-UE does not transmit - PUSCH or PUCCH if a last symbol of the PUSCH or PUCCH transmission would not be at least NTx-Rx⋅Tc [4, TS 38.211] prior to a first symbol of the next earliest SS/PBCH block - PUSCH or PUCCH if a first symbol of the PUSCH or PUCCH transmission would not be at least NRx-Tx⋅Tc [4, TS 38.211] after a last symbol of the previous latest SS/PBCH block - SRS in symbols that would not be at least NTx-Rx⋅Tc prior to a first symbol of the next earliest SS/PBCH block - SRS in symbols that would not be at least NRx-Tx⋅Tc after a last symbol of the previous latest SS/PBCH block

We discuss the handling of PUSCH or PUCCH in the above specification. The same observation applies to SRS too. During the email discussion on the CR to TS 38.213, it was commented that the joint interpretation of the first condition (red) and the second condition (red) will result in dropping the PUSCH or PUCCH in undesired cases. Assuming the cited specification is not for the case that the PUSCH or PUCCH and the SSB are overlapped, Figure 1 shows the symbol positions when the first or second condition becomes true. That is,

- The first condition is true in both the desired symbol positions shown by the small red block and unwanted symbol positions shown by the large red block

- The second condition is true in both the desired symbol positions shown by the small yellow block and unwanted symbol positions shown by the large yellow block

In summary, since the logic between the first condition and the second condition is essentially ‘OR’, UE will always drop any PUSCH or PUCCH if the PUSCH or PUCCH is not overlapped with the SSB.

Figure 1: The two conditions handling switching time between PUSCH/PUCCH and SSB

The correct behaviour is only dropping the PUSCH or PUCCH in the period of the small red block and the small yellow block in Figure 1. A simple way to fix the problem is to add a logic ‘AND’ to the two conditions. On the other hand, it is not clear which part in section 11.1 in TS 38.213 defines the prioritization of SSB if the SSB overlaps with a PUSCH or PUCCH. It would be preferable to explicitly capture it in section 17.2 in TS 38.213. The following TP is proposed to capture the above changes.

------------------------------ TP#1: TS 38.213 ----------------------------------- ## 17.2 Half-Duplex UE in paired spectrum *** Unchanged text is omitted *** If a HD-UE would transmit a PUSCH, or PUCCH, or SRS based on a configuration by higher layers and the HD-UE is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, the HD-UE does not transmit - PUSCH or PUCCH if a last symbol of the PUSCH or PUCCH transmission would not be at least NTx-Rx⋅Tc [4, TS 38.211] prior to a first symbol of the next earliest SS/PBCH block and if a first symbol of the PUSCH or PUCCH transmission would not be at least NRx-Tx⋅Tc [4, TS 38.211] after a last symbol of the previous latest SS/PBCH block - SRS in symbols that would not be at least NTx-Rx⋅Tc prior to a first symbol of the next earliest SS/PBCH block and would not be at least NRx-Tx⋅Tc after a last symbol of the previous latest SS/PBCH block If a HD-UE would transmit a PUSCH, or PUCCH, or PRACH, or SRS and the HD-UE is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols. *** Unchanged text is omitted ***

Proposal 2: It is proposed to agree on TP 1 for overlap handling of Case 5.

# Available slots for PUCCH repetition

In NR Rel-15/16, UE determines all slots as available slots for physical uplink control channel (PUCCH) repetitions for FDD system. Further, for TDD system, UE determines a slot as available slot when a PUCCH repetition does not overlap with DL symbols which are indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For a RedCap UE in HD-FDD operation, for PUCCH repetitions, UE determines a slot as an available slot for PUCCH repetitions when a PUCCH repetition does not overlap with a symbol with synchronization signal block (SSB) transmission indicated by ssb-PositionsInBurst.

The following TP is proposed to capture available slot determination for PUCCH repetition in Section 17.2 in TS38.213 [1]

------------------------------ TP#2: TS 38.213 ----------------------------------- ## 9.2.6 PUCCH repetition procedure *** Unchanged text is omitted *** For paired spectrum or supplementary uplink band, the UE, except when it is a half-duplex UE, determines the NPUCCHrepeat slots for a PUCCH transmission as the NPUCCHrepeat consecutive slots starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting *** Unchanged text is omitted *** ## 17.2 Half-Duplex UE in paired spectrum *** Unchanged text is omitted *** A HD-UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting, where a repetition of the PUCCH transmission does not include a symbol indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. *** Unchanged text is omitted ***

Proposal 3: On PUCCH repetition

- HD-FDD RedCap UE determines a slot as an available slot for PUCCH repetitions when a PUCCH repetition does not overlap with a symbol with synchronization signal block (SSB) transmission indicated by ssb-PositionsInBurst

- Agree on TP 2 for available slot determination for PUCCH repetition.

# Conclusion

In this contribution, we provide our views on open issues for HD-FDD operation. we make the following proposals.

Proposal 1: For Case 5, if a SSB overlaps with Msg3 (re)transmission and PUCCH for Msg4, the existing NR TDD behaviour is reused, i.e., SSB is prioritized.

- No TP is needed since the existing specification already reflect the proposed behaviour

Proposal 2: It is proposed to agree on TP 1 for overlap handling of Case 5.

Proposal 3: On PUCCH repetition

- HD-FDD RedCap UE determines a slot as an available slot for PUCCH repetitions when a PUCCH repetition does not overlap with a symbol with synchronization signal block (SSB) transmission indicated by ssb-PositionsInBurst

- Agree on TP 2 for available slot determination for PUCCH repetition.

# References

[1] Chairman’s notes for RAN1#106bis-e

[2] TS 38.213-h00, ‘Physical layer procedures for control’

# ANNEX A: Overlap handling between DL and UL channels/signals in NR TDD

Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - If the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6, TS38.214], or the PRACH transmission in the set of symbols. - If the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6, TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols. - The UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0.

Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.

Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

For a set of symbols of a slot that are indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, the UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE in the set of symbols of the slot.

Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

Case 8: Dynamic or semi-static DL vs. valid RO

For a set of symbols of a slot corresponding to a valid PRACH occasion and symbols before the valid PRACH occasion, as described in Clause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

# ANNEX B: All RAN1 agreements

Agreements:

- For HD-FDD, for cases (if any) where collision handling needs to be specified, then the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum are used as a starting point if deemed applicable.

Agreements:

- (Working assumption) For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

  - FFS: whether to define the guard times in symbol units

  - FFS: the switching positions

- Sending an LS to RAN4 to inform the above working assumption, and to ask for feedback if any

  - The LS will not include the two FFS bullets

Agreements:

- For HD-FDD operation for RedCap UEs, consider at least the following DL/UL collision cases collisions may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary:

  - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

    - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH, or RO

  - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

    - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

  - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

  - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

  - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

    - e.g., PUSCH, PUCCH, PRACH, SRS

  - Case 6: Monitoring for UL cancellation indication (if supported) while transmitting in UL

  - Case 7: Collision due to BWP switching (if supported)

  - Case 8: Dynamic or semi-static DL vs. valid RO

  - Case 9: Collision due to direction switching

Agreements:

For Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission), reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum.

- FFS whether the timeline is extended to include the RX/TX switching time for HD-FDD

For Case 4: dynamically scheduled DL reception vs. dynamic scheduled UL transmission, reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum

- That is, it is considered as an error case if a dynamically scheduled DL reception overlaps with a dynamically scheduled UL transmission

For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier/single cell in unpaired spectrum

- The semi-statically configured DL reception may include PDCCH (excluding ULCI), SPS PDSCH, CSI-RS or PRS.

  - FFS on PDCCH carrying ULCI, including whether or not it is supported by RedCap UEs (including potential difference between HD vs. FD RedCap UEs)

- The dynamically scheduled UL transmission may include PUSCH, PUCCH, SRS or PRACH triggered by PDCCH order

Agreements:

For Case 3, semi-statically configured DL reception vs. semi-statically configured UL transmission

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and cell specific higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both cell specific higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- FFS on cell-specifically configured DL reception vs. cell-specifically configured UL transmission

- FFS: whether or not there are conditions that need to be considered

Working assumption: For HD-FDD, no additional UE behavior for switching position determination is specified as compared to the existing specification.

Conclusion: Enhancement for potential UL and DL collision handling due to TA misalignment is not considered for Type-A HD-FDD operation of RedCap UEs

Working Assumption: For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication

- A HD-FDD UE is not expected to transmit in the uplink earlier than [NRX-TX Tc] after the end of the last received downlink symbol in the same cell

- A HD-FDD UE is not expected to receive in the downlink earlier than[NTX-RX Tc] after the end of the last transmitted uplink symbol in the same cell

- FFS NTX-RX and NRX-TX

- FFS: how it jointly works with the agreement for other collision cases

Working assumption:

- If a dynamically scheduled UL transmission overlaps with an SSB, down-select one of the following options:

  - Option 1: Follow the handling of case 2 that dynamic UL is prioritized over SSB

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamic UL

  - Option 3: Leave to UE implementation (e.g. UE can receive the SSB if UE needs to receive the SSB; otherwise, UE can transmit the UL transmission) whether to receive the SSB or transmit the UL transmission

  - Other options are not precluded

- If a semi-static configured UL transmission overlaps with an SSB, down-select one of from the following options

  - Option 1: Up to gNB configuration to avoid such collision and if it happens it is an error case

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over semi-static UL

  - Option 3: Leave to UE implementation (e.g. UE can receive the SSB if UE needs to receive the SSB; otherwise, UE can transmit the UL transmission) whether to receive the SSB or transmit the UL transmission

  - Other options are not precluded

- FFS: whether/how to account for Tx/Rx switching time before and after the set of SSB symbols

- FFS: whether or not the semi-static configured UL transmission includes a valid RO

Agreement:

- For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), a HD-FDD RedCap UE is not required to monitor ULCI

  - No special handling on the priority rule for PDCCH carrying ULCI

Conclusion:

- No consensus of specification support of semi-static UL/DL pattern to HD-FDD RedCap UEs in Rel-17.

Agreement:

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured PDCCH

  - Option 2: Leave to UE implementation whether to receive the configured PDCCH or transmit the PRACH on the valid RO

  - Option 3: If configured PDCCH is in a Type-2 CSS set, then PDCCH is prioritized; otherwise the valid RO is prioritized

  - Option 4: Configured PDCCH is prioritized over valid RO

  - Option 5: Configured by network, e.g. via a priority indicator

  - FFS: whether or not the set of symbols overlapping with PDCCH in CSS set includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

  - FFS whether a valid RO follows TDD’s or FDD’s definition, and if so, the corresponding impact

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

Agreement:

- For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured DL

  - Option 2: Leave to UE implementation whether to receive the configured DL or transmit the PRACH on the valid RO

  - Option 5: Configured by network, e.g. via a priority indicator

  - Other options are not precluded.

  - FFS: whether or not the set of symbols overlapping with configured DL includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

Agreement:

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD for operation on a single carrier /single cell in unpaired spectrum

  - Option 2: Leave to UE implementation whether to receive the DL or transmit the PRACH on a valid RO

  - Option 3: Follow the handling of Case 1 to cancel PRACH based on a timeline that when the cancellation timeline is satisfied, the UE cancels the PRACH transmission and receives the DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 2 in R1-2103809)

  - Option 4: Valid RO is prioritized over dynamic DL that UE performs PRACH transmission and does not perform the DL receptions (Interpretation 3 in R1-2103809)

  - Option 5: When the cancellation timeline is satisfied, the UE neither performs transmission nor receives any DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 1 in R1-2103809)

  - FFS: whether or not the set of symbols overlapping with dynamic DL reception includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

Agreement:

- For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission

  - The configured UL transmission includes CG-PUSCH, or SRS

  - FFS: Confirm that PUCCH is included

Agreement

- For Case 5 of SSB overlaps with configured UL transmission, the configured UL transmission includes PUCCH transmission configured by higher layers

- Note:  The UL transmission indicated by DCI is supposed to be dynamic UL transmission.

Working Assumption

- For Type-A HD-FDD UEs, all ROs applicable to RedCap UEs are valid (same as FD-FDD RedCap UEs), and for the case of SSB overlapping with valid RO from cell specific point of view, leave it to UE implementation whether to receive SSB or transmit PRACH

- No support of differentiating of ROs for Type-A HD-FDD Redcap UEs and FD FDD RedCap UEs

Working Assumption

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH

- FFS: whether or not there are conditions (e.g., exception for valid RO not intended for PRACH transmission) that need to be considered.

- Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2.

Agreement

Confirm this Working Assumption.

Working Assumption

- For Type-A HD-FDD UEs, all ROs applicable to RedCap UEs are valid (same as FD-FDD RedCap UEs), and for the case of SSB overlapping with valid RO from cell specific point of view, leave it to UE implementation whether to receive SSB or transmit PRACH

- No support of differentiating of ROs for Type-A HD-FDD Redcap UEs and FD FDD RedCap UEs

Agreement

Confirm this Working Assumption.

Working Assumption

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH

- FFS: whether or not there are conditions (e.g., exception for valid RO not intended for PRACH transmission) that need to be considered.

- Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2.

Agreement

- For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), leave it to UE implementation whether to receive the DL or transmit PRACH

- Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2.

Agreement

- For Case 5 of dynamically scheduled UL transmission vs. SSB, one or both of the following options to be determined till next meeting:

  - Option 1: Dynamically scheduled UL transmission is prioritized over SSB

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

- FFS: whether or not the same UE behavior is applied to Msg3 (re)transmission and PUCCH for msg4

Agreement

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, downselect one of following options in next meeting

  - Option 2: Leave to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH

  - Option 3: Follow the handling of Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission)

  - Option 4: Valid RO is prioritized over dynamic DL reception

Agreement

For Case 1, the existing timeline in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum is reused for HD-FDD

Agreement

- For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

Note: With this agreement, no need to confirm below Working Assumption(From RAN1#104e)

Working Assumption (FromRAN1#104e )

- For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

  - FFS: whether to define the guard times in symbol units

  - FFS: the switching positions

Conclusion:

- No consensus on defining a guard time in symbol units for HD-FDD Type A operation in Rel-17

Agreement

Revise the RAN1#104bis-e agreement for Case 3 as the following

- For Case 3, semi-statically configured DL reception vs. semi-statically configured UL transmission

  - A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

  - A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and cell specific higher layer parameters configuring reception in the set of symbols of the slot

    - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set

  - FFS: whether or not there are conditions that need to be considered

Agreement

- For Type-A HD-FDD, no additional UE behaviour for UL/DL collision handling based on a priority indicator is specified as compared to the existing specification

Agreement

- Whether or not to account for the Tx/Rx switching time before and after the set of SSB symbols can be further discussed under Case 9

Agreement

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, leave it to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH

Agreement

- The same validation rules of MsgA PUSCH occasions and RO/Preamble-to-PRU mapping rules for FDD can be reused for HD-FD

Agreement

- For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication

  - A HD-FDD UE is not expected to transmit in the uplink earlier than NRX-TX Tc after the end of the last received downlink symbol in the same cell

  - A HD-FDD UE is not expected to receive in the downlink earlier than NTX-RX Tc after the end of the last transmitted uplink symbol in the same cell

  - NRX-TX Tc and NTX-RX Tc are the same as the transition time for FR1 in Table 4.3.2-3, TS 38.211 for a UE not capable of full-duplex communication

- (Working Assumption) The “back-to-back” non-overlapping UL/DL without sufficient gap between RRC configured UL and DL may happen, i.e., are allowed for HD-FDD UEs.

  - RRC configured DL/UL includes at least cell specific higher layer parameters configured DL/UL

  - Discuss further whether to specify a clear UE behavior, or leave it to UE implementation to ensure that the switching time is satisfied

  - Note: This does not mean a HD-FDD UE is required to support the back-to-back UL/DL switching without sufficient gap

Agreement

- For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

Agreement

- For MsgA PUSCH occasion overlapping with dynamic or semi-static DL reception, leave it to UE implementation to prioritize the DL reception or MsgA PUSCH transmission

Agreement

- For the case of the “back-to-back” non-overlapping UL/DL without sufficient gap between cell specific configured DL and cell-specific configured UL, e.g., SSB or PDCCH in CSS vs. valid RO, it is up to UE implementation to ensure that the switching time is satisfied

Agreement

      The “back-to-back” non-overlapping UL/DL without sufficient gap between cell-specific configured DL and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs

  - E.g., SSB vs. CG PUSCH, PUCCH or SRS

  - Configured UL transmission is cancelled (as in the overlapping case)

- The “back-to-back” non-overlapping UL/DL without sufficient gap between dedicated configured DL and cell-specific configured UL may happen, i.e., allowed for HD-FDD UEs

  - E.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS vs. valid RO

  - Leave it to UE implementation to cancel either DL reception or UL transmission to ensure sufficient switching time

Agreement

      No additional UE behavior for DL/UL collision handling is specified in Rel-17 if SFI monitoring is supported for HD-FDD RedCap UEs.