3GPP TSG RAN WG1 #108-e		R1-2201704

e-Meeting, February 21st – March 3rd, 2022

Source: 	Intel Corporation

Title: 	UE features for Rel.17 NR sidelink enhancements

Agenda item:	8.16.11

Document for: 	Discussion and Decision

# Introduction

In this contribution, we provide our views on UE features for NR sidelink enhancements based on input provided in [1] and considering agreement made at RAN1#107bis-e [2].

Working assumption (RAN1#107bis-e) Necessary Rel-16 FGs are handled, via pre-requisites of Rel-17 FGs for sidelink enhancements - Necessary components in Rel-16 FGs are added to Rel-17 FG components for sidelink enhancements if an entire Rel-16 FG cannot be included as pre-requisites - FFS whether/how to address the case where pre-requisites or additional components cannot address the issue for FG 15-x supported by Rel-17 UEs Agreements (RAN1#107bis-e) FG 32-4 is updated as follows - FFS whether capability for synchronization as components or prerequisite or another FG - FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a FG 32-4a is updated as follows - FFS whether capability for synchronization as components or prerequisite or another FG - FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a FG 32-5b is split into multiple FGs; One for Tx capability and the other for Rx capability as follows Conclusions (RAN1#107bis-e) There is no consensus in RAN1 on “FFS whether to split the capabilities for PSFCH and S-SSB receptions as different FGs” - Remove the FFS in FG 32-2, highlight FG 32-2 UE without NR SL reception is supported in Rel-17

# Views on UE Features for NR Sidelink Enhancement

## Intermediate Version of UE Features

The intermediate version of UE features for NR sidelink enhancements was captured in [1] and is expected to be used as a starting point for further discussion (please refer to Table 1):

Table 1: Intermediate version of feature groups for NR sidelink enhancements - R1-2200780 [1]

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 32-2 | Receiving NR sidelink of PSFCH/S-SSB | 1) UE can receive NR PSFCH/S-SSB. FFS whether other components will be included | None |
| 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] |
| 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] |
| 32-5a | Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a into multiple FGs | [TBD] |
| 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. | 32-5b-2, TBD |
| 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | [TBD] |

The above table is used as a starting point for further discussion and revisions in the next subsections.

## Feature Groups for Sidelink Enhancements

### FG 32-2: Receiving NR sidelink of PSFCH/S-SSB

For FG 32-2 it needs to be clarified whether reception of PSFCH is supported only for HARQ or both HARQ and IUC operation. In our view, RAN1 can proceed with either of the alternatives:

Alt.1: Clarify that “NR PSFCH/SSB reception” means “NR PSFCH HARQ/SSB” reception. In this case UE does not support resource reselection based on Scheme-2 IUC feedback.

Alt 2: Introduce two FGs in terms of PSFCH reception for HARQ and PSFCH reception for IUC as PSFCH reception for IUC require changes in UE resource allocation behavior.

In our view, pre-requisites need to be defined for FG 32-2 as UE supporting PSFCH/SSB reception is not a useful feature and thus at least FG 32-4a (TX-M2 w/random resource selection) should be pre-requisite for FG 32-2.

  - Select one of the following alternatives for FG 32-2 (Receiving NR sidelink of PSFCH/S-SSB):

    - Alt.1: Clarify that FG 32-2 is applicable for PSFCH HARQ reception

    - Alt.2: Split FG 32-2 into two FGs:

      - FG 32-2a: Receiving NR sidelink of S-SSB and PSFCH for HARQ operation

      - FG 32-2b: Receiving NR sidelink of S-SSB and PSFCH for IUC operation

      - FG32-2a is a pre-requisite of FG 32-2b

  - Define FG 32-4a (TX-M2 w/random resource selection) as a pre-requisite for FG 32-2/a/b

### FG 32-4: Transmitting NR sidelink mode 2 with partial sensing

Components and pre-requisites of the FG 32-4 are still under discussion. Any NR sidelink transmission require support of NR sidelink synchronization procedure. The following FG was agreed for NR sidelink synchronization support in R16. It includes all sources for sidelink synchronization (UE, gNB, GNNS):

15-4 Synchronization sources for NR sidelink

  - 1) UE can receive S-SSB in NR sidelink if it supports 15-1.

  - 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3.

  - 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false.

  - 4) UE can transmit or receive NR sidelink based on the synchronization to an gNB

  - 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb.

  - 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true.

- One of the pre-requisites for FG 15-4 is FG 15-1(Receiving NR sidelink)

15-1 Receiving NR sidelink

  - 1) UE can receive NR PSCCH/PSSCH. Up to a total of A sidelink HARQ processes across all links are supported.

  - 2) UE can receive X PSCCH in a slot.

  - 3) UE can attempt to decode Y= NRB non-overlapping RBs per slot

  - 4) UE supports reception of PSSCH according to the 64QAM MCS table

  - 5) UE supports PT-RS reception in FR2.

  - 8) UE can receive using the subcarrier spacing and CP length defined for a given band in RAN4

  - 10) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH.

  - 12) UE can receive using 30 kHz subcarrier spacing with normal CP in FR1, 120 kHz subcarrier spacing with normal CP FR2

Components of FG 15-1 and FG 15-4 are consistent with components of the FG 32-4. Given that FG 15-1 is a pre-requisite of FG 15-4, only FG 15-4 can be considered and added as a prerequisite for FG 32-4. As an optimization it can be considered to add only part of the components from FG 15-4 (e.g., corresponding to subset of synchronization sources).

Besides FG for synchronization, the FG 15-11 PSFCH Format 0 for sidelink HARQ transmission/reception can be added as FG or separate components. On the other hand, PSFCH support for HARQ can be indicated separately.

FG 15-4 (Synchronization sources for NR sidelink) is a pre-requisite of the FG 32-4

Consider defining FG 15-11 (PSFCH Format 0) as a pre-requisite of the FG 32-4

### FG 32-4a: Transmitting NR sidelink mode 2 with random resource selection

Components and pre-requisites of the FG 32-4a are still under discussion. RAN1 concluded that UE without NR SL reception is supported in Rel-17. Sidelink transmission with random resource selection can be performed by UEs that do not support sidelink reception. UEs w/o sidelink reception capability still require support of sidelink synchronization w/o SSB reception. Therefore FG 15-4 cannot be fully reused as a pre-requisite for FG 32-4a. Therefore, the following alternatives can be considered

Alt.1: Define new FG for sidelink synchronization w/o SSB reception support

Alt.2: Add components of the FG 15-4 except SSB reception support

Support of the FG 32-4a does not necessary imply UE without sidelink reception capabilities. For UEs with sidelink reception capabilities, the FG 15-4 can be reused.

Considering that FG 32-4a can be applied for UEs with and w/o sidelink reception capabilities, we have following proposal:

  - Define new R17 FG for sidelink synchronization w/o SSB reception support:

  - UE supports GNSS as the sidelink synchronization reference

  - UE supports gNB as the sidelink synchronization reference

  - UE can transmit S-SSB in NR sidelink when it uses GNSS or gNB as the sidelink synchronization reference

The following FGs are supported as a pre-requisite for FG 32-4a:

  - FG 15-4 (Sync sources for sidelink)

  - R17 FG for sidelink synchronization w/o SSB reception component

### FG 32-5a: Inter-UE coordination scheme 1 in NR sidelink mode 2

For inter-UE coordination scheme-1 we prefer to further split functionality required for transmission and reception of request and condition based inter-UE coordination feedback. In addition, the following container options were agreed for carrying scheme-1 IUC feedback: 1) MAC-CE and 2) MAC-CE + Stage-2 SCI. Therefore, we propose the following split of FG 32-5a:

FG 32-5a1: Support of request-based IUC feedback using MAC-CE

FG 32-5a2: Support of condition-based IUC feedback using MAC-CE

FG 32-5a3: Support of request-based IUC feedback using MAC-CE and Stage-2 SCI

FG 32-5a4: Support of condition-based IUC feedback using MAC-CE and Stage-2 SCI

- In addition, at the last meeting optional support of MAC-CE and Stage-2 SCI for request of inter-UE coordination information was received.

  - Split FG 32-5a to the following FGs:

    - FG 32-5a1: Support of request-based IUC using MAC-CE

      - TX of request for IUC information using MAC CE

      - RX of request for IUC information using MAC CE

      - TX of feedback with IUC information using MAC CE

      - RX of feedback with IUC information using MAC CE

    - FG 32-5a2: Support of condition-based IUC using MAC-CE

      - TX of condition-based feedback with IUC information using MAC CE

      - RX of condition-based feedback with IUC information using MAC CE

    - FG 32-5a3: Support of request-based IUC feedback using MAC-CE and SCI format 2C

      - TX of feedback with IUC information using MAC CE and SCI format 2C

      - RX of feedback with IUC information using MAC CE and SCI format 2C

    - FG 32-5a4: Support of condition-based IUC feedback using MAC-CE and SCI format 2C

      - TX of condition-based feedback with IUC information using MAC CE and SCI format 2C

      - RX of condition-based feedback with IUC information using MAC CE and SCI format 2C

    - FG 32-5a5: Support of request for IUC feedback using MAC-CE and SCI format 2C

      - TX of request for IUC information using MAC CE and SCI format 2C

      - RX of request for IUC information using MAC CE and SCI format 2C

### FG 32-5b: Inter-UE coordination scheme 2 in NR sidelink mode 2

For IUC scheme-2, the pre-requisites for FGs 32-5b1(transmission of scheme-2 feedback) and 32-5b2 (reception of scheme-2 feedback) are still under discussion.

In our view, reception of the scheme 2 feedback requires at least support of the FG 15-11 (PSFCH Format 0) or its components and FG 15-3 or its sensing related components

Transmission of the scheme 2 feedback additionally requires support of the FG 15-3 (TX M2) or FG 32-4 (TX M2 w/partial sensing)

  - Reception of the scheme 2 feedback (FG 32-5b2) requires support of the at least following FGs

    - FG 15-11 (PSFCH Format 0) or its PSFCH reception components

    - FG 15-3 (TX M2) or its sensing related components

  - Transmission of the scheme 2 feedback (FG 32-5b1) additionally requires support of the one of the following FGs

    - FG 15-11 (PSFCH Format 0) or its PSFCH transmission components

    - FG 15-3 (TX M2)

    - FG 32-4 (TX M2 w/partial sensing)

# Conclusions

In this contribution, we have provided our current views on UE features for R17 WI on NR sidelink enhancements. We observe that additional work progress is needed to finalize the list. In summary, at this stage we have following proposals:

  - Select one of the following alternatives for FG 32-2 (Receiving NR sidelink of PSFCH/S-SSB):

    - Alt.1: Clarify that FG 32-2 is applicable for PSFCH HARQ reception

    - Alt.2: Split FG 32-2 into two FGs:

      - FG 32-2a: Receiving NR sidelink of S-SSB and PSFCH for HARQ operation

      - FG 32-2b: Receiving NR sidelink of S-SSB and PSFCH for IUC operation

      - FG32-2a is a pre-requisite of FG 32-2b

  - Define FG 32-4a (TX-M2 w/random resource selection) as a pre-requisite for FG 32-2/a/b

FG 15-4 (Synchronization sources for NR sidelink) is a pre-requisite of the FG 32-4

Consider defining FG 15-11 (PSFCH Format 0) as a pre-requisite of the FG 32-4

  - Define new R17 FG for sidelink synchronization w/o SSB reception support:

  - UE supports GNSS as the sidelink synchronization reference

  - UE supports gNB as the sidelink synchronization reference

  - UE can transmit S-SSB in NR sidelink when it uses GNSS or gNB as the sidelink synchronization reference

The following FGs are supported as a pre-requisite for FG 32-4a:

  - FG 15-4 (Sync sources for sidelink)

  - R17 FG for sidelink synchronization w/o SSB reception component

  - Split FG 32-5a to the following FGs:

    - FG 32-5a1: Support of request-based IUC using MAC-CE

      - TX of request for IUC information using MAC CE

      - RX of request for IUC information using MAC CE

      - TX of feedback with IUC information using MAC CE

      - RX of feedback with IUC information using MAC CE

    - FG 32-5a2: Support of condition-based IUC using MAC-CE

      - TX of condition-based feedback with IUC information using MAC CE

      - RX of condition-based feedback with IUC information using MAC CE

    - FG 32-5a3: Support of request-based IUC feedback using MAC-CE and SCI format 2C

      - TX of feedback with IUC information using MAC CE and SCI format 2C

      - RX of feedback with IUC information using MAC CE and SCI format 2C

    - FG 32-5a4: Support of condition-based IUC feedback using MAC-CE and SCI format 2C

      - TX of condition-based feedback with IUC information using MAC CE and SCI format 2C

      - RX of condition-based feedback with IUC information using MAC CE and SCI format 2C

    - FG 32-5a5: Support of request for IUC feedback using MAC-CE and SCI format 2C

      - TX of request for IUC information using MAC CE and SCI format 2C

      - RX of request for IUC information using MAC CE and SCI format 2C

  - Reception of the scheme 2 feedback (FG 32-5b2) requires support of the at least following FGs

    - FG 15-11 (PSFCH Format 0) or its PSFCH reception components

    - FG 15-3 (TX M2) or its sensing related components

  - Transmission of the scheme 2 feedback (FG 32-5b1) additionally requires support of the one of the following FGs

    - FG 15-11 (PSFCH Format 0) or its PSFCH transmission components

    - FG 15-3 (TX M2)

    - FG 32-4 (TX M2 w/partial sensing)

# References

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-bis-e, Moderators (AT&T, NTT DOCOMO, INC.), January 2022

- R1-2200256, Summary on UE features for NR sidelink enhancement, Moderator (NTT DOCOMO, INC.), January 2022

- Annex A: UE features for NR sidelink enhancements from [R1-2200780] – After RAN1#107bis-e

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supports | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of PSFCH/S-SSB | 1) UE can receive NR PSFCH/S-SSB. FFS whether other components will be included | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the random resource selection and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: Component 4 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-4 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-4 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-4 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 6 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a | Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. | 32-5b-2, TBD | [Yes] | [Yes] | UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | [TBD] | [Yes] | [Yes] | UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |