3GPP TSG RAN WG1 #108-e			R1-2201716

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	Remaining Opens of Sidelink Inter-UE Coordination Schemes

Agenda item:	8.11.1.2

Document for:	Discussion and Decision

Introduction

The RAN WG approved work item on NR Sidelink Enhancements [1] with the following objective:

- Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. Note: RAN2 work will start after RAN#89.

In this contribution, we continue discussion on remaining open issues of sidelink inter-UE coordination framework considering agreements made at the previous meetings. Our views on remaining details of sidelink power saving objective are provided in companion contribution [2].

# Inter-UE Coordination Scheme-1

## Inter-UE Coordination Information and Signaling

### Sidelink RSRP thresholds for preferred / non-preferred resource sets

RAN1 agreed on RSRP conditions for generation of preferred and non-preferred resource sets:

Agreements from RAN1#106bis-e - Condition 1-A-1 (preferred resource set) - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold - Condition 1-B-1 (non-preferred resource set) - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s) Agreement from RAN1#107-e - For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS 38.214 Section 8.1.4. - FFS: Whether/how to introduce the maximum limit of RSRP threshold increase

In our understanding, pre-configured fixed RSRP settings were agreed for non-preferred resource sets while the RSRP thresholds for preferred resource sets can be incremented to reach minimum size of preferred resource set. If both preferred and non-preferred resource sets are enabled for inter-UE coordination feedback the same resource may become a part of both preferred and non-preferred resource sets. To avoid this situation, we have the following proposal:

  - Support pre-configuration of the maximum value of RSRP threshold for generation of preferred resource set, so that the following conditions can be controlled for RSRR thresholds

    - ThrRSRPpreferred ≤ ThrRSRPmax,preferred ≤ ThrRSRPnon-preferred (Option 1)

    - ThrRSRPnon-preferred (Option 2) ≤ ThrRSRPpreferred

### Preferred and non-preferred resource set conditions

Preferred resource set

RAN1 made the following agreement on Condition 1-A-2 to determine a set of resources preferred for UE transmission.

- Agreement from RAN1#106e - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission - FFS: Condition 1-A-2: - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B - FFS: Other details (if any) Working Assumption from RAN1#106bis-e - For Scheme 1 with preferred resource set, support following condition: - Condition 1-A-2: - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation - This can be disabled by RRC (pre-)configuration Agreement RAN1#107-e - For Condition 1-A-2 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4 - UE-A excludes candidate single-slot candidate(s) belonging to “slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation” after Step 6) of TS 38.214 Section 8.1.4

Non-preferred resource set

RAN1 made the following agreement on Condition 1-B-2 to determine a set of resources non-preferred for UE transmission

- Agreement from RAN1#106 - UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission - FFS: Condition 1-B-2: - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B - FFS: Other details (if any) Working Assumption from RAN1#106bis-e - For Scheme 1 with non-preferred resource set, support following condition: - Condition 1-B-2: - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

Clarification of “half-duplex operation”

In our view the meaning of half-duplex operation in both conditions (1-A-2 and 1-B-2) needs to be clarified. In our understanding, whether UE decides to perform SL reception or SL/UL transmission that may results in half-duplex is dependent on priority of sidelink reception (indicated by TX-UE in SCI) vs priority of sidelink or uplink transmission that leads to half-duplex problem. For definition, clarifying half-duplex operation please refer to section 2.11.

Separate indication of non-preferred resources sets associated with conditions 1-B-1 and 1-B-2

The non-preferred resource set is generated based on conditions: 1-B-1(RSRP criteria) and 1-B-2(half-duplex criteria). Non-preferred resources associated with half-duplex criteria should not be applied for broadcast transmissions or for unicast transmissions when source of feedback and destination of sidelink transmission are different UEs. To handle such scenarios by TX UE, we propose to support separate indication of non-preferred resources associated with condition 1-B-1 and condition 1-B-2.

  - For non-preferred resource set,

    - Resources generated based on Condition 1-B-1 and 1-B-2 are indicated separately

### Indication of parameters for request-based feedback

The following list of parameters was agreed to be provided by TX-UE to UE providing request-based feedback

- Agreement from RAN1#106bis-e - For Condition 1-A-1 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4 - When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4. - Priority value to be used for PSCCH/PSSCH transmission - It replaces prio_TX - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot - It replaces L_subCH - Resource reservation interval - It replaces P_rsvp_TX - FFS: Starting/ending time location of resource selection window Agreement from RAN#107-e - For Scheme 1, at least following parameters are provided by UE-B’s request: - Priority value to be used for PSCCH/PSSCH transmission - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot - Resource reservation interval Agreement from RAN#107bis-e - For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request, - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

Container and remaining parameters (if any) of request for inter-UE coordination feedback are still under discussion. The following design alternatives are possible:

Priority, number of sub-channels and resource reservation interval for request based inter-UE coordination feedback

    - Alt.1: are indicated in SCI format-2C and MAC-CE of UE-B’s transmission carrying request for feedback

    - Alt.2: are indicated in MAC-CE only of UE-B’s transmission carrying request for feedback

    - Alt.3: are derived from SCI format-1A of UE-B’s transmission carrying request for feedback

The start/end time locations (or start and duration) of resource selection window

    - Alt.1: are indicated in SCI format-2C and MAC CE of UE-B’s transmission carrying request for feedback

    - Alt.2: are indicated in MAC CE only of UE-B’s transmission carrying request for feedback

Considering that per pool pre-configuration was agreed to determine whether SCI format 2C + MAC CE or MAC CE only options are supported for request signaling, we think that the following alternatives should be supported:

  - The following alternatives are supported to indicate parameters of the request for inter-UE coordination feedback:

    - Alt.1: Parameters are indicated in SCI Format-2C and MAC CE carrying request for feedback

    - Alt.2: Parameters are indicated in MAC CE only carrying request for feedback

  - The start and end timing locations of resource selection window are indicated as a start slot index associated with (n+T1) and duration (T2-T1) of resource selection window

    - Start slot index is defined relative to the reference slot which is separately indicated in MAC CE or indicated as offset relative to the slot carrying SCI format-2C

### Indication of parameters for condition-based feedback

- The following agreement was made by RAN1 for determining preferred set of condition based inter-UE coordination feedback

Agreement 107bis-e: - For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception, - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters - prio_TX - L_subCH - P_rsvp_TX - UE-A determines by its implementation values of following parameters - n+T_1, n+T_2 - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2 - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set

Parameters for determining preferred resource set

For determining preferred resource set in condition-based feedback, the values for subset of parameters (priority, number of subchannels, resource reservation interval) are either pre-configured or determined by UE implementation, if pre-configuration is not provided. Determination of start/end timing locations of resource selection window should be also left up to UE implementation subject to certain conditions, however duration of resource selection window should be preconfigured for condition-based feedback. Otherwise, feedback provided by different UEs can have different value/meaning especially if preferred resource set is considered. The pre-configuration of resource selection window duration can be done per priority and at least should be supported for the case when pre-configuration of parameters such as priority, number of subchannels and reservation period are provided.

Regarding remaining pre-configuration parameters, majority of those can be directly reused from existing parameters applied for sidelink sensing and communication framework. One additional parameter that can be separately configured for condition-based inter-UE coordination feedback is the relative threshold X that controls minimum number of preferred resources.

  - For condition-based feedback, support pre-configuration of at least the following parameters

    - resource selection window duration (T2-T1)

    - minimum number of candidate single-slot resources for feedback in percent – X

### Timing Relationships for Feedback Resource Selection Windows

Timing relationship for request-based feedback

The timing relationship of timing locations for resource selection window indicated in request, resource selection window used for feedback, initial feedback transmission and feedback sensing window need to be defined. To define these timing relationships, we introduce the following notations:

(n+T1) – start slot of resource selection window indicated in request for feedback

(n+T2) – end slot of resource selection window indicated in request for feedback

(n’+T1’) – start slot of resource selection window used for feedback generation

(n’+T2’) – end slot of resource selection window used for feedback generation

mFBCK – slot carrying initial transmission with request based-feedback

mSensing – end slot of sensing window carrying feedback

Tproc,3 – processing time to bound maximum time interval in slots b/w initial transmission carrying request-based feedback and start of resource selection window used for request-based feedback generation

Tproc,4 – processing time to bound maximum time interval in slots b/w end slot of sensing window and start slot of resource selection window used for request-based feedback generation

Figure 1: Timing relationships for request based inter-UE coordination feedback

We propose to define the following timing relationships for resource selection window indicated in request for feedback by UE-B and resource selection window used for feedback generation:

(n+T1) ≤ (n'+T1')

(n'+T2') ≤ (n+T2)

T2,min ≤ (T2'-T1')

In addition, to ensure that transmission of the feedback contains the most recent sensing information the following timing relationships need to be defined:

(n'+T1') – mFBCK ≤ Tproc,3; e.g., Tproc,3 = Tproc,1

(n'+T1') – mSensing ≤ Tproc,4;  e.g., Tproc,4 = Tproc,0 + Tproc,1

  - Define the following timing relationships for request-based inter-UE coordination resource selection window:

    - (n+T1) ≤ (n'+T1')

    - (n'+T2') ≤ (n+T2)

    - T2,min ≤ (T2'-T1')

    - (n'+T1') – mFBCK ≤ Tproc,3; e.g., Tproc,3 = Tproc,1

    - (n'+T1') – mSensing ≤ Tproc,4; e.g., Tproc,4 = Tproc,0 + Tproc,1

  - UE indicates start/end time of feedback resource selection window (i.e., slots (m+T1') and (m+T2’) in feedback content (MAC CE)

Timing relationship for condition-based feedback

The condition-based feedback should contain the most recent sensing information and thus the following timing relationships need to be met:

(n'+T1') – mFBCK ≤ Tproc,3 = Tproc,1

(n'+T1') – mSensing ≤ Tproc,4 = Tproc,0 + Tproc,1

  - Define the following timing relationships for condition-based inter-UE coordination resource selection window:

    - (n'+T1') – mFBCK ≤ Tproc,3 = Tproc,1

    - (n'+T1') – mSensing ≤ Tproc,4 = Tproc,0 + Tproc,1

  - UE indicates start/end time of feedback resource selection window (i.e., slots (n'+T1') and (n'+T2') in feedback content (MAC CE)

### On dynamic and semi-persistent transmission for inter-UE coordination

First, RAN1 needs to discuss whether procedure for generation of preferred/non-preferred resource sets considers:

Option 1: Semi-persistent transmissions only

Option 2: Dynamic and semi-persistent transmissions

According to our analysis, major benefits from inter-UE coordination scheme 1 are observed for semi-persistent transmissions. It can be explained by the fast aging of resource set/feedback, if it is constructed based on dynamic transmissions.

  - For feedback generation (construction of preferred/non-preferred resource sets) support the following alternatives

    - Alt.1: Sensing and resource exclusion procedure considers only semi-persistent transmissions for construction of inter-UE coordination feedback

    - Alt.2: Sensing and resource exclusion procedure considers both semi-persistent and dynamic transmissions

## Determination of UEs for Inter-UE Coordination Feedback

The following options were agreed as conditions to serve as assisting UE (UE-A):

- Option 1 (request based): A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A - Option 2 (condition based): A UE that satisfies the condition (other than explicit request reception) and sends inter-UE coordination information is UE-A

In this section, we discuss additional conditions for the following procedures:

Transmission of request for feedback (conditions for TX UE-B)

Transmission of request-based feedback (conditions for UE-A)

Transmission of condition-based feedback (conditions for UE-A)

### Conditions for transmission of feedback request

RAN1 agreed on the following conditions for transmission of feedback request by TX UE

Agreement RAN1#107bis-e - For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1, - A resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1: it is up to UE-B’s implementation whether to trigger the request generation - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

The request for inter-UE coordination feedback was agreed to be supported at least for unicast transmissions. In order, to control frequency of standalone request transmissions at system level, we propose to support additional elapsed timer condition for generation of standalone feedback request at least for Alt.1

  - Introduce pre-configuration of the minimum elapsed time for transmission of standalone request for inter-UE coordination feedback

### Conditions for transmission of request-based feedback

RAN1 agreed on the following conditions for request-based feedback transmission

Agreement (RAN1#107bis-e) - For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures. - Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control

- On sufficient sensing information

In our view, additional condition related to the amount of available sensing information should be considered as a condition for transmission of a request-based feedback (e.g., if a UE was transmitting most of the time and does not have enough sensing information it should not provide feedback).

  - For request-based feedback, introduce pre-configuration of the minimum number of sensed slots in a sensing window as a condition for transmission of inter-UE coordination feedback

- On maximum latency of request-based feedback

To extract performance benefits from request-based feedback, it should be delivered in time and UE requesting feedback should know till what time it can expect request-based feedback.

UE requesting inter-UE coordination feedback indicates requested resource selection window for feedback and by default expects to receive it right before the start of requested resource selection window. UE providing feedback may not be able to deliver it right before the start of the requested resource selection window and feedback may be transmitted within resource selection window. The last slot for transmission of request-based feedback can be determined by the following condition:

(n+T2) - T2,min ≥ ∆ (e.g., ∆ = 0)

  - Introduce pre-configuration of time offset relative to the end (or start) slot of resource selection window to control the maximum delay for request-based feedback transmission

### Conditions for transmission of condition-based feedback

RAN1 agreed on the following conditions for condition-based feedback transmission.

Agreement RAN1#107bis-e - For inter-UE coordination triggered by a condition rather than request reception in Scheme 1, - A resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

For condition-based feedback, availability of sufficient amount of sensing information should be also considered as a condition for generation or transmission of inter-UE coordination feedback.

  - Introduce pre-configuration of the minimum number of sensed slots in a sensing window as a condition for transmission of condition-based inter-UE coordination feedback

    - Before generation of inter-UE coordination feedback, UE should check whether number of sensed slots for feedback generation exceeds pre-configured value

## Signaling and Content of Inter-UE Coordination Feedback and Request

The following working assumptions and agreements were made during RAN1#107b-e as a refinement of the agreements made during RAN1#107-e:

Agreement (RAN1#107-e) - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - FFS: Whether/How to use resource reservation information as coordination information Agreement (RAN1#107b-e) - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - (Working Assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A - A single format SCI 2-C is used for inter-UE coordination information and request - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information - SCI 2-C is UE RX optional - It is up to UE implementation to additionally use 2nd SCI (for UE-B). - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A - The following working assumption is confirmed with modification in RED. - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

According to above, Stage-2 SCI and MAC CE signaling needs to be designed. The corresponding signaling details and feedback content are discussed in the following sub-sections.

### Stage-2 SCI signaling of inter-UE coordination feedback and request

For Stage-2 SCI, the following conditions and side information are available:

A single resource set can be signaled only

In the range of 0 to 3 triplets of TRIV, FRIV, and Preserve can be signaled

Current slot is known as 2nd stage SCI can change across different transmissions of the same TB

Signaling whether this SCI contains inter-UE coordination feedback or request

Based on RAN1 agreement, it is in principle possible to transmit preferred, non-preferred and half duplex resource set in the SCI. This in turn generates a lot of different signalling design option as well as in the worst case a large number of additional bits inside of the 1st and 2nd stage SCI. Thus, our preference is to limit the usage of the 2nd stage SCI signalling to preferred resource set only.

  - Clarify that only the preferred resource set can be signaled in the 2nd stage SCI

New Stage-2 SCI format need to be defined and indicated in the 1st stage SCI replicating all Rel-16 2nd stage SCI   functionality with Rel.17 inter-UE coordination feedback. This means that the functionality of both current 2nd stage SCI formats 2-A and 2-B need to be contained within the new SCI signaling. This can be achieved by signaling within 2nd stage SCI format 2-C if the fields of 2-A or 2-B are present in it.

  - SCI format 2-C contains a field indicating if the fields of SCI format 2-A or 2-B are present

In the last meeting an agreement was added that the SCI size is always calculated under the assumption of 3 triplets. In general, this is not enough as this SCI can possibly has a different size dependent on if Preserve values are present or not. This SCI can also signal an inter-UE coordination feedback request. As before the decoding of the SCI the number of SCI bits needs to be known and this maximum size is dependent on the resource pool configuration a more general statement on the assumed SCI size is necessary.

  - The size of SCI for inter-UE coordination feedback or feedback request is always the maximum possible required number of signaling bits given the resource pool configuration

The new signaling of the 2nd stage SCI mainly contains information about the resources itself. The RAN1 agreement states that the SCI size is fixed to the maximum number of signaled triplets of 3. This means that in principle the number of present triplets can be in the range from 1 to 3.

Regarding the reference slots the following agreements were made during RAN1#107b-e:

Working assumption RAN1#107bis-e - First resource location of each TRIV is a slot offset with respect to a reference slot - Alt 2: - The slot offset is the number of logical slots from the reference slot - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256] - FFS: The detailed value range including granularity - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information - For the reference slot, - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

As in the Rel-16 definition of the TRIV, the current slot carrying SCI is defined as the starting slot, it is necessary to signal the starting slot for inter-UE coordination feedback. A different form of signaling can be used if information of other triplets is already available. As for the 2nd stage SCI the current slots can always be taken as a reference the starting slot of the first triplet is signaled as a slot offset relative to the slot which contains the SCI. To save signaling bits the starting slot for all other triplets can be relative to the last signaled resource of the preceding triplet.

For the Rel-16 definition of the FRIV, it is assumed that the starting sub-channel of the resource in the slot that contains the SCI is known (position of the PSCCH). In the case of using the FRIV for inter-UE coordination feedback this information is unknown and needs to be signaled in addition to the FRIV.

  - SCI format 2-C contains the following fields in addition to the already agreed TRIV, FRIV, Preserve, and start slot for each TRIV:

    - Indication that the SCI contains inter-UE coordination feedback

    - Indication whether the fields of 2-A or 2-B are present

    - Number of present triplets in the range of 1 to 3

    - Starting sub-channel of the first resource in a triplet

### MAC CE signaling of inter-UE coordination feedback

MAC CE signaling can be similar to the Stage-2 SCI. There are however a few side conditions that make it different.

Based on the following agreement reached during RAN1#107bis-e, only one resource set can be present for one instance of MAC CE signaling:

Agreement (RAN1#107bis-e): - For inter-UE coordination information is triggered by UE-B’s request, - A resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1: - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set - Alt 2: - Resource set type to be provided by inter-UE coordination information transmission is indicated by UE-B’s request - UE-B’s request indicates either preferred resource set or non-preferred resource set - Note that it is up to RAN2 decision whether/how UE-B provides its support of sensing/resource exclusion to UE-A via PC5-RRC signaling and UE-A uses the received information to determine the type of resource set to be transmitted to UE-B Agreement (RAN1#107bis-e): - For inter-UE coordination information is triggered by a condition other than explicit request reception, - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

Thus, the main difference to the signaling of resource sets in the SCI is that the reference for the starting slot cannot use the slot of the transmission as PSSCH content of multiple retransmissions need to be the same. Also, the number of triplets that are signaled need to be part of the MAC CE signaling. For the half duplex set only time information is required.

  - MAC CE signaling for resource sets with frequency information contains the following fields

    - Resource set type for the case when it is determined by UE-A

    - Number of signaled resources

    - Start slot of the first triplet in either of the following cases:

      - Note: Can be omitted if the start slot is a part of the feedback request

    - Start slot of all other triplets is signaled relative to the last resource in preceding triplet in logical slots

    - TRIV for each triplet

    - FRIV for each triplet (not needed if only slots are signaled)

    - Starting sub-channel of the first resource in a triplet

    - Preserve for each triplet as an index of all Preserve values configured for the resource pool

Below we provide our views on additional content of MAC-CE inter-UE coordination feedback signaling. Beside resource sets, at least the following additional information can be signaled as a part of inter-UE coordination feedback:

Additional MAC CE content

  - Start and end time of resource selection window for feedback (e.g., start and end slot index, start and duration)

    - This information is needed to determine aging time of feedback and whether provided feedback can be used for resource selection by the TX UE. It is applicable to both request and condition-based feedback.

  - Source ID(s) associated with transmissions on non-preferred resources

    - This information can be useful to mitigate the half-duplex problem as TX UEs may try to avoid selecting resources reserved by target RX UEs.

  - SL-RSRP measurements associated with non-preferred resources and priority level of sidelink transmissions associated with non-preferred resources

    - Beneficial to predict SL-SINR on reserved resource and improve resource selection respecting priority of transmissions

  - Periodicity of reservation associated with non-preferred resources (at least indication dynamic or semi-persistent reservation)

    - Beneficial for enhanced resource selection and from aging consideration perspective

  - The following additional information is provided as part of MAC-CE inter-UE coordination feedback signaling

    - Start/end time of resource selection window for feedback (e.g., start / end slot index)

      - Applicable for both the preferred and the non-preferred resource set

    - Source ID(s) associated with transmissions on non-preferred resources

## Stage-2 SCI and MAC-CE Inter-UE Coordination Feedback

Discussion on latency benefits

Latency benefit was used as a main argument to support stage-2 SCI container for inter-UE coordination feedback (preferred resource set) on top of MAC-CE. The potential latency benefits of stage-2 SCI container may come from reduced time for the following latency components:

MAC-CE and Stage 2 SCI preparation time + PSCCH/PSSCH preparation time

Transmission time and reception time

    - PSCCH/PSSCH transmission duration, including number of retransmissions

    - Decoding time for PSCCH/PSSCH

MAC-CE vs Stage 2 SCI processing time

On MAC CE / Stage-2 SCI preparation time

The MAC-CE and Stage-2 SCI preparation time is expected to be different (i.e., Tprep,SCI2 ≤ Tprep,MAC-CE). The resource set for inter-UE coordination feedback is expected to be updated every slot and thus information in MAC-CE may be outdated comparing to Stage-2 SCI containers (e.g., resource indicated as preferred in MAC-CE container may be already reserved).

To simplify design, content of both containers can be synchronized (associated with the same timeline/resource set), obtained based on the max(Tprep,SCI2, Tprep,MAC-CE). The end slot of sensing window corresponding to resource set should be determined by maximum preparation time relative to time instance of initial transmission, so that only the most recent sensing results are included in feedback. The time T3 = Tproc,1 can be reused to bound MAC-CE and Stage-2 SCI preparation time(s).

Figure 2: Stage-2 SCI and MAC-CE preparation time

  - Introduce maximum preparation time for inter-UE coordination feedback, including MAC-CE, Stage-2 SCI and PSSCH/PSCCH preparation times, which is bounded by T3 (e.g., T3 = Tproc,1)

    - The maximum time for inter-UE coordination feedback preparation is calculated in slots starting from the end slot of sensing window used for feedback generation

On selection of resources for Stage-2 SCI

To achieve latency benefits, resources indicated in Stage-2 SCI should precede the slot with successful MAC CE processing, otherwise there is no latency gain relative to the MAC-CE only container. This implies that the first(early) in time resource selection procedure should be applied for determination of resources indicated in Stage-2 SCI container. In addition, re-evaluation of feedback resources indicated in Stage 2 SCI transmission(s) should be supported. The sub-set of Stage-2 SCI resources can be re-evaluated for all transmissions of a given TB using the remaining valid set of resources indicated in MAC-CE.

Finally, the processing time bounds for Stage-2 SCI and MAC-CE can be considered in determination of resources indicated in Stage-2 SCI and MAC-CE feedback.

On number of resources indicated in Stage-2 SCI

It was agreed that it is up to UE implementation to additionally use Stage-2 SCI if [N <= 3] (SCI format 2C), where N is a number of triplets. For preferred resource set, threshold on X% of resources is applied. Therefore, minimum number of resources in preferred resource set is at least X% Mtotal and thus there is static relationship between N and X% Mtotal. We are not sure if it was the original intention of the proponents of Stage-2 SCI design. In our view, there is no need to use N <= 3 as a condition allowing SCI format 2-C for feedback transmission. The [N = 3] condition can be assumed to determine field size for indication of resource set in SCI format 2-C.

  - Apply the following change in the previous agreement:

    - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

  - Define the first(early) in time resource selection procedure to determine resources indicated in SCI format 2-C

  - Support re-evaluation of resources indicated in SCI format 2-C based on the remaining valid set of resources indicated in MAC-CE

## Aging of Inter-UE Coordination Feedback

Generation of inter-UE coordination feedback at assisting UE is expected to be associated with the reference time represented by either feedback generation time, the last sensed slot used for feedback generation tgen or resource selection window start time. For each generated feedback, the aging time can be defined as follows tagging = tcurr - tgen where tcurr is a current time / slot index. The aging time can be used to decide whether the specific inter-UE coordination feedback can be considered for resource allocation procedure.

For scheme 1, the maximum allowed feedback aging time may depend on how it was constructed (e.g., whether only dynamic transmissions were considered for construction of the preferred and non-preferred resource set or only semi-persistent transmissions were considered and allowed sets of resource reservation periods).

  - For inter-UE coordination scheme 1,

    - Feedback is associated with a reference slot (feedback timestamp)

    - Introduce feedback aging time (feedback delay) as an elapsed time since a reference feedback slot

    - Feedback aging time is used to decide on whether feedback can be considered by TX UE in resource allocation procedure for transmission

## Resources for R17 Sidelink Transmissions

Resource for transmission of inter-UE coordination feedback or request can be determined based on the Mode-2 resource allocation framework, including potential Rel.17 enhancements. RAN1 has reached the following agreement on this aspect:

Agreement RAN1#107bis-e - For sidelink transmission carrying inter-UE coordination information in Scheme 1, - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B. - For sidelink transmission carrying request in Scheme 1, - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection - Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request.

The above agreement does not specify whether to use R16 or R17 resource selection procedure for transmissions carrying inter-UE coordination feedback or request. At this stage, it is undefined how to determine whether R16 or R17 procedure should be used for sidelink transmissions not carrying inter-UE coordination information or request.

From system perspective, there should be a mechanism to configure capable R17 UEs to follow R17 resource selection procedure for all sidelink transmissions and to activate generation / transmission of inter-UE coordination feedback.

  - Introduce pre-configuration to configure R17 UEs to apply R17 resource selection procedure for sidelink transmissions and procedures for generation and transmission of inter-UE coordination feedback

    - If such pre-configuration is not provided it is up to R17 UEs whether to follow R16 or R17 resource selection procedures for sidelink transmissions

## Cast Types for Feedback Request

It was agreed that at least destination UEs of a TB can provide request-based inter-UE coordination feedback and that unicast transmission supports explicit request transmission.

Agreement RAN1#107bis-e - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

In our view, the above agreement can be also extended to at least groupcast transmissions, if TX UE indicates candidate UE(s) to provide request-based inter-UE coordination feedback.

  - For sidelink groupcast transmission, request for scheme-1 inter-UE coordination feedback is supported, if TX UE indicates group member UE(s) for feedback transmission among destination UEs

## Cast Types for Inter-UE Coordination Feedback

From system perspective, inter-UE coordination feedback is beneficial for all UEs independently of sidelink communication cast types. At this stage, RAN1 made the following agreements for cast types of sidelink transmissions carrying inter-UE coordination information or request:

Agreement (RAN1#107bis-e) - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request Working Assumption (RAN1#107bis-e) - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported

### Request-based feedback

Support of request-based inter-UE coordination feedback can be extended to groupcast sidelink transmissions. We are supportive of this extension if groupcast request with indication of the set of transmitters for feedback is supported.

  - Support request-based feedback of preferred and/or non-preferred resource set for groupcast sidelink transmissions

### Condition-based feedback

Preferred resource set for groupcast and broadcast transmissions carrying inter-UE coordination feedback

Indication of preferred resource set can be supported by sidelink transmissions of all cast types. We do not see the specific reason to preclude it for groupcast and broadcast transmissions. If any constraint needs to be introduced, then it is better to impose it on procedure that determines resources for sidelink transmission.

- Conditions for unicast/groupcast/broadcast transmissions carrying inter-UE coordination feedback

There is an FFS on conditions to carry feedback by unicast, groupcast and broadcast sidelink transmissions. We do not see the need to define conditions dependent on cast type of transmission. Additional conditions can be transparent to cast types as discussed in section 2.2.3.

  - Support condition-based feedback of preferred and/or non-preferred resource set for all cast types of sidelink transmissions

The above proposal is consistent with the following agreement made at the RAN1#107bis-e:

Agreement RAN1#107bis-e - For inter-UE coordination information is triggered by a condition other than explicit request reception, - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

## Multiplexing of Inter-UE Coordination Feedback and Data

RAN1 made the following agreements on multiplexing of request and inter-UE coordination information with data:

- Agreement RAN#107bis-e - For inter-UE coordination information transmission in Scheme 1, - Inter-UE coordination information can be multiplexed with other data only if the source/destination ID pair is the same - Retransmission of the TB carrying inter-UE coordination information is supported - For explicit request transmission in Scheme 1, - Explicit request can be multiplexed with other data only if the source/destination ID pair is the same - Retransmission of the TB carrying request is supported

Below we propose to clarify that additional combinations for multiplexing of inter-UE coordination information or request in sidelink transmissions are supported.

  - For scheme 1, clarify that the following multiplexing options are supported

    - Multiplexing of request-based and condition-based feedback and data in the same transmission is supported if the source/destination ID pair is the same

    - Multiplexing of request for feedback with condition-based feedback and data in the same transmission is supported if the source/destination ID pair is the same

    - Multiplexing of request for feedback with request-based feedback and data in the same transmission is supported if the source/destination ID pair is the same

## Prioritization of Inter-UE Coordination Feedback Transmission

RAN1 reached the following agreements on rules to determine priority for inter-UE coordination information and request for inter-UE coordination information:

Agreement RAN1#107bis-e - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B. - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data - For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. - FFS: Otherwise, the priority value is determined by UE-A’s implementation. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

There is one remaining FFS on how to determine priority value for condition-based feedback if pre-configuration is not provided. In our view, the following solutions can be used for condition-based feedback:

Alt.1: Pre-configuration of priority value is always provided

Alt.2: Define default priority value for condition-based feedback

  - Default priority value is used for condition-based inter-UE coordination information, if pre-configuration of priority value is not provided

## Determination of non-preferred resources for Condition 1-B-2

RAN1 made the following agreement on definition of condition 1-B-2 for determination of non-preferred resource set.

Condition 1-B-2

- For Scheme 1 with non-preferred resource set, support the following condition: - Condition 1-B-2: - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

In our view the notion of “half-duplex operation” may need some clarification as potentially companies may have different interpretations how to determine non-preferred resources based on condition 1-B-2. Our interpretation of this condition is that UE-A is expected to check reservations made by other TX UEs and its own reservations (i.e., reservations made by UE-A). Subject to comparison of priorities for transmission and reception, UE-A decides whether there is a half-duplex conflict (i.e., priority of sidelink or uplink transmission is higher than priority of any target sidelink reception(s) in the same slot). In this case, reserved resources/slots with detected half-duplex conflict are marked as non-preferred resources. In addition, UE-A can determine that at a given time it is unaware about half-duplex conflict for certain set of UE-A’s reservation. In our view, such reserved resources/slots should also be indicated as non-preferred resources as potential half-duplex conflict may be avoided by TX-UEs if feedback is provided. Based on discussion, we have the following proposal:

  - Clarify that non-preferred resources based on Condition 1-B-2 are determined based on TX/RX prioritization procedure for sidelink and includes the following resources

    - Slots with reserved resources where half-duplex conflict was detected by UE-A based on priority rules for sidelink reception and UE-A transmission (sidelink or uplink) resulting in half-duplex conflict for reception

    - Slots with reserved resources for transmission and potential half-duplex conflict (i.e., half-duplex conflict is not detected yet by UE-A but may happen due to future reservations made by TX-UEs)

    - If UE-A is aware that reserved resource for transmission will not be used for transmission (e.g., due to ACK feedback) the reserved resource is not indicated as non-preferred

    - Note: pre-selected but not reserved resources are not indicated as non-preferred resources

  - Procedure for determination of non-preferred resources is transparent to cast type of inter-UE coordination feedback and type of inter-UE coordination feedback (i.e., request or condition-based)

## Feedback Cast Type and Its Application to Resource Selection

RAN1 made the following agreement on cast types of sidelink transmissions carrying inter-UE coordination feedback/request.

Agreement (RAN1#107bis-e) - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request Working Assumption (RAN1#107bis-e) - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported

In this section, we discuss relationship of feedback cast type and its application to resource selection for sidelink transmission of certain cast type by UE-B.

The RSRP conditions 1-A-1 and 1-B-1 are applied for generation of the preferred and non-preferred resource set(s) respectively. These conditions are transparent to cast types and therefore, irrespectively of feedback cast type can be considered for resource selection targeting sidelink transmission of any cast type.

The half-duplex conditions 1-A-2 and 1-B-2 are dependent on UE-A resource reservations/potential transmissions. Condition 1-A-2 can be disabled. If it is enabled, UE-A removes from RSRP based preferred resource set resources corresponding to slots where UE-A has made reservations. This is in general beneficial or at least not harmful for resource allocation procedure as preferred resource set is not directly excluded from candidate resources determined by TX-UE.

Condition 1-B-2 cannot be disabled (i.e., there is no agreed pre-configuration to enable/disable it) and is used to identify non-preferred half-duplex resources. Resources reserved for transmission by UE-A (non-preferred half-duplex resources), do not need to be directly excluded by UE-B if UE-A is not a target recipient of UE-B transmission. It means that whether to exclude non-preferred resources associated with condition 1-B-2 depends on whether UE-A is target RX in considered sidelink transmission.

  - Content of request and condition-based inter-UE coordination information is transparent to cast type of inter-UE coordination feedback

  - Procedure for filtering inter-UE coordination information is defined to determine which of the received feedback and its content can be used to form preferred and non-preferred resource sets for resource selection

  - Resource selection procedure is transparent to (1) cast type of inter-UE coordination information and (2) cast type and destination UE(s) of sidelink transmission

  - Non-preferred resources corresponding to Condition 1-B-2 are filtered out by UE-B in case of (1) broadcast sidelink transmissions or (2) when UE-A is not target receiver

  - Inter-UE coordination information (i.e., resources associated with conditions 1-A-1 & 1-A-2, 1-B-1) from target receivers is used to form preferred and non-preferred resource set for resource selection procedure (including resource exclusion and re-evaluation)

## Inter-UE Coordination Feedback Filtering

Any TX UE may receive inter-UE coordination feedback from multiple UEs. The inter-UE coordination feedbacks may be request-based or condition-based feedback. Feedback information is associated with feedback timestamps, resource selection or sensing windows. The feedback may come from UEs having different SL-RSRP ranges and geographical locations.

The first step that needs to be done by TX UE is filtering of the received inter-UE coordination feedback information to determine which one is valid for current iteration of resource selection procedure executed in each slot.

The filtering of inter-UE coordination feedback can be done at least based on the following feedback attributes:

Feedback type (request- or condition- based, preferred- or non-preferred resource set)

    - UE can sort and prioritize request-based feedback over condition-based and keep relevant resource sets separate

Feedback source ID

    - For each transmission a UE can use feedback source ID to determine whether feedback comes from target RX (TRX) or from non-target RX (nTRX). Priority in resource selection can be given to resources indicated by target RXs

Feedback delay / aging time

    - UE can  apply feedback with aging time which does not exceed a pre-configured threshold / amount of time

Overlap ratio of resource selection windows used for generation of feedback and TX sensing results

    - UE can  apply feedback if TX resource selection window overlaps with feedback resource selection window on more than X% of slots/resources

SL-RSRP measurement range (or pathloss range) or geographical distance range from UE providing inter-UE coordination feedback

    - UE can apply feedback if it is provided by UE within the pre-configured SL-RSRP / pathloss range or distance range

Priority level used for feedback generation and feedback transmission

    - UE can apply feedback if it was generated for priority level within pre-configured range of priority levels .

  - The following criteria/conditions can be pre-configured to determine whether given UE can apply received inter-UE coordination feedback for resource exclusion and selection

    - Feedback type (e.g., request- or condition- based, preferred or non-preferred resource sets)

    - Feedback source ID

    - Feedback aging time

    - Overlap ratio of resource selection windows

    - Radio range or geographical distance from the source of inter-UE coordination feedback

    - Priority level used for generation of inter-UE coordination feedback

## Format Translation of Inter-UE Coordination Feedback

- For the set of valid/applicable feedbacks, UE may need to perform translation of inter-UE coordination information to common format, e.g., used by TX UE for resource selection. For instance, translation procedure from request- and condition-based feedback to TX UE candidate resource format may be needed for resource selection procedure, if parameters used for generation of preferred/non-preferred resource set are not aligned.

  - For preferred resource set,

    - Any TX candidate resource of M consecutive subchannels is considered as preferred resource if all M subchannels overlap with one or more resources of N consecutive subchannels from preferred resource set

  - For non-preferred resource set,

    - Any TX candidate resource of M consecutive subchannels that overlaps in at least K subchannels 1 ≤ K ≤ min(N, M) with at least one of the resources of N consecutive subchannels from non-preferred resource set is considered as non-preferred resource

      - Note: K = 1 by default

## Control of Exclusion of Non-preferred Resources

The following agreement was made at RAN1#107-e meeting due to majority support.

Agreement (RAN1#107e) - For Scheme 1 with non-preferred resource set, - Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

According to our understanding of this agreement, the exclusion of non-preferred resource set is performed before checking remaining amount of candidate resources (i.e., comparing the size of candidate resource set with X% Mtotal). According to current resource exclusion procedure condition on X% of candidate resources is expected to be satisfied. However, there are following issues:

- The minimum amount of candidate resources may not be satisfied due to exclusion of non-preferred resources

- The increment of RSRP threshold to achieve X% of candidate resources may result in selection of resources with high RX power

To mitigate above issue the following solutions can be considered:

- Fallback to the set of candidate resources obtained based on TX sensing procedure only, if X% of candidate resources cannot be achieved when non-preferred resources are excluded.

- Apply threshold X% only to the set of candidate resources obtained based on TX sensing procedure (including RSRP increment procedure) and introduce additional threshold Y% on remaining resource set after exclusion of non-preferred resources. UE exclude non-preferred resources until size of the remaining candidate resource set exceeds Y%.

- Exclusion of non-preferred resources from the set of candidate resources obtained based on TX sensing procedure only if this candidate resource set exceeds X% (i.e., before exclusion of non-preferred resources).

Our preference is to apply mechanism of RSRP threshold increase to TX sensing candidate resources only, so that after exclusion of non-preferred resources, the Y% of remaining candidate resources is met (the threshold Y% can be the same as X% or new parameter and value can be pre-configured).

  - Resolve the issue of not satisfying minimum required size of candidate resource set due to exclusion of non-preferred resources

    - Introduce additional threshold Y% to control minimum size of candidate resource set after exclusion of non-preferred resources from the set of candidate resources determined by TX sensing procedure

    - UE excludes non-preferred resources from the set of candidate resources obtained based on TX sensing procedure until size of the remaining candidate resource set exceeds Y%

# Inter-UE Coordination Scheme-2

## Considerations on Periodic Reservation

During the discussion on UE-B behaviour when receiving Scheme 2 conflict indication, it was highlighted that conflicts in periodic reservations may need to be separately considered.

Agreement (RAN1#107bis-e) For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

It should be reminded, that there is already similar issue and procedure for pre-emption since Rel-16:

Agreement from RAN1#102-e: - If periodic reservation is in use by a UE, and if pre-emption is enabled in a resource pool, the UE checks pre-emption for resources provided by MAC layer to L1, according to specified procedures - L1 expects that MAC layer provides resources intended for transmission of one TB, which can fit to resource selection window of current TB of the UE, and for which the relevant priority is available - If a resource is pre-empted, a re-selection for the pre-empted resource is triggered based on the specified step 1 and step 2 procedures, - with details up to UE implementations, including whether/how to set the reservation period in the re-selected resource - FFS in TP phase how/where to capture this in specification - During the pre-emption checking, j is up to Cresel-1

We further discuss two potential cases:

Case A: UE-A cannot differentiate aperiodic and periodic conflicts in Scheme 2 conflict indication

    - In such situation, the reselection by UE-B is only affecting resource for current TB in current reservation period. In Rel-16, similar issue was discussed for pre-emption, and it is up to UE implementation whether/how to set the reservation period in SCI, i.e., whether to terminate the reservation process or continue. This procedure can be reused for Case A, i.e., let the UE decide whether to reselect only current period resources or also resources in upcoming periods for other TBs (by setting the reservation period in SCI)

Case B: UE-A can differentiate aperiodic and periodic reservations conflicts and indicate that in Scheme 2 conflict indication

    - If UE-A can detect conflicts in future period(s), the UE-B may optimize resource reselection taking into account which / how many periods have conflicts. For example, if UE-A indicates conflicts only in the current period, the UE-B can reselect only this period resource and set the period to 0, while resume on the previously reserved resources in the next period.

  - For inter-UE coordination scheme 2, regarding whether/how the conflict in periodic transmission is indicated by UE-A and handled by UE-B, the following alternatives are considered

    - Alt.1: UE-A cannot indicate conflicts in periodic reservations differently from aperiodic reservations

      - UE-B’s re-selection for the indicated resource is triggered based on the agreed procedures, where it up to UE implementations whether/how to set the reservation period in the re-selected resource

    - Alt.2: UE-A can indicate conflicts in periodic reservations differently from aperiodic reservations

      - When periodic conflict is indicated, UE-B’s re-selection for the indicated resource is triggered based on the agreed procedures, and UE-B terminates reservation for future periods by selecting another resource with periodic reservation

      - Otherwise, UE follows the agreed procedures for aperiodic reservation conflict

## Determination of UEs for Inter-UE Coordination Feedback

RAN1 made the following agreement to determine the conditions for UEs to serve as UEs providing inter-UE coordination feedback scheme#2:

- Agreement from RAN1#106e - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A - Working assumption At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s) - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured - FFS: Additional details and condition(s) on UE-A and UE-B

Further, in the last RAN1#107-e meeting the following additional agreement was made

Working Assumption - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B. - FFS whether/how to set additional condition for UE-A to send PSFCH. - Conclude on whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2 at the subsequent meetings

In addition, in RAN1#107bis-e, the issue of UE not capable of Scheme 2 conflict indication was resolved by the following WA:

Working assumption - For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not. - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication

We discuss two cases separately:

Case 1: (Pre-)configuration enables SCI indication of Scheme 2 support by a UE-B

Case 2: (Pre-)configuration does not enable SCI indication of Scheme 2 support by a UE-B

For Case 1, we believe it is crucial that UE-A can only send Scheme 2 PSFCH to a UE which indicated Scheme 2 support in SCI. In this case, all the following conditions should be met to determine that UE-A generates PSFCH to a given UE:

- Priority condition is met, i.e., a UE has a lower priority (higher priority value)

- Timing condition is met, i.e., there is a valid PSFCH resource

- Request condition is met, i.e., SCI of this UE indicates support of Scheme 2 conflict indication (whether UE scheduling a conflicting TB can be UE-B or not)

For Case 2, since there is no SCI indication, only the first two conditions could be checked:

- Priority condition is met, i.e., a UE has a lower priority (higher priority value)

- Timing condition is met, i.e., there is a valid PSFCH resource

Potentially, to control system performance in this case, additional timer condition can be introduced, i.e. when a timer/counter is running, a UE could not generate the conflict indication.

If the UEs exchange Scheme 2 capability by PC5 RRC and if a UE-A knows Scheme 2 support capability of a given conflicting UE, it can be taken into account in Case 2.

Note, that we don’t agree to ignore the priority condition if the PSFCH timing / validity condition is not met. This contradicts the intention of Scheme 2 conflict indication to trigger reselection on a lower priority UE, and may lead to unstable system performance due to unnecessary reselections.

When more than one conflicting TB / UE has equal and higher priority value, then it may be up to UE-A implementation to choose which of them to send the feedback to.

  - For inter-UE coordination scheme 2, the feedback is provided by UE-A(s) to UE-B(s) subject to the following additional conditions

    - Case 1: (Pre-)configuration enables SCI indication of Scheme 2 support by a UE B

      - Priority condition is met, i.e., a UE has a lower priority (higher priority value)

      - Timing condition is met, i.e., there is a valid PSFCH resource

      - Request condition is met, i.e., SCI of this UE indicates support of Scheme 2 conflict indication (whether UE scheduling a conflicting TB can be UE-B or not)

    - Case 2: (Pre-)configuration does not enable SCI indication of Scheme 2 support by a UE B

      - Priority condition is met, i.e., a UE has a lower priority (higher priority value)

      - Timing condition is met, i.e., there is a valid PSFCH resource

      - UE A is unaware that the UE does not support Scheme 2 conflict indication

      - Optionally, additional timer condition can be introduced, i.e., when a timer/counter is running, a UE could not generate the conflict indication.

  - For inter-UE coordination scheme 2, when priorities of conflicting TBs are equal, it is up to UE implementation which of the UEs is UE-B

Regarding the FFS on RSRP threshold configuration, it is suggested that the RSRP threshold is a function of two priorities of overlapping UEs, similar to other RSRP thresholds configurable for sensing operation. We also don’t think further optimization by considering MCS and level of overlap is required.

  - For Condition 2-A-1 of Scheme 2, the RSRP thresholds are function of priority levels of conflicting TBs

## Resource Allocation Behavior by TX UE

Considerations on autonomous conflict detection

In general, the TX UEs can autonomously detect expected/potential resource conflicts in reservations. This is already possible for Rel.16 UEs. However, Rel.16 UEs do not reselect resources unless preemption is enabled by configuration and preemption event is detected. If preemption is disabled UEs are not expected to reselect resources. The reason for such behavior is that frequent reselections of reserved resources diminish the gains from sensing. Therefore, RAN1 needs to identify conditions when resource reselection due to detection of expected/potential resource conflicts in reservations can be beneficial. In addition, mechanisms such as inter-UE coordination feedback to improve detection of such conflicts can be considered.

In our view, resource reselection due to detection of the expected/potential conflict can be considered for limited set of scenarios that may lead to potential losses in the system:

Case 1: Expected/potential conflict is identified in semi-persistent reservations

Case 2: Expected/potential conflict is identified for TX UEs which are close to each other

Case 3: Expected/potential conflict is identified, and pre-emption is enabled

In general, all the above cases can be addressed by TX UEs autonomously in the majority of situations, except the following exceptional situations/conditions (i.e., when expected/potential resource conflict in reservations may not be identified):

Half-duplex on sidelink transmissions in the same slot reserving resources with expected/potential conflict

    - This problem can be potentially solved by inter-UE coordination feedback for resource reselection

Unknown reservation due to PSCCH processing delay

    - Case 1: Reservation is made before identification of expected/potential conflict

      - This problem can be potentially solved by inter-UE coordination feedback

    - Case 2: Reservation is detected after transmission on reserved resources

      - This problem cannot be solved by inter-UE coordination feedback for resource reselection (i.e., only feedback to determine retransmission behavior can be used)

Failure to decode the control channel and thus identify expected/potential resource conflict (e.g., due to being out of communication range or experiencing high interference)

    - This problem can be potentially solved by inter-UE coordination feedback

  - It should be further noted, that for the Option 1 of time resource determination for conflict indication (the slot is determined by the slot of transmission of SCI(s) in conflict), in average ~50% of conflicts could not be indicated due to timing relationship rule, i.e., when the PSFCH transmission occasion has already passed when the conflict is determined at the UE-A. In these scenarios, usage of autonomous conflict detection at the UE-B is crucial, since Scheme 2 conflict indication may not be received.

Some of the above problems can be resolved by inter-UE coordination feedback for resource re-selection only if there is a sufficient time to react on feedback.

Besides the expected/potential resource conflict, the scheme 2 can be also applied for the case of expected/potential half-duplex on overlapping or non-overlapping resources. In our view, these conflicts should be also considered and addressed by scheme 2.

Based on discussion, we have the following proposal to support resource reselection under selected conditions for the case when expected/potential resource conflict is detected by TX UE through PSCCH monitoring or through inter-UE coordination feedback:

  - For the case when Scheme 2 conflict indication is applied together with UE-B autonomous conflict detection:

    - Resource re-selection is triggered when Scheme 2 conflict indication is received OR if the TX UE has detected an expected/potential resource conflict subject to either of the following conditions

      - Expected/potential resource conflict is detected among semi-persistent transmissions (this condition may be subject to (pre-)configuration)

      - Expected/potential half-duplex conflict is detected

      - TX UE has detected that it has the lower or equal transmission priority than other UE with the expected/potential resource conflict

## Resource Determination

It was concluded that there is no consensus for resource conflict type differentiation, thus a single cyclic shift for PSFCH sequence is enough to indicate Scheme 2 conflict. In this case, m_CS fixed to 0 can be adopted, while setting of m_0 can be reused from PSFCH for HARQ-ACK.

  - For Scheme 2 conflict indication

    - the PSFCH sequence frequency shift mCS is fixed to 0

    - m0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3

# Conclusions

In this contribution, we provided our views on enhancements of Mode-2 sidelink communication by means of inter-UE coordination feedback. In summary, we have following proposals:

  - Support pre-configuration of the maximum value of RSRP threshold for generation of preferred resource set, so that the following conditions can be controlled for RSRR thresholds

    - ThrRSRPpreferred ≤ ThrRSRPmax,preferred ≤ ThrRSRPnon-preferred (Option 1)

    - ThrRSRPnon-preferred (Option 2) ≤ ThrRSRPpreferred

  - For non-preferred resource set,

    - Resources generated based on Condition 1-B-1 and 1-B-2 are indicated separately

  - The following alternatives are supported to indicate parameters of the request for inter-UE coordination feedback:

    - Alt.1: Parameters are indicated in SCI Format-2C and MAC CE carrying request for feedback

    - Alt.2: Parameters are indicated in MAC CE only carrying request for feedback

  - The start and end timing locations of resource selection window are indicated as a start slot index associated with (n+T1) and duration (T2-T1) of resource selection window

    - Start slot index is defined relative to the reference slot which is separately indicated in MAC CE or indicated as offset relative to the slot carrying SCI format-2C

  - For condition-based feedback, support pre-configuration of at least the following parameters

    - resource selection window duration (T2-T1)

    - minimum number of candidate single-slot resources for feedback in percent – X

  - Define the following timing relationships for request-based inter-UE coordination resource selection window:

    - (n+T1) ≤ (n'+T1')

    - (n'+T2') ≤ (n+T2)

    - T2,min ≤ (T2'-T1')

    - (n'+T1') – mFBCK ≤ Tproc,3; e.g., Tproc,3 = Tproc,1

    - (n'+T1') – mSensing ≤ Tproc,4; e.g., Tproc,4 = Tproc,0 + Tproc,1

  - UE indicates start/end time of feedback resource selection window (i.e., slots (m+T1') and (m+T2’) in feedback content (MAC CE)

  - Define the following timing relationships for condition-based inter-UE coordination resource selection window:

    - (n'+T1') – mFBCK ≤ Tproc,3 = Tproc,1

    - (n'+T1') – mSensing ≤ Tproc,4 = Tproc,0 + Tproc,1

  - UE indicates start/end time of feedback resource selection window (i.e., slots (n'+T1') and (n'+T2') in feedback content (MAC CE)

  - For feedback generation (construction of preferred/non-preferred resource sets) support the following alternatives

    - Alt.1: Sensing and resource exclusion procedure considers only semi-persistent transmissions for construction of inter-UE coordination feedback

    - Alt.2: Sensing and resource exclusion procedure considers both semi-persistent and dynamic transmissions

  - Introduce pre-configuration of the minimum elapsed time for transmission of standalone request for inter-UE coordination feedback

  - For request-based feedback, introduce pre-configuration of the minimum number of sensed slots in a sensing window as a condition for transmission of inter-UE coordination feedback

  - Introduce pre-configuration of time offset relative to the end (or start) slot of resource selection window to control the maximum delay for request-based feedback transmission

  - Introduce pre-configuration of the minimum number of sensed slots in a sensing window as a condition for transmission of condition-based inter-UE coordination feedback

    - Before generation of inter-UE coordination feedback, UE should check whether number of sensed slots for feedback generation exceeds pre-configured value

  - Clarify that only the preferred resource set can be signaled in the 2nd stage SCI

  - SCI format 2-C contains a field indicating if the fields of SCI format 2-A or 2-B are present

  - The size of SCI for inter-UE coordination feedback or feedback request is always the maximum possible required number of signaling bits given the resource pool configuration

  - SCI format 2-C contains the following fields in addition to the already agreed TRIV, FRIV, Preserve, and start slot for each TRIV:

    - Indication that the SCI contains inter-UE coordination feedback

    - Indication whether the fields of 2-A or 2-B are present

    - Number of present triplets in the range of 1 to 3

    - Starting sub-channel of the first resource in a triplet

  - MAC CE signaling for resource sets with frequency information contains the following fields

    - Resource set type for the case when it is determined by UE-A

    - Number of signaled resources

    - Start slot of the first triplet in either of the following cases:

      - Note: Can be omitted if the start slot is a part of the feedback request

    - Start slot of all other triplets is signaled relative to the last resource in preceding triplet in logical slots

    - TRIV for each triplet

    - FRIV for each triplet (not needed if only slots are signaled)

    - Starting sub-channel of the first resource in a triplet

    - Preserve for each triplet as an index of all Preserve values configured for the resource pool

  - The following additional information is provided as part of MAC-CE inter-UE coordination feedback signaling

    - Start/end time of resource selection window for feedback (e.g., start / end slot index)

      - Applicable for both the preferred and the non-preferred resource set

    - Source ID(s) associated with transmissions on non-preferred resources

  - Introduce maximum preparation time for inter-UE coordination feedback, including MAC-CE, Stage-2 SCI and PSSCH/PSCCH preparation times, which is bounded by T3 (e.g., T3 = Tproc,1)

    - The maximum time for inter-UE coordination feedback preparation is calculated in slots starting from the end slot of sensing window used for feedback generation

  - Apply the following change in the previous agreement:

    - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

  - Define the first(early) in time resource selection procedure to determine resources indicated in SCI format 2-C

  - Support re-evaluation of resources indicated in SCI format 2-C based on the remaining valid set of resources indicated in MAC-CE

  - For inter-UE coordination scheme 1,

    - Feedback is associated with a reference slot (feedback timestamp)

    - Introduce feedback aging time (feedback delay) as an elapsed time since a reference feedback slot

    - Feedback aging time is used to decide on whether feedback can be considered by TX UE in resource allocation procedure for transmission

  - Introduce pre-configuration to configure R17 UEs to apply R17 resource selection procedure for sidelink transmissions and procedures for generation and transmission of inter-UE coordination feedback

    - If such pre-configuration is not provided it is up to R17 UEs whether to follow R16 or R17 resource selection procedures for sidelink transmissions

  - For sidelink groupcast transmission, request for scheme-1 inter-UE coordination feedback is supported, if TX UE indicates group member UE(s) for feedback transmission among destination UEs

  - Support request-based feedback of preferred and/or non-preferred resource set for groupcast sidelink transmissions

  - Support condition-based feedback of preferred and/or non-preferred resource set for all cast types of sidelink transmissions

  - For scheme 1, clarify that the following multiplexing options are supported

    - Multiplexing of request-based and condition-based feedback and data in the same transmission is supported if the source/destination ID pair is the same

    - Multiplexing of request for feedback with condition-based feedback and data in the same transmission is supported if the source/destination ID pair is the same

    - Multiplexing of request for feedback with request-based feedback and data in the same transmission is supported if the source/destination ID pair is the same

  - Default priority value is used for condition-based inter-UE coordination information, if pre-configuration of priority value is not provided

  - Clarify that non-preferred resources based on Condition 1-B-2 are determined based on TX/RX prioritization procedure for sidelink and includes the following resources

    - Slots with reserved resources where half-duplex conflict was detected by UE-A based on priority rules for sidelink reception and UE-A transmission (sidelink or uplink) resulting in half-duplex conflict for reception

    - Slots with reserved resources for transmission and potential half-duplex conflict (i.e., half-duplex conflict is not detected yet by UE-A but may happen due to future reservations made by TX-UEs)

    - If UE-A is aware that reserved resource for transmission will not be used for transmission (e.g., due to ACK feedback) the reserved resource is not indicated as non-preferred

    - Note: pre-selected but not reserved resources are not indicated as non-preferred resources

  - Procedure for determination of non-preferred resources is transparent to cast type of inter-UE coordination feedback and type of inter-UE coordination feedback (i.e., request or condition-based)

  - Content of request and condition-based inter-UE coordination information is transparent to cast type of inter-UE coordination feedback

  - Procedure for filtering inter-UE coordination information is defined to determine which of the received feedback and its content can be used to form preferred and non-preferred resource sets for resource selection

  - Resource selection procedure is transparent to (1) cast type of inter-UE coordination information and (2) cast type and destination UE(s) of sidelink transmission

  - Non-preferred resources corresponding to Condition 1-B-2 are filtered out by UE-B in case of (1) broadcast sidelink transmissions or (2) when UE-A is not target receiver

  - Inter-UE coordination information (i.e., resources associated with conditions 1-A-1 & 1-A-2, 1-B-1) from target receivers is used to form preferred and non-preferred resource set for resource selection procedure (including resource exclusion and re-evaluation)

  - The following criteria/conditions can be pre-configured to determine whether given UE can apply received inter-UE coordination feedback for resource exclusion and selection

    - Feedback type (e.g., request- or condition- based, preferred or non-preferred resource sets)

    - Feedback source ID

    - Feedback aging time

    - Overlap ratio of resource selection windows

    - Radio range or geographical distance from the source of inter-UE coordination feedback

    - Priority level used for generation of inter-UE coordination feedback

  - For preferred resource set,

    - Any TX candidate resource of M consecutive subchannels is considered as preferred resource if all M subchannels overlap with one or more resources of N consecutive subchannels from preferred resource set

  - For non-preferred resource set,

    - Any TX candidate resource of M consecutive subchannels that overlaps in at least K subchannels 1 ≤ K ≤ min(N, M) with at least one of the resources of N consecutive subchannels from non-preferred resource set is considered as non-preferred resource

      - Note: K = 1 by default

  - Resolve the issue of not satisfying minimum required size of candidate resource set due to exclusion of non-preferred resources

    - Introduce additional threshold Y% to control minimum size of candidate resource set after exclusion of non-preferred resources from the set of candidate resources determined by TX sensing procedure

    - UE excludes non-preferred resources from the set of candidate resources obtained based on TX sensing procedure until size of the remaining candidate resource set exceeds Y%

  - For inter-UE coordination scheme 2, regarding whether/how the conflict in periodic transmission is indicated by UE-A and handled by UE-B, the following alternatives are considered

    - Alt.1: UE-A cannot indicate conflicts in periodic reservations differently from aperiodic reservations

      - UE-B’s re-selection for the indicated resource is triggered based on the agreed procedures, where it up to UE implementations whether/how to set the reservation period in the re-selected resource

    - Alt.2: UE-A can indicate conflicts in periodic reservations differently from aperiodic reservations

      - When periodic conflict is indicated, UE-B’s re-selection for the indicated resource is triggered based on the agreed procedures, and UE-B terminates reservation for future periods by selecting another resource with periodic reservation

      - Otherwise, UE follows the agreed procedures for aperiodic reservation conflict

  - For inter-UE coordination scheme 2, the feedback is provided by UE-A(s) to UE-B(s) subject to the following additional conditions

    - Case 1: (Pre-)configuration enables SCI indication of Scheme 2 support by a UE B

      - Priority condition is met, i.e., a UE has a lower priority (higher priority value)

      - Timing condition is met, i.e., there is a valid PSFCH resource

      - Request condition is met, i.e., SCI of this UE indicates support of Scheme 2 conflict indication (whether UE scheduling a conflicting TB can be UE-B or not)

    - Case 2: (Pre-)configuration does not enable SCI indication of Scheme 2 support by a UE B

      - Priority condition is met, i.e., a UE has a lower priority (higher priority value)

      - Timing condition is met, i.e., there is a valid PSFCH resource

      - UE A is unaware that the UE does not support Scheme 2 conflict indication

      - Optionally, additional timer condition can be introduced, i.e., when a timer/counter is running, a UE could not generate the conflict indication.

  - For inter-UE coordination scheme 2, when priorities of conflicting TBs are equal, it is up to UE implementation which of the UEs is UE-B

  - For Condition 2-A-1 of Scheme 2, the RSRP thresholds are function of priority levels of conflicting TBs

  - For the case when Scheme 2 conflict indication is applied together with UE-B autonomous conflict detection:

    - Resource re-selection is triggered when Scheme 2 conflict indication is received OR if the TX UE has detected an expected/potential resource conflict subject to either of the following conditions

      - Expected/potential resource conflict is detected among semi-persistent transmissions (this condition may be subject to (pre-)configuration)

      - Expected/potential half-duplex conflict is detected

      - TX UE has detected that it has the lower or equal transmission priority than other UE with the expected/potential resource conflict

  - For Scheme 2 conflict indication

    - the PSFCH sequence frequency shift mCS is fixed to 0

    - m0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3

References

- RP-202846, “WID revision: NR sidelink enhancement”, LGE

- R1-2201715, “Remaining opens of sidelink resource allocation schemes for UE power saving”, Intel Corporation, February 2022.