3GPP TSG RAN WG1 #108-e			    R1-2201718

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	Reliability for RRC_CONNECTED UEs

Agenda item:	8.12.2

Document for:	Discussion and Decision

# Introduction

In this paper, remaining aspects for reliability of NR multicast and broadcast services (MBS) for RRC_CONECTED UEs are discussed.

# Discussion

In this section, design considerations for reliability mechanisms for NR MBS focused on RRC_CONNECTED UEs is provided.

## 2.1. Improving Reliability based on UL Feedback

### Support of HARQ/ACK Schemes

Since NR MBS supports FDM of MBS and unicast PDSCH, as well as HARQ feedback, the case of out-of-order HARQ feedback needs to be considered i.e., if the last OFDM symbol of the MBS PDSCH occurs before the last OFDM symbol of the unicast PDSCH, is the UE allowed to transmit MBS HARQ feedback after unicast? In some cases, allowing out of order HARQ feedback may be beneficial where unicast and HARQ may have independently configured set of K1 values.

- For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

### HARQ Codebook Design and PUCCH Configuration

#### HARQ Codebook Design:

ACK/NACK-based or NACK-only-based feedback can be dynamically disabled by DCI. However, this may have some non-trivial impact on codebook construction.  For Type 1 HARQ Codebook disabling the HARQ-ACK for a GC-PDSCH does not change the Type 1 HARQ-ACK codebook generation. For the GC-PDSCHs whose ACK/NACK feedback is disabled, padding NACK can be used such that the CB size is same as in the case if HARQ feedback was enabled.

- For Type 1 Codebook if HARQ is disabled by DCI for a GC-PDSCH, Type 1 codebook generation does not change, and padding NACK can be used such that CB size is same as in the case if HARQ-ACK feedback was enabled.

The following agreement was made in RAN1#107bis-e:

Agreement Support multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with the same priority for the following cases: - If the unicast and multicast HARQ-ACK codebooks are both Type-1 - Option1-1: the 1-bit UL DAI with value “1” indicates multiplexing unicast and multicast HARQ-ACK codebooks onto the same PUSCH. - Option1-2: two bits UL DAI separately indicate whether multiplexing unicast and/or multicast HARQ-ACK codebooks onto the same PUSCH - FFS whether it is applied to a single G-RNTI or applied to all configured G-RNTIs. - If both unicast and multicast HARQ-ACK codebooks are Type-2, down-select from: - Option2-1: the 2-bit UL DAI is applicable to both HARQ-ACK codebooks. - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast. - FFS whether a single UL DAI field is applied to all G-RNTIs, or separate UL DAI fields are applied for each configured G-RNTI. - FFS the unicast and multicast HARQ-ACK codebooks are different Types

For the above agreement, for Type 1 codebook, Option 1-1 and for Type 2 codebook Option 2-1 can be supported. Additionally, the case when unicast and multicast HARQ codebooks are of different Types does not need to be supported in Rel-17 MBS at this late stage

- When multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with same priority,

  - For Type 1 codebooks: the 1-bit UL DAI with value “1” indicates multiplexing unicast and multicast HARQ-ACK codebooks onto the same PUSCH

  - For Type 2 codebooks: the 2-bit UL DAI is applicable to both HARQ-ACK codebooks

- Unicast and multicast HARQ-ACK codebooks of different types cannot be multiplexed onto the same PUSCH with same priority

Agreement: When UE supports and is configured with more than one G-RNTI, - for Type-2 codebook construction, DAI is separately counted per G-RNTI. - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value.

In light of the above agreement if different priorities are supported and configured, then the DAI should be counted per G-RNTI per priority index. Similarly, concatenation should also be for each priority index.

- When UE is configured with more than one G-RNTI as well as more than one priority index, for Type 2 codebooks, DAI is separately counter per G-RNTI per priority index and the codebook concatenation is also per priority index. Same principle is also followed for NACK only feedback.

Proposal 3.2.2-1 When more than one NACK-only feedback are available for transmission in the same PUCCH slot, - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - for more than 4 TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK. - The PUCCH resource to be used for the transmission is based on the last DCI scheduling multicast. Proposal 3.2.2-2 (If Proposal 3.2.2-1 is not agreeable) Does not support more than one NACK-only transmission in the same PUCCH

The above proposals are copied from the latest phase of discussion in the FL summary from RAN1#107bis-e. For the first proposal, we don’t think there is any need to support the first sub-bullet separately for the case of up to 4 TBs. The second sub-bullet can be used for combinations of all TBs. Otherwise, we can also support Proposal 3.2.2-2.

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK.

  - The PUCCH resource to be used for the transmission is based on the last DCI scheduling multicast

#### PUCCH Configuration:

Agreement: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit. - This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately. - Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others. - FFS the case of PUCCH for SR.

For the above agreement, there is an unresolved FFS for the case when PUCCH-SR for unicast collides with PUCCH transmissions for NACK-only feedback for the same priority. In this case, the multiplexing of SR and NACK-only can be complicated since NACK-only is transmitted on a shared PUCCH resource and there may be multiple hypotheses for the gNB to check. In this case, PUCCH SR can be prioritized and the NACK only feedback can be dropped.

- When a PUCCH-SR for unicast collides with a PUCCH resource carrying NACK-only feedback for multicast with the same priority, the PUCCH-SR for unicast is prioritized.

Agreement When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, down-select from: - Option 1: the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast. - Option 2: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

- For the above agreement from RAN1#107bis-e, since multicast PDSCH uses dynamic grants, it may be better to configure the PUCCH from multicast list.

- When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast

# Conclusions

In this paper, design options were proposed for reliability improvements for NR MBS. To this end, the following proposals were made:

- For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

- For Type 1 Codebook if HARQ is disabled by DCI for a GC-PDSCH, Type 1 codebook generation does not change, and padding NACK can be used such that CB size is same as in the case if HARQ-ACK feedback was enabled.

- When multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with same priority,

  - For Type 1 codebooks: the 1-bit UL DAI with value “1” indicates multiplexing unicast and multicast HARQ-ACK codebooks onto the same PUSCH

  - For Type 2 codebooks: the 2-bit UL DAI is applicable to both HARQ-ACK codebooks

- Unicast and multicast HARQ-ACK codebooks of different types cannot be multiplexed onto the same PUSCH with same priority

- When UE is configured with more than one G-RNTI as well as more than one priority index, for Type 2 codebooks, DAI is separately counter per G-RNTI per priority index and the codebook concatenation is also per priority index. Same principle is also followed for NACK only feedback

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK. The PUCCH resource to be used for the transmission is based on the last DCI scheduling multicast

- When a PUCCH-SR for unicast collides with a PUCCH resource carrying NACK-only feedback for multicast with the same priority, the PUCCH-SR for unicast is prioritized

- When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast

# References

- RP-193248, New Work Item on NR Support of Multicast and Broadcast Services, Huawei, RAN#86, Sitges, Spain, December 2019.

- 3GPP TS 38.211 v17.0.0, NR Physical Channels and Modulation (Release 17)

- 3GPP TS 38.212 v17.0.0, NR Multiplexing and Channel Coding (Release 17)

- 3GPP TS 38.213 v17.0.0, NR Physical Layer Procedures for Control (Release 17)

- 3GPP TS 38.214 v17.0.0, NR Physical Layer Procedures for Data (Release 17)