3GPP TSG RAN WG1 #108-e	R1-2201724

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.10

Source: 	Intel Corporation

Title: 	UE features for IAB

Document for:	Discussion and decision

# 1   Introduction

In this contribution, based on RAN1#107-e discussion on IAB items [1] and UE features for IAB [2], we further provide our views on the updated RAN1 IAB UE features list.

# 2   New FG to support RRC signaling enhancements for frequency-domain soft availability indication

RAN1#107e has the following agreements regarding RRC signaling enhancements for frequency-domain soft availability indication.

- For DCI format 2_5 indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell:

- AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set group

  - One RB set group consists of one or multiple RB sets

Accordingly, new feature group is needed to support RRC signaling enhancements for frequency-domain RB set group soft availability indication.

Proposal 1: Add the following new FG to to support RRC signaling enhancements for frequency-domain RB set group soft availability indication.

| 31-7 | Frequency-domain soft availability | Support MT reception of frequency-domain soft availability configuration |  | yes | N/A | IAB-node cannot interprete soft availability indication from parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# 3   New FG to support FDM required or not transmission

RAN1#107e has the following agreement regarding indication of whether FDM is required or not.

- Support indication of whether FDM is required or not for an enhanced multiplexing operation mode to donor-CU.

Accordingly, new feature group is needed to support FDM required or not transmission to donor CU.

Proposal 2: Add the following new FG to to support FDM required or not transmission to donor CU.

| 31-8 | FDM required or not indication | Support MT transmission of FDM required or not indication |  | yes | N/A | Donor CU will not be aware of IAB-node’s multiplexing operation mode of FDM required or not | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# 4   New FG to support Timing Case Indication reception

RAN1#107e has the following agreement regarding timing case indication.

- An IAB-MT is provided with a Timing Case Indication via MAC-CE that explicitly indicates a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot).

Accordingly, new feature group is needed to support Timing Case Indication Reception.

Proposal 3: Add the following new FG to to support Timing Case Indication Reception.

| 31-9 | Timing Case Indication | Support MT reception of Timing Case Indication |  | yes | N/A | IAB-node cannot be controlled by parent node regarding timing cases | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# 5   FG 31-4: Case#6 timing alignment

RAN1#107e has the following agreement regarding Case#6 timing alignment signaling.

- A child IAB-MT can inform a parent node via MAC-CE whether Case 6 timing is required for simultaneous operation.

Accordingly, FG 31-4 needs to be extended to support IAB-MT transmission to inform its parent node whether Case#6 timing is required.

Proposal 4: Revise FG 31-4 as follows to support IAB-MT transmission to inform its parent node whether Case#6 timing is required.

| Index | Feature group | Components |
| --- | --- | --- |
| 31-4 | Case 6 timing alignment | - Support Case 6 timing alignment indication reception 2) Support Case 6 required or not information transmission |

xxxx

# 3   Conclusion

In this contribution, we discussed feature groups regarding Rel-17 NR UE features for IAB.  It is summarized by the following proposals.

Proposal 1: Add the following new FG to to support RRC signaling enhancements for frequency-domain RB set group soft availability indication.

| 31-7 | Frequency-domain soft availability | Support MT reception of frequency-domain soft availability configuration |  | yes | N/A | IAB-node cannot interprete soft availability indication from parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Add the following new FG to to support FDM required or not transmission to donor CU.

| 31-8 | FDM required or not indication | Support MT transmission of FDM required or not indication |  | yes | N/A | Donor CU will not be aware of IAB-node’s multiplexing operation mode of FDM required or not | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Add the following new FG to to support Timing Case Indication Reception.

| 31-9 | Timing Case Indication | Support MT reception of Timing Case Indication |  | yes | N/A | IAB-node cannot be controlled by parent node regarding timing cases | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Revise FG 31-4 as follows to support IAB-MT transmission to inform its parent node whether Case#6 timing is required.

| Index | Feature group | Components |
| --- | --- | --- |
| 31-4 | Case 6 timing alignment | - Support Case 6 timing alignment indication reception 2) Support Case 6 required or not information transmission |

# 4   References

[1] Chairman’s Notes, RAN1#107-e, November 2021.

[2] R1-2111811, “Summary of UE features for IAB enhancements”, RAN1#107-e, November 2021.