3GPP TSG-RAN WG1 Meeting #108-e	Tdoc R1-2201734

e-Meeting, 21st February – 3rd March, 2022

Agenda Item:	8.2.1

Source:	Ericsson

Title:	Initial Access Aspects

Document for:	Discussion, Decision

# Introduction

In this contribution, initial access aspects related to DBTW, value range of Q, and CORESET#0 configuration are discussed.

# DBTW applicability to licensed operation

During RAN1#107-bis it was heavily discussed if the DBTW should be applicable to licensed operation and whether or not the UE should assume Q=64 for licensed operation. It remains our strong view that DBTW is irrelevant for licensed operation, and a licensed-only gNB implementation should be required to implement signaling of a parameter (Q) that is only relevant for unlicensed operation. The current spec defines Q only for unlicensed operation, and we believe that it should remain that way. Moreover, it was extensively discussed in the last meeting whether the UE, prior to reading SIB1, requires knowledge of whether the band is licensed or if it is unlicensed operating with Q = 64. During the discussion, it became clear that the UE can make either assumption and the UE behavior would be the same in either case. In other words, for either assumption, the UE would assume only a single Type0-PDCCH monitoring occasion corresponding to the detected SSB index. After reading SIB1, the UE becomes aware of whether the band is licensed or unlicensed, but before that it does not need to know.

Despite this, some companies still wanted assurance that the gNB would do the right thing and set the subCarrierSpacingCommon parameter to 'scs30or120' when operating without shared spectrum channel access. Hence, towards the end of the meeting, the discussion had converged towards TP# 1-9a and TP# 1-9b [4] for TS 38.213. In addition, RAN2 has decided that the spare bit cannot be used to signal Q [3]. Considering this development, TP# 1-9a is not applicable anymore and thus we propose to agree on TP# 1-9b (repeated below).

Agree on TP1 below.

TP1 for 38.213 (TP# 1-9b from [4]):

## 4.1 Cell search *** Unchanged part omitted *** For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index. *** Unchanged part omitted *** For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. For operation without shared spectrum channel access in FR2-2, a UE expects subCarrierSpacingCommon = ‘scs30or120’ from a MIB provided by a SS/PBCH block. *** Unchanged part omitted ***

# 3	Value range of Q

In RAN2#116-bis it was agreed that the spare bit cannot be used to signal Q (see LS reply to RAN1 in [3]). As a consequence, the working assumption and agreement from RAN1#107-e below needs to be revisited.

Agreement - Support DBTW with 480 and 960 kHz SCS. - For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame - Working assumption: Use 2 bits for Q: - SubcarrierSpacingCommon - spare bit in MIB - Send LS to RAN2 for confirming the use of the spare bit in MIB - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used Agreement - Same NSSBQCL values using the same set of signaling bits are supported for 120, 480, and 960 kHz. - Supported values of NSSBQCL: {16, 32, 64} - Note: - For operation with shared spectrum channel access, any supported value of NSSBQCL can be indicated and value < 64 indicates DBTW enabled - UE is expected to be configured with NSSBQCL=64 in licensed operations - For operation with and without shared spectrum channel access, NSSBQCL=64 indicates that the SS/PBCH block index and the candidate SS/PBCH block index have a one-to-one mapping relationship.

In our view the reasonable course of action at this point in time is to limit the number of bits for Q to 1, that is to only use subCarrierSpacingCommon. As Q=64 is agreed to be used to signal that the DBTW is off, the only remaining question is if the second value of Q should be 16 or 32. In our view, operation in FR2-2 will most likely require a large number of beams, thus we prefer Q=32 over Q=16. This also has the consequence that the use of spare needs to be removed from 38.213 and that the RRC parameter list [5] needs to be updated to remove Q=16 from the comment field for discoveryBurstWindowLength-r17.

For operation with shared spectrum channel access, use 1 bit to signal Q (subCarrierSpacingCommon) with the value range for Q = {32, 64}. Update RRC parameter list to reflect this and remove spare from 38.213 according to TP2 (below).

TP2 for 38.213:

*** Unchanged part omitted *** Table 4.1-2: Mapping between the combination of subCarrierSpacingCommon to NSSBQCL for operation with shared spectrum channel access in FR2-2 subCarrierSpacingCommon; spare; NSSBQCL. scs15or60; 0; 16. scs15or60; 1; 32. scs30or120; 0; 64. scs30or120; 1; reserved. *** Unchanged part omitted ***

# 4	Higher Layer Parameters Related to Initial Access

Conclusion:

RRC parameters list to capture changes identified below:

- Add the following note to the comment section of discoveryBurstWindowLength-r17 row in RRC parameter list

  - “Note: This parameter is to be included in both SIB1 and the common serving cell configuration parameters”.

- Support adding “SSB-PositionQCL-Relation-r17” to RRC parameter list as both UE-specific and cell-specific parameter.

- Inform RAN2 (by adding notes to RRC parameter list) that the either the value range of the information element SSB-PositionQCL-Relation-r16 needs to be extended, or a new Rel-17 IE need to be defined to allow configuration of Q = 16, 32, or 64 in SIB2, SIB3, SIB4, MeasObjectNR, and ServingCellConfigCommon for RRM measurements when operating with shared spectrum channel access in FR2-2.

In the previous meeting, the following conclusion was reached regarding the RRC information element SSB-PositionQCL-Relation. This was added to the RRC parameter spreadsheet as agreed (see [8]); however, the moderator marked the row as "unstable" and it was not included in the LS to RAN2. This is because the supported values of Q still need to be revisited due to the RAN2 decision not to use the 'spare' bit in MIB.

Based on our Proposal 2 in the previous section where we propose that a reasonable course of action is to support only Q = {32, 64}, we propose to update the RRC parameter spreadsheet with these values and then mark the row as 'stable' and inform RAN2.

Update the RRC parameter spreadsheet regarding information element SSB-PositionQCL-Relation-r16 to clarify that for operation with shared spectrum channel access in FR2-2, the value range is {32,64}. Mark the row as 'stable' and include it in the RRC parameter LS to RAN2.

# 5 CORESET#0 Configuration

One of the main outstanding issues to finalize for operation in FR2-2 is the CORESET#0 configurations. Since the exact RB offset values that are required depends on what channelization that is decided in RAN4, it has not been possible to resolve the configuration tables. However, a number of working assumptions are agreed:

Working assumption: - For {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz, support multiplexing pattern 1 with 96 PRB CORESET#0, and {1, 2} symbol durations - Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design Working assumption For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets), if additional entries are left, support multiplex pattern 3 with 24 PRB and 2 symbol duration, and multiplexing pattern 3 with 48 PRB and 2 symbol duration. Working assumption For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets) and multiplex pattern 3 with 24 and 48 PRB and 2 symbol duration (with required RB offsets), if additional entries are left, support multiplexing pattern 1 with 96 PRB and 2 symbol duration Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design.

In addition, the following agreements have been made:

Agreement: For CORESET#0 and Type0-PDCCH search space configured in MIB: - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {120, 120} kHz - Support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz. - FFS: Supporting additional values - FFS: Supported values for SSB to CORESET#0 offset RBs Agreement: - For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - Support the following set of parameters. SS/PBCH block and CORESET multiplexing pattern; Number of RBs; Number of Symbols. 1; 24; 2. 1; 48; 1. 1; 48; 2. - Note: the number of entries corresponding the same {mux pattern, number of RB, number of symbol} tuple (listed above) will depend on required RB offsets that needs to be supported based on channel and sync raster design. - FFS: addition other set of parameters

Based on these agreements and working assumptions, 3 new tables for configuration of CORESET0 for the case of 120, 480, and 960 kHz SCS, respectively are captured in 38.213 Section 13 [1] (Tables 13-10A, B, and C). Since RAN4 has not yet concluded on a channelization design including supported ARFCN, GSCN, and spectral utilization values, the "Offset (RBs)" column is still empty. From the latest RAN4 meeting, there is an LS to RAN1 [2] showing that there is an agreement to support a floating channelization design for licensed bands and a fixed channelization design for unlicensed bands. So far, the portion of FR2-2 that has been identified for licensed operation is the frequency range 66 – 71 GHz, hence we assume that a floating channelization design will be supported for this range, and we determine what offsets are required. In addition, we evaluate what offset values are required for an exemplary fixed channelization in the 57 – 66 GHz range for unlicensed bands.

## 5.1 SSB-CORESET0 offsets for a floating channelization

During Rel-15, the channel and sync rasters were designed with the goal of maximizing configuration flexibility to account for different spectrum allocations around the world, while simultaneously avoiding large UE search complexity. In the end, a fine channel raster (ARFCN) granularity of ∆fGlobal=60 kHz and a more coarse sync raster (GSCN) granularity of ∆fSync=17.28 MHz was adopted for FR2-1 to achieve this joint goal. This is referred to as a "floating channelization" design which captures the notion that a channel or channels can be configured very flexibly on the channel raster overtop the coarse sync raster. The following four tables extracted from 38.101-2 [7] show the ARFCN (channel raster) and GSCN (sync raster) values for the floating channelization design for various FR2-1 bands supported in Rel-15.

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279165 |

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n257 | 60 | 2054166 – <1> – 2104165 |
|  | 120 | 2054167 – <2> – 2104165 |
| n258 | 60 | 2016667 – <1> – 2070832 |
|  | 120 | 2016667 – <2> – 2070831 |
| n259 | 60 | 2270832 – <1> – 2337499 |
|  | 120 | 2270832– <2> – 2337499 |
| n260 | 60 | 2229166 – <1> – 2279165 |
|  | 120 | 2229167 – <2> – 2279165 |
| n261 | 60 | 2070833 – <1> – 2084999 |
|  | 120 | 2070833 – <2> – 2084999 |

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 24250 – 100000 MHz | 24250.08 MHz + N * 17.28 MHz, N = 0:4383 | 22256 + N | 22256 – 26639 |

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n257 | 120 kHz | Case D | 22388 - <1> - 22558 |
|  | 240 kHz | Case E | 22390 - <2> - 22556 |
| n258 | 120 kHz | Case D | 22257 - <1> - 22443 |
|  | 240 kHz | Case E | 22258 - <2> - 22442 |
| n259 | 120 kHz | Case D | 23140 – <1> – 23369 |
|  | 240 kHz | Case E | 23142 – <2> – 23368 |
| n260 | 120 kHz | Case D | 22995 - <1> - 23166 |
|  | 240 kHz | Case E | 22996 - <2> - 23164 |
| n261 | 120 kHz | Case D | 22446 - <1> - 22492 |
|  | 240 kHz | Case E | 22446 - <2> - 22490 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. |  |  |  |

In order to define the floating channelization for FR2-2, it is necessary to determine the following for each supported SCS (120, 480, and 960 kHz)

- ARFCN range and step size for 120/480/960 kHz

- GSCN range and step size for 120/480/960 kHz

So far, RAN4 has agreed on a minimum gap of 3 x 17.28 MHz between adjacent GSCN values for FR2-2, meaning the minimum GSCN step size would be 3 for any channelization design. We point out that for the floating channelization design, once the above ranges/step sizes are defined, they are common for all channel bandwidths supported for a given SCS. It is not necessary to come up with a separate design for each channel bandwidth (as required for the fixed channelization designs discussed in RAN4 (see discussion in [6])).

So far, RAN4 has defined the following nominal channel bandwidths:

Table 1: Nominal channel bandwidths agreed in RAN4

In what follows we define the following:

- Nominal channel bandwidth B (in MHz)

  - The supported values of B are shown in Table 1

- Spectral utilization ρSU (in %)

  - The spectral utilization ρSU is defined as the transmission bandwidth configuration (i.e., number of available RBs NRB – see Section 5.3 of [7]) divided by the nominal channel bandwidth B. Hence, given a spectral utilization value for a given subcarrier spacing ∆f, the number of RBs is then determined as follows

NRB=ρSUB12∙∆f

  - In our design, we assume values of NRB based on a conservatieve assumption that the spectral utilization is at most 90%. We find that whatever SSB-CORESET0 offsets are appropriate for these values of NRB will also work for larger values corresponding to higher spectral utilization. This makes the RAN1 offset design less dependent on what value(s) of SU that RAN4 eventually decides.

    - For example, with this conservative assumption, the number of RBs for the minimum channel bandwidth for 120 / 480 / 960 kHz SCS is NRB=62 / 62 / 31 RBs, respectively.

To define the channel and sync rasters we use the following straightforward criteria:

- Channel Raster (Allowed ARFCNs)

  - The ARFCN step size is chosen as 2/8/16 for 120/480/960 kHz. This means that the channel raster granularity is equal to the SCS for each allowed SCS value. Consequently, channel center frequencies are always spaced by an integer number of subcarriers, thus allowing for single FFT operation when multiple channels are aggregated, analogous to Rel-15.

  - The ARFCN range is chosen to ensure that for the smallest/largest ARFCN, a channel of minimum bandwidth does not step outside the 66 – 71 GHz range. In addition, the starting ARFCN must satisfy the constraint that there is an integer number of subcarriers between the channel center frequency (defined by ARFCN) and any SSB center frequency (defined by GSCN). Otherwise kSSB signalled in MIB (offset between SSB and RB boundaries) will be a non-integer value which does not work.

- Sync Raster (Allowed GSCNs)

  - The GSCN step size is set to 3 x 17.28 MHz for operation with 120 kHz SCS and to 12 x 17.28 MHz for operation with 480 kHz SCS. The GSCN step size for 960 kHz is chosen to be 6 x 17.28 MHz. Note that one might expect a value of 12 considering that the minimum channel bandwidth for 960 kHz (400 MHz) is the same as for 480 kHz. However, due to the fact the SSB bandwidth is twice as large for 960 kHz compared to 480 kHz and that the minimum bandwidth does not scale, it is necessary to compensate by using a GSCN step size smaller than 12 so that the SSB will fit within the transmission bandwidth configuration for at least one GSCN for every allowed ARFCN.

  - The GSCN range is chosen to additionally ensure that for the smallest/largest GSCN, the SS/PBCH block does not step outside the transmission bandwidth configuration (NRB RBs) of the channel of minimum bandwidth corresponding to the smallest/largest allowed ARFCN determined above.

For 120 and 480 kHz SCS, Figure 1 illustrates the transmission bandwidth configuration for a channel of minimum bandwidth centered at an arbitrary configured ARFCN overtop of the sync raster. Assuming NRB=62 (see above discussion on spectral utilization) a sync raster granularity (GSCN step size) of every 3rd/12th GSCN will ensure that for every allowed ARFCN a suitable GSCN can be found.

Figure 1: Transmission bandwidth configuration (NRB = 62 RBs) corresponding to minimum channel bandwidth B = 100 / 400 MHz for 120 / 480 kHz SCS assuming spectral utilization at most 90%. The illustration shows the proposed sync raster (every 3rd/12th GSCN) in relation to an arbitrary configured ARFCN (channel center frequency).

The complete design showing both ARFCN and GSCN ranges and step sizes for all 3 supported subcarrier spacings is shown in Table 2.

Table 2: Floating channelization design for the 66 – 71 GHz band (licensed portion of 57 – 71 GHz band)

| SSB SCS | ARFCN Range and <Step Size> | GSCN Range and <Step Size> | Number of Sync Raster Points |
| --- | --- | --- | --- |
| 120 kHz | 2713333 <2> 2794999 (66050.04 - 70950.00 MHz) | 24674 <3> 24959 (66033.12 - 70957.92 MHz) | 96 |
| 480 kHz | 2715835 <8> 2792499 (66200.16 - 70800.00 MHz) | 24677 <12> 24953 (66084.96 - 70854.24 MHz) | 24 |
| 960 kHz | 2715835 <16> 2792491 (66200.16 - 70799.52 MHz) | 24680 <6> 24950 (66136.80 – 70802.40 MHz) | 46 |

The needed SSB-CORESET0 offsets for the floating channelization design are determined by considering the minimum channel bandwidth for each SCS, i.e., 100/400/400 for 120/480/960 kHz SCS, respectively. As discussed previously, the transmission bandwidth configuration (NRB RBs) corresponding to the minimum channel bandwidth depends on the spectral utilization ρSU, and we assume a conservatieve value of at most 90%. This translates to NRB=62 / 62 / 31 RBs.

Here we start by considering the maximum CORESET0 size for each SCS that will fit within the minimum channel bandwidth assuming SSB and CORESET0 multiplexing Pattern 1, i.e., 48 RB CORESET0 for 120/480 kHz SCS and 24 RB CORESET0 for 960 kHz SCS. Figure 2 illustrates the required offsets for the floating channelization design. In this diagram, the different rows (corresponding to different offsets) represent different example ARFCN/GSCN combinations which determine the relative location of the SSB within the transmission bandwidth configuration (NRB RBs).

Since the sync raster granularity is quite coarse, especially for 120 / 480 kHz SCS (GSCN step size 3 / 12), it is necessary to define offsets such that CORESET0 is shifted both to the right (Offset = 0) and to the left (Offset 28) with respect to the SSB as well as roughly centered on the SSB (Offset 14). For 960 kHz SCS, where the sync raster granularity is not as coarse, defining offsets for right and left shifts is sufficient. We observe that offset value 14 for 48 RB CORESET0 and offset values 0 and 4 for 24 RB CORESET0 are the same as for FR2-1 in Rel-15. Offset values 0 and 28 are new. With this design, CORESET0 will fit with the NRB RBs for at least one of the offset values for every applicable ARFCN.

Figure 2: SSB-CORESET0 offsets for floating channelization design assuming at most 90% spectral utilization and the minimum channel bandwidth for each SCS.

We performed an exhaustive check that the SSB-CORESET offsets shown in Figure 2 work for the floating channelization design captured in Table 2. The exhaustive check was performed as follows assuming the transmission bandwidth configuration NRB=62 / 62 / 31 RBs for 120 / 480 / 960 kHz SCS, respectively (corresponding to at most 90% SU).

- For each SCS and the corresponding NRB

  - For each allowed ARFCN from the defined channel raster

    - Determine the set of allowed GSCNs from the defined sync raster that occur within the NRB RBs

    - For each GSCN in the set

      - Check if the SSB centered at this GSCN fits within the NRB RBs

      - For each of the defined SSB-CORESET0 offsets

        - Check if CORSET0 fits within the NRB RBs

      - End SSB-CORESET0 offset loop

    - End GSCN loop

  - End ARFCN loop

- End SCS loop

Through this exhaustive check, we verified that for all ARFCNs there is at least one GSCN point within the transmission bandwidth configuration (NRB RBs) for which CORESET0 fits for at least one of the defined SSB-CORESET0 offsets. Hence, all ARFCNs in the channel raster are usable and the UE can determine the position of CORESET0 based on the SSB-CORESET0 offset and indicated kSSB in MIB.

Based on the above design methodologoy, we have determined what SSB-CORESET0 offset values that are needed for all of the CORESET0 configurations that are contained in the agreements and working assumptions achieved so far. We reiterate that if RAN4 agrees on spectral utilization values greater than the conservatieve value (90%) we used in our analysis, the offset values we determine here are still valid.

Support the SSB-CORESET0 offset values shown in Tables 13-10A, B, and C below for 120, 480, and 960 kHz, respectively.

Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 48 | 1 | 0 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 1 | 28 |
| 4 | 1 | 48 | 2 | 0 |
| 5 | 1 | 48 | 2 | 14 |
| 6 | 1 | 48 | 2 | 28 |
| 7 | 1 | 96 | 1 | 0 |
| 8 | 1 | 96 | 2 | 0 |
| 9 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 10 | 3 | 24 | 2 | 24 |
| 11 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 12 | 3 | 48 | 2 | 48 |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |

Table 13-10B: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 48 | 1 | 0 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 1 | 28 |
| 4 | 1 | 48 | 2 | 0 |
| 5 | 1 | 48 | 2 | 14 |
| 6 | 1 | 48 | 2 | 28 |
| 7 | 1 | 96 | 2 | 0 |
| 8 | 1 | 96 | 2 | 56 |
| 9 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 10 | 3 | 24 | 2 | 24 |
| 11 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 12 | 3 | 48 | 2 | 48 |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |

Table 13-10C: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {960, 960} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 2 | 0 |
| 4 | 1 | 96 | 2 | 0 |
| 5 | 1 | 96 | 2 | 56 |
| 6 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 7 | 3 | 24 | 2 | 24 |
| 8 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 9 | 3 | 48 | 2 | 48 |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |

## 5.2 SSB-CORESET0 offsets for a fixed channelization

Secondly, we analyze what SSB-CORESET offset values are needed assuming a fixed channelization design for unlicensed bands. Based on the draft LS from RAN4 [2], the following is agreed in RAN4:

Agreement: Consider the different channelization for licensed band(s) and unlicensed band(s) - Fixed sync raster for unlicensed bands - Fixed scheme should not be constrained by IEEE channel raster - Send LS to RAN1 to make sure that RAN1 accommodates the solution already now for both fixed and floating sync raster. - For the contiguous carrier aggregation, the channel spacing of adjacent channels should be multiple of the larger SCS, i.e., 960KHz, used by two channels/CCs - Floating sync raster for licensed bands - Stick to the agreement last meeting for 3 x 17.28MHz as the minimum granularity. - Refer to gap between adjacent GSCN values is not smaller than 3 x 17.28MHz - FFS: Unlicensed bands tries to use the sub-set of sync raster for licensed bands

A fixed channelization design is, just as the floating channelization, defined by a set of ARCNs and GSCNs. The difference is that a fixed set of non-overlapping channels for each channel bandwidth is defined, and most likely one GSCN value is selected per channel. For operation with 120 kHz SCS, 100 MHz and 400 MHz channels are defined, see Table 1. For operation with 480 kHz SCS, 400 MHz, 800 MHz, and 1600 MHz channels are defined and for operation with 960 kHz SCS, 400 MHz, 800 MHz, 1600 MHz, and 2000 MHz channels are defined.

Let’s focus on the minimum channel bandwidths for operation with 120 and 480 kHz subcarrier spacing to start, that is 100 and 400 MHz channels respectively. A sketch of an exemplary fixed channelization showing only these channels is shown in Figure 3 for the frequency range 57 – 66 GHz. We consider such an exemplary design here simply to make the point that the offsets needed for a fixed design are simply a subset of those required for the floating design. It is not necessary to define two sets of different offsets.

Figure 3 Sketch of a fixed channelization showing 100 and 400 MHz channels for 120 and 480 kHz subcarrier spacing in the 57 – 66 GHz spectrum range.

A more detailed picture of 100 MHz channels for parts of the spectrum range is shown in Figure 4.

.

Figure 4 Detailed picture of an exemplary fixed channelization of 100 MHz channels for 120 kHz subcarrier spacing.

For the minimum bandwidth channels the ARFCNs can be defined using a formula specifying the spacing between adjacent channels in terms of number of subcarriers and a starting ARFCN. The number of subcarriers for channel separation need to be chosen to comply with the RAN4 agreement highlighted above that the channel spacing of adjacent channels should be multiple of 960 kHz subcarriers. Wider channels can be constructed in a similar manner and will generate a separate set of ARFCNs since the channel center frequencies will be different. Table 3 shows an example of how a fixed channelization can be designed in terms of channel separation. Since channel spacing is defined in integer multiples of 960 kHz subcarriers, the channel bandwidths will not be exactly the nominal values of 100/400/800/1600/2000 MHz wide; they will be slightly less, but still within +/- 960 kHz of the nominal.

Table 3 Channel spacing (# of subcarriers) for each bandwidth and subcarrier spacing for the exemplary fixed design in the 57 – 66 GHz spectrum range. These values satisfy the constraint that the channel spacing should be an integer number of 960 kHz subcarriers.

| Subcarrier spacing [kHz] | Nominal channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 100 | 400 | 800 | 1600 | 2000 |
| 120 | 832 | 3328 | - | - | - |
| 480 | - | 832 | 1666 | 3332 | - |
| 960 | - | 416 | 833 | 1666 | 2083 |

For such a channelization design, the applicable ARFCNs per channel bandwidth can be defined as shown in Table 4. In this table, the <step size> is determined as N * (Δf / ΔfGlobal) where N is the channel spacing (# of subcarriers) from Table 3, Δf is the subcarrier spacing (kHz), and ΔfGlobal = 60 kHz is the global channel raster granularity defined in 38.101-2. The start/end ARFCNs are determined using similar principles as described in the previous section for the floating channelization design.

Table 4 Applicable ARFCN values for a fixed channelization design for the 57 – 66 GHz band

| Subcarrier spacing [kHz] | ARFCN Range and <Step Size> per nominal channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| 120 | 2563339 <1664> 2711435 (57050.40 – 65936.16 MHz) | 2565835 <6656> 2705611 (57200.16 – 65586.72 MHz) | - | - | - |
| 480 | - | 2565835 <6656> 2705611 (57200.16 – 65586.72 MHz) | 2569179 <13328> 2702459 (57400.80 – 65397.60 MHz) | 2575835 <26656> 2682459 (57800.16 – 64197.60 MHz) | - |
| 960 | - | 2565835 <6656> 2705611 (57200.16 – 65586.72 MHz) | 2569179 <13328> 2702459 (57400.80 – 65397.60 MHz) | 2575835 <26656> 2682459 (57800.16 – 64197.60 MHz) | 2579179 <33328> 2679163 (5800.80 – 63999.84 MHz) |

For a fixed design, most likely there will be one GSCN selected per minimum bandwidth channel. Since each GSCN will be individually selected (hand-picked), it can be selected in many different ways, see Figure 4. For example, the set of GSCN points can be selected so that the RB offset values derived for the floating channelization design can be used. One example of a GSCN set for the ARFCN values in Table 4 are shown in Table 5, where the GSCN was selected to be the lowest GSCN value such that the SSB fits within the minimum bandwidth channel.

Table 5 Applicable GSCN values for a fixed channelization design for the 57 – 66 GHz band

| Subcarrier spacing [kHz] | Allowed GSCN raster points | Number of Sync Raster Points |
| --- | --- | --- |
| 120 | 24153, 24159, 24164, 24170, 24176, 24182, 24188, 24193, 24199, 24205, 24211, 24216, 24222, 24228, 24234, 24240, 24245, 24251, 24257, 24263, 24268, 24274, 24280, 24286, 24292, 24297, 24303, 24309, 24315, 24320, 24326, 24332, 24338, 24344, 24349, 24355, 24361, 24367, 24372, 24378, 24384, 24390, 24396, 24401, 24407, 24413, 24419, 24424, 24430, 24436, 24442, 24448, 24453, 24459, 24465, 24471, 24476, 24482, 24488, 24494, 24500, 24505, 24511, 24517, 24523, 24528, 24534, 24540, 24546, 24552, 24557, 24563, 24569, 24575, 24580, 24586, 24592, 24598, 24604, 24609, 24615, 24621, 24627, 24632, 24638, 24644, 24650, 24656, 24661, 24667 | 90 |
| 480 | 24156, 24179, 24203, 24226, 24249, 24272, 24295, 24318, 24341, 24364, 24387, 24411, 24434, 24457, 24480, 24503, 24526, 24549, 24572, 24595, 24619, 24642 | 22 |
| 960 | 24162, 24185, 24209, 24232, 24255, 24278, 24301, 24324, 24347, 24370, 24393, 24417, 24440, 24463, 24486, 24509, 24532, 24555, 24578, 24601, 24625, 24648 | 22 |

A similar check as described for the floating design has been done for this fixed design assuming at most 90% SU to verify that no additional RB offsets are needed, that the CORESET0 fits in the transmission bandwidth configuration (NRB RBs) and that the guard band requirements are fulfilled.

For the 57 – 66 GHz band, the SSB-CORESET0 offsets proposed in Section 5.1 for the floating channelization design are also valid for the fixed design, and no additional offset values need to be defined.

# Conclusion

In this paper we observed the following:

Observation 1	For the 57 – 66 GHz band, the SSB-CORESET0 offsets proposed in Section 5.1 for the floating channelization design are also valid for the fixed design, and no additional offset values need to be defined.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Agree on TP1 below.

Proposal 2	For operation with shared spectrum channel access, use 1 bit to signal Q (subCarrierSpacingCommon) with the value range for Q = {32, 64}. Update RRC parameter list to reflect this and remove spare from 38.213 according to TP2 (below).

Proposal 3	Update the RRC parameter spreadsheet regarding information element SSB-PositionQCL-Relation-r16 to clarify that for operation with shared spectrum channel access in FR2-2, the value range is {32,64}. Mark the row as 'stable' and include it in the RRC parameter LS to RAN2.

Proposal 4	Support the SSB-CORESET0 offset values shown in Tables 13-10A, B, and C below for 120, 480, and 960 kHz, respectively.

# References

3GPP TS 38.213, "Physical layer procedures for control," v17.0.0, December 2021.

R1-2200894 (R4-2200081), “LS for the channelization for up to 71 GHz”, RAN1#108-e, February 2022.

R1-2200874 (R2-2201720), "Reply LS on initial access for 60 GHz," RAN1#108-e, February 2022.

R1-2200690, “Summary #2 of email discussion on initial access aspect of NR extension up to 71 GHz”, Moderator (Intel), RAN1#107-bis-e, January 2022.

R1-2200699, “Consolidated higher layers parameter list for Rel-17”, Moderator (Ericsson), RAN1#107-bis-e, January 2022.

R4-2205022, "52.6-71 GHz System Parameters," Ericsson, RAN4#102-e, February 2022.

3GPP TS 38.101-2, "User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone," v17.1.0, March 2021.

R1-2200702, "Collection of updated higher layer parameter list," Moderator (Ericsson), RAN1#107bis-e, January 2022.