3GPP TSG-RAN WG1 Meeting #108-e	Tdoc R1-2201472

e-Meeting, 21st February – 3rd March, 2022

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on LS from RAN3 on NR-U channel information and procedures

Document for:	Discussion, Decision

# 1	Introduction

This contribution contains a discussion related to the LS from RAN3 on NR-U channel information and procedures.

# 2	Background

In the context of SON optimization for NR-U, RAN3 has agreed to support NR-U in Mobility Load Balancing and identified that it is beneficial to exchange load metrics on a per cell and per NR-U channel granularity between base stations. In order to define the load metrics in network interfaces and keep in line with the concept in RAN1 and RAN2, RAN3 would like to ask the following questions to RAN1 and RAN2:

RAN3 has sent an LS to both RAN1 and RAN2 on NR-U channel information and procedures [1]. So far RAN2 has provided a response to RAN3 in which RAN1 is copied [2]. The LS from RAN3 provides the following background.

# 3	Discussion on Questions from RAN3

## 3.1	Question 1

Q1: How should an NR-U channel be represented?

A possible description identified by RAN3 for the NR-U channel representation is as following. In this representation an NR-U channel can be recognized via its centre frequency and bandwidth.

| NR-U Channel List |  | 0..1 |  |
| --- | --- | --- | --- |
| >NR-U Channel Item |  | 1..<maxnoofNR-UChannels> |  |
| >>Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannels, …) |
| >>NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) |
| >>Bandwidth | M |  | ENUMERATED (10Mhz, 20Mhz. …) |

RAN3 would like to check with RAN1 and RAN2 if the above information is enough to identify a NR-U channel or if more details are needed.

RAN3 asks the following question:

Answer from RAN2:

From RAN2 perspective, at least 'NR ARFCN' and 'Bandwidth' are defined, and 'Channel ID' does not exist in RAN2 specifications.

RAN2 has provided the following response:

First of all, we agree with the response from RAN2, and here we provide additional background on the answer to Q1. In Rel-16, RAN4 defined two bands for operation with shared spectrum channel access (aka unlicensed bands), both in FR1. The specified bands are Band n46 which spans the frequency range 5150 – 5925 MHz and Band n96 which spans the frequency range 5925 – 7125 MHz (see [4] for details). In Rel-17, RAN4 is currently discussing definition of a new unlicensed band, tentatively named Band n263 which spans the frequency range 57 – 71 GHz in FR2. However, the details of the band definition are not yet complete.

For Band n46 and Band n96, the following fixed set of ARFCNs are defined for each supported carrier bandwidth [4]. Note that RAN4 refers to a carrier as "channel." For operation with shared spectrum channel access, RAN1 has a different definition of "channel," as described below.

Table 5.4.2.3-2: Allowed NREF (NR-ARFCN) for operation in Band n46

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 10 MHz | 782000, 788668 |
| 20 MHz | 744000, 745332, 746668, 748000, 749332, 750668, 752000, 753332, 754668, 756000, 765332, 766668, 768000, 769332, 770668, 772000, 773332, 774668, 776000, 777332, 778668, 780000, 781332, 783000, 784332, 785668, 787000, 788332, 789668, 791000, 792332, 793668 |
| 40 MHz | 744668, 746000, 748668, 751332, 754000, 755332, 766000, 767332, 770000, 772668, 775332, 778000, 780668, 783668, 786332, 787668, 790332, 793000 |
| 60 MHz | 745332, 746668, 748000, 752000, 753332, 754668, 766668, 768000, 769332, 773332, 774668, 778668, 780000, 784332, 785668, 791000, 792332 |
| 80 MHz | 746000, 747332, 752668, 754000, 767332, 768668, 774000, 779332, 785000, 791668 |
| NOTE: 10 MHz channel bandwidth shall only apply in certain regions where the absence of non 3GPP technologies can be guaranteed on a long-term basis in this version of specification. |  |

Table 5.4.2.3-3: Allowed NREF (NR-ARFCN) for operation in Band n96

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668, 829000, 830332, 831668, 833000, 834332, 835668, 837000, 838332, 839668, 841000, 842332, 843668, 845000, 846332, 847668, 849000, 850332, 851668, 853000, 854332, 855668, 857000, 858332, 859668, 861000, 862332, 863668, 865000, 866332, 867668, 869000, 870332, 871668, 873000, 874332 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000, 829668, 832332, 835000, 837668, 840332, 843000, 845668, 848332, 851000, 853668, 856332, 859000, 861668, 864332, 867000, 869668, 872332 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332, 830332, 831668, 835668, 837000, 841000, 842332, 846332, 847668, 851668, 853000, 857000, 858332, 862332, 863668, 867668, 869000, 873000 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668, 831000, 836332, 841668, 847000, 852332, 857668, 863000, 868332 |

In these bands, the supported carrier bandwidths are 20, 40, 60, or 80 MHz. We note that 10 MHz is also supported; however, only in one specific geographic location of the world (India), and only if the absence of non-3GPP technologies (e.g., IEEE 802.11) can be guaranteed on a long-term basis. The ARFCN and carrier bandwidth are provided to the UE either via broadcast or dedicated signaling for standalone and non-standalone operation, respectively. The carrier bandwidth is provided as a number of RBs (NRB), and the following values are supported [4]. Note that the indicated ARFCN for a carrier maps to the center subcarrier of the NRB RBs.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

From a RAN1 point of view, in Clause 4 of TS 37.213 [5], a "channel" is defined as "a carrier or a part of a carrier consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum." The contiguous set of RBs over which a channel access procedure is performed is called an "RB set" which spans approximately 20 MHz and is defined precisely in Clause 7 of TS 38.214  [6]. Hence a 20, 40, 60, or 80 MHz carrier consists of 1, 2, 3, or 4 such RB sets. In the RAN2 reply LS, it is indicated that "Channel ID" does not exist in RAN2 specifications which is correct. However, in Clause 7 of TS 38.214 RAN1 does define an RB set index which can take values s = 0, 1, 2, or 3, and RAN2 specifications do refer to RB sets.

Based on the above discussion, we propose the following reply to RAN3 (see draft LS reply in [3]).

Provide answer to RAN3 with respect to Q1 that the frequency location of a channel in a band defined for operation with shared spectrum channel access (aka unlicensed operation) is determined by the ARFCN of the carrier and the carrier bandwidth, and that in RAN2 specifications, "Channel ID," does not exist. In Rel-16 for FR1, the carrier bandwidths 20, 40, 60, and 80 MHz are supported in Bands n46 and n96, and are divided into 1, 2, 3, and 4 sets of contiguous RBs, respectively, each referred to an "RB set." RAN1 has specified in 37.213 that a "channel" corresponds to a carrier or part of a carrier consisting of a set of contiguous RBs over which a channel access procedure is performed. Hence, from a RAN1 perspective an RB set corresponds to a "channel," and an index is associated with each RB set with value in the range 0 .. 3.

## 3.2	Question 2

In order to define the channel occupancy time of an NG-RAN node, RAN3 also discussed whether channel sensing is carried out even when no data needs to be transmitted or whether channel sensing is performed only when the NG-RAN node needs to exchange traffic over the NR-U channel.

Q2:  According to current specifications, is an NG-RAN node supposed to sense the NR-U channel even when no data needs to be transmitted or is channel sensing performed only when the NG-RAN node needs to exchange traffic over the NR-U channel?

RAN3 asks the following question:

Answer from RAN2:

According to TS 38.300 Section 5.6.1, the NG-RAN node may apply LBT in order to transmit packets to UEs over the air interface. It is not specified in 3GPP specifications whether the NG-RAN node can sense the NR-U channel even when no data are available for transmission.

RAN2 has provided the following response:

We agree with the response from RAN2 that the channel access procedures may be performed when there is a need to access the NR-U channel for packet transmissions over the NR-U channel. In other words, before performing a packet transmission on an NR-U channel, access to the NR-U channel should be obtained by successfully completing the corresponding channel access procedures that are specified in TS 37.213. However, a node (NG-RAN or UE) is neither required to continuously sense an NR-U channel even no data is available for transmission, nor is a node (NG-RAN or UE) required to perform the actual packet transmission even after the corresponding channel access procedures are successfully completed.

Based on the above discussion, we propose the following reply to RAN3 (see draft LS reply in [3]).

Provide answer to RAN3 with respect to Q2 that the channel access procedures specified in TS 37.213 may be performed when there is a need to access an NR-U channel for packet transmissions over the NR-U channel. In other words, before performing a packet transmission on an NR-U channel, access to the NR-U channel should be obtained by successfully completing the corresponding channel access procedures that are specified in TS 37.213. However, a node (NG-RAN or UE) is neither required to continuously sense an NR-U channel even no data is available for transmission, nor is a node (NG-RAN or UE) required to perform the actual packet transmission even after the corresponding channel access procedures are successfully completed.

## 3.3	Questions 3/4

Furthermore, RAN3 discussed exchanging Energy Detection (ED) threshold between RAN nodes and have two more questions:

Q3: How is the ED threshold configured in RAN node?

Q4: What is the ED threshold granularity (per channel, per cell, per UE…)?

RAN3 asks the following questions:

Answer from RAN2 [on Q3]:

Regarding how to determine the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213 Section 4.1.5). Hence, RAN1 confirmation would be required.

In RAN2 point of view, the energy detection threshold is configured to the UE either as an offset to the default maximum energy detection threshold value, or as an absolute configurable maximum energy detection threshold value.

Answer from RAN2 [on Q4]:

Regarding how to determine the ED threshold in RAN node, RAN2 understands that it is specified in RAN1 specification (i.e. TS 37.213 Section 4.1.5). Hence, RAN1 confirmation would be required.

In RAN2 point of view, ED threshold for the specific UE can be configured under ServingCellConfig, which usually contains the configuration per cell.

RAN2 has provided the following responses:

Regarding Q3, we agree with the response from RAN2 that the determination of the ED threshold for NG-RAN nodes to perform DL transmissions for n46 and n96 NR-U channels is specified in Clause 4.1.5 of TS 37.213. It is important to note that there are few conditions as described in Clause 4.1.5 of TS 37.213 that determine how the ED threshold for a node is obtained. One of the conditions is whether absence of any other technology sharing the NR-U channel can be guaranteed. If this condition is not fulfilled, for example due to coexistence with Wi-Fi access nodes on the NR-U channel, one can derive the ED threshold for a RAN-node as -72 dBm for 20 MHz channel based on values of few parameters as described in this clause.

We would like also to clarify that the ED threshold by itself is not specified to be “exchanged” between RAN-nodes as RAN3 expressed in the questions. Whether the contributing parameters in determining the corresponding ED threshold are exchanged between RAN nodes is not within the RAN1 expertise.

Regarding Q4, it is not clear the intention of Q4. From the question itself and the answer provided by RAN2, we suspect that perhaps the existing RRC parameters maxEnergyDetectionThreshold-r16 or energyDetectionThresholdOffset-r16 have contributed to this question.

If that is the case, we would like to clarify that these parameters are configured to the UE to determine the ED threshold to be used for sensing to perform UL transmissions and are not applicable to the NG-RAN for determining the ED threshold to perform sensing for DL transmissions. The corresponding configurations are UE specific and configured per serving cell. How these parameters are used to determine the ED threshold at UE for sensing to perform UL transmissions is specified in Clause 4.2.3 of TS 37.213.

Otherwise, irrespective of the node being NG-RAN or UE that intends to access the NR-U channel, the corresponding ED threshold is determined for the RBs of the channel that the node intends to perform sensing as described in the answer to Q1.

Based on the above discussion, we propose the following reply to RAN3 (see draft LS reply in [3]).

Provide answer to RAN3 with respect to Q3 that the determination of the ED threshold for NG-RAN nodes to perform DL transmissions for n46 and n96 NR-U channels is specified in Clause 4.1.5 of TS 37.213 where the ED threshold is associated with a set of contiguous RBs of the “channel” that an NG-RAN node intends to perform sensing as described in the answer to Q1. It is important to note that there are few conditions as described in Clause 4.1.5 of TS 37.213 that determine how the ED threshold for a node is obtained. The ED threshold by itself is not specified to be “exchanged” between RAN-nodes as RAN3 expressed in the question. Whether the contributing parameters in determining the corresponding ED threshold are exchanged between RAN nodes is not within the RAN1 expertise.

Provide answer to RAN3 with respect to Q4 that RAN1 suspects that perhaps the existing RRC parameters maxEnergyDetectionThreshold-r16 or energyDetectionThresholdOffset-r16 have contributed to this question. If that is the case, we would like to clarify that these parameters are configured to the UE to determine the ED threshold to perform sensing for UL transmissions and are not applicable to the ED threshold at NG-RAN to perform sensing for DL transmissions. The corresponding configurations are UE specific and configured per serving cell. How these parameters are used to determine the ED threshold at UE for sensing to perform UL transmissions is specified in Clause 4.2.3 of TS 37.213. We would like to clarify that irrespective of the node being NG-RAN or UE, the corresponding ED threshold is determined for the set of contiguous RBs of the “channel” that the node intends to perform sensing as described in the answer to Q1.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Provide answer to RAN3 with respect to Q1 that the frequency location of a channel in a band defined for operation with shared spectrum channel access (aka unlicensed operation) is determined by the ARFCN of the carrier and the carrier bandwidth, and that in RAN2 specifications, "Channel ID," does not exist. In Rel-16 for FR1, the carrier bandwidths 20, 40, 60, and 80 MHz are supported in Bands n46 and n96, and are divided into 1, 2, 3, and 4 sets of contiguous RBs, respectively, each referred to an "RB set." RAN1 has specified in 37.213 that a "channel" corresponds to a carrier or part of a carrier consisting of a set of contiguous RBs over which a channel access procedure is performed. Hence, from a RAN1 perspective an RB set corresponds to a "channel," and an index is associated with each RB set with value in the range 0 .. 3.

Proposal 2	Provide answer to RAN3 with respect to Q2 that the channel access procedures specified in TS 37.213 may be performed when there is a need to access an NR-U channel for packet transmissions over the NR-U channel. In other words, before performing a packet transmission on an NR-U channel, access to the NR-U channel should be obtained by successfully completing the corresponding channel access procedures that are specified in TS 37.213. However, a node (NG-RAN or UE) is neither required to continuously sense an NR-U channel even no data is available for transmission, nor is a node (NG-RAN or UE) required to perform the actual packet transmission even after the corresponding channel access procedures are successfully completed.

Proposal 3	Provide answer to RAN3 with respect to Q3 that the determination of the ED threshold for NG-RAN nodes to perform DL transmissions for n46 and n96 NR-U channels is specified in Clause 4.1.5 of TS 37.213 where the ED threshold is associated with a set of contiguous RBs of the “channel” that an NG-RAN node intends to perform sensing as described in the answer to Q1. It is important to note that there are few conditions as described in Clause 4.1.5 of TS 37.213 that determine how the ED threshold for a node is obtained. The ED threshold by itself is not specified to be “exchanged” between RAN-nodes as RAN3 expressed in the question. Whether the contributing parameters in determining the corresponding ED threshold are exchanged between RAN nodes is not within the RAN1 expertise.

Proposal 4	Provide answer to RAN3 with respect to Q4 that RAN1 suspects that perhaps the existing RRC parameters maxEnergyDetectionThreshold-r16 or energyDetectionThresholdOffset-r16 have contributed to this question. If that is the case, we would like to clarify that these parameters are configured to the UE to determine the ED threshold to perform sensing for UL transmissions and are not applicable to the ED threshold at NG-RAN to perform sensing for DL transmissions. The corresponding configurations are UE specific and configured per serving cell. How these parameters are used to determine the ED threshold at UE for sensing to perform UL transmissions is specified in Clause 4.2.3 of TS 37.213. We would like to clarify that irrespective of the node being NG-RAN or UE, the corresponding ED threshold is determined for the set of contiguous RBs of the “channel” that the node intends to perform sensing as described in the answer to Q1.

# References

R1-2200860, "LS on NR-U channel information and procedures," RAN3, RAN1#108-e, February 2022.

R1-2200891, "Reply LS on NR-U channel information and procedures," RAN2, RAN1#108-e, February 2022.

R1-2201743, "Draft LS Reply to RAN3 on NR-U channel information and procedures," Ericsson, RAN1#108-e, February 2022.

3GPP TS 38.101-1, "User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone," v17.4.0, December 2021.

3GPP TS 37.213, "Physical layer procedures for shared spectrum channel access," v17.0.0, December 2021.

3GPP TS 38.214, "Physical layer procedures for data," v17.0.0, December 2021.