3GPP TSG RAN WG1 #108-e	R1-2201747

e-Meeting, Feb 21st – Mar 3rd, 2022

Title:	Draft Reply LS on Enhanced TCI State Indication for UE-Specific PDCCH MAC CE

Response to:	R1-2200886/R2-2201995

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	Apple, [RAN1]

To:	RAN2

Cc:

Contact Person:

Name:	Yushu Zhang

E-mail Address:	yushu_zhang@apple.com

# 1. Overall Description

RAN1 would like to thank RAN2 for the LS on Enhanced TCI State Indication for UE-Specific PDCCH MAC CE. RAN1 would like to provide the answer to the following question.

Q: RAN2 would like to ask whether “Enhanced TCI state indication for UE specific PDCCH MAC CE” can be applied to CORESET zero or not.

A: It is RAN1’s understanding that the “Enhanced TCI state indication for UE specific PDCCH MAC CE” can be applied to CORESET zero. For CORESET zero, only the TCI state with the CSI-RS QCLed with an SSB from serving cell configured as the source reference signal is allowed to be indicated.

# 3. Actions:

To: RAN2

RAN1 respectfully asks RAN2 to take the above information into account for future work.

# 4. Date of Next RAN1 Meetings:

TSG RAN WG1   Meeting #109-e            	   May. 16 – 27, 2021          	Online