3GPP TSG- RAN WG1 Meeting #108-e                                              R1- 2201760

e-Meeting, Feb 21st – Mar 3rd, 2022

Agenda Item:	8.1.2.2

Source:	Apple Inc.

Title:	On Inter-Cell Multi-TRP Enhancement

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussion on remaining issues for inter-cell multi-TRP enhancement.

# Discussion

## Simultaneous reception of SSB + other signal

For SSB + PDSCH and SSB + CSI-RS for BM, the following restriction to ensure the QCL is defined in38.214.

If the UE receives the DM-RS for PDSCH and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that the DM-RS and SS/PBCH block are quasi co-located with 'typeD', if 'typeD' is applicable.

If the UE is configured with the CSI-RS resource in the same OFDM symbol(s) as an SS/PBCH block, the UE may assume that the CSI-RS and the SS/PBCH block are quasi co-located with 'QCL-TypeD' if 'QCL-TypeD' is applicable.

The QCL rule above should be applicable for signals associated with the same PCI. Further, it can also be extended to support SSB + other signals case.

Proposal 1: If SSB collides with DL signals associated with the same PCI, gNB should ensure the DL signals and SSB are QCLed with QCL-TypeD.

## Cell ID for signal generation

Rel-15 supports the configuration of virtual cell ID for downlink signal generation. If the virtual cell ID is not configured, physical cell ID would be used. For example, currently the virtual cell ID for PDSCH is defined as follows:

- equals the higher-layer parameter dataScramblingIdentityPDSCH if configured and the RNTI equals the C-RNTI, MCS-C-RNTI, or CS-RNTI, and the transmission is not scheduled using DCI format 1_0 in a common search space; - nID∈0,1,…,1023 equals - the higher-layer parameter dataScramblingIdentityPDSCH if the codeword is scheduled using a CORESET with CORESETPoolIndex equal to 0; - the higher-layer parameter dataScramblingIdentityPDSCH2 if the codeword is scheduled using a CORESET with CORESETPoolIndex equal to 1; if the higher-layer parameters dataScramblingIdentityPDSCH and dataScramblingIdentityPDSCH2 are configured together with the higher-layer parameter CORESETPoolIndex containing two different values, and the RNTI equals the C-RNTI, MCS-C-RNTI, or CS-RNTI, and the transmission is not scheduled using DCI format 1_0 in a common search space; - nID=NIDcell otherwise

For inter-cell mTRP, the definition of NIDcell needs to be clarified. For signals associated with the non-serving cell, it is not reasonable to use PCI for serving cell as the default cell ID, but the PCI for the non-serving cell should be applied.

Proposal 2: For downlink signals associated with a non-serving cell, if virtual cell ID is not configured, the default ID should be PCI for the non-serving cell.

# Conclusion

In this contribution, we discussed inter-cell multi-TRP enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: If SSB collides with DL signals associated with the same PCI, gNB should ensure the DL signals and SSB are QCLed with QCL-TypeD.

Proposal 2: For downlink signals associated with a non-serving cell, if virtual cell ID is not configured, the default ID should be PCI for the non-serving cell.