3GPP TSG- RAN WG1 Meeting #108-e                                              R1- 2201761

e-Meeting, Feb 21st – Mar 3rd, 2022

Agenda Item:	8.1.2.3

Source:	Apple Inc.

Title:	On Multi-TRP Beam Management Enhancement

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussion on remaining issues on multi-TRP beam management enhancement.

# Discussion

## Beam Measurement and Report

### Effective time window for a group-based beam report

For the enhanced group-based beam report, the beams reported in one pair can be received simultaneously. However, after UE reports one beam pair, it does not mean the two beams can always be received simultaneously. An effective time window for each beam report instance needs to be defined. Only the reported beams within the effective time window can be assumed to be received simultaneously. Thus, a UE capability on the duration for the effective time window for a group-based beam report should be supported.

Proposal 1: The beams in a beam pair reported in a group-based beam report instance can be simultaneously received by UE within an effective time window.

- The duration of the effective window can be reported by UE capability

## BFR Enhancement

### Beam failure detection

It was agreed that 2 BFD RS sets can be used for TRP-specific BFR. One open issue is how to derive the BFD RS. With regard to high latency to update the BFD RS by RRC, it is better to consider some mechanism like DCI/MAC CE based BFD RS update, which can be aligned with TCI states. One possible way is to use the same approach as PL-RS, where a BFD RS selected from the candidate 2 BFD RS sets can be associated with a TCI state as shown in Figure 2. Then it can support more flexible BFD RS update and UE can derive potential failed BFD RS set for single-DCI mode.

Figure 1: BFD RS and TCI state association

Proposal 2: Support to configure a BFD RS index selected from the 2 candidate BFD RS sets configured by RRC to be associated with a TCI state

For single DCI mode, since dynamic switching between sDCI and sTRP mode is supported, when it is switched back to sTRP mode, UE does not need to perform BFD. UE may still maintain the BFD counter and timer within a short time in case UE switches back to sDCI mode.

Proposal 3: For sDCI mode, UE should not be required to always keep TRP-specific BFR when it switches to sTRP mode

- UE may maintain the BFD counter and timer within a time window in case it switches back to sDCI mode

For implicit BFD RS selection, it is agreed that the RS configured in TCI state for a CORESET is used as BFD RS. When there are two DL RSs configured in TCI state, it is straight-forward to reuse Rel-15 behaviour to use the one to provide QCL-TypeD indication as the BFD RS. Hence the following text proposal is proposed.

Proposal 4: Endorse the following text proposal on implicit BFD RS selection

# 6 Link recovery procedures <unrelated part omitted> If the UE is not provided q0 by failureDetectionResourcesToAddModList for a BWP of the serving cell, the UE determines the set q0 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH. If the UE is not provided q0,0 or q0,1 for a BWP of the serving cell, the UE determines the set q0,0 or q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for first and second CORESETs that the UE uses for monitoring PDCCH, where the UE is provided two coresetPoolIndex values 0 and 1 for the first and second CORESETs, or is not provided coresetPoolIndex value for the first CORESETs and is provided coresetPoolIndex value of 1 for the second CORESETs, respectively. If there are two RS indexes in a TCI state, the set q0, or q0,0, or q0,1q0includes RS indexes configured with qcl-Type set to 'typeD' for the corresponding TCI states. If a CORESET that the UE uses for monitoring PDCCH includes two TCI states and the UE is provided sfnSchemePdcch set to 'sfnSchemeA' or 'sfnSchemeB', the set q0 includes RS indexes in the RS sets associated with the two TCI states. The UE expects the set q0 to include up to two RS indexes. The UE expects the set q0,0 or the set q0,1 to include up to a number of NBFD RS indexes indicated by capabilityparametername. If a number of active TCI states for PDCCH receptions in the first or second CORESETs is larger than NBFD, the UE determines the set q0,0 or q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets associated with the active TCI states for PDCCH receptions in the first or second CORESETs corresponding to search space sets according to an ascending order for monitoring periodicity. If more than one first or second CORESETs correspond to search space sets with same monitoring periodicity, the UE determines the order of the first or second CORESETs according to a descending order of a CORESET index. The UE expects single port RS in the set q0, or q0,0, or q0,1. The UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in the set q1, or q1,0, or q1,1.

### Candidate Beam Detection

With regard to inter-cell multi-TRP operation, the non-serving cell SSB can be configured for candidate beam detection. As different cells may have different Tx power, the RSRP threshold for candidate beam detection can be configured in a cell-specific manner.

Proposal 5: Support to configure non-serving cell SSB for candidate beam detection.

Proposal 6: Support to configure cell-specific candidate beam detection RSRP threshold.

### Beam failure recovery request procedure

In RAN1 #106b, the following agreement on 2 PUCCH-SR is achieved.

Agreement - Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. - FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR - A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case.

One open issue is which PUCCH-SR should be triggered when one BFD RS set fails. Since the association is configured by gNB, the associated PUCCH-SR should be triggered when corresponding BFD RS set fails. Whether the associated PUCCH-SR and the BFD RS set belong to the same TRP or not can be transparent to UE. If multiple PUCCH-SRs are triggered, legacy SR multiplexing/dropping rule can be reused.

Proposal 7: When one SCell BFD RS set fails, UE can trigger the associated PUCCH-SR

- If multiple PUCCH-SRs are triggered, legacy SR multiplexing/dropping rule can be reused

## QCL-TypeD collision handling

In Rel-15, some QCL-TypeD collision handling rules have been defined as shown in Table 1.

Table 1: QCL-TypeD collision handling rules

| Signal 1 | Signal 2 | Behavior |
| --- | --- | --- |
| PDSCH (scheduling offset before threshold) | PDCCH | Prioritize PDCCH. |
| PDSCH | SSB not used for RLM/CBD | gNB should ensure QCL. |
| CSI-RS not used for RLM/BFD/RLM | PDCCH | Not allowed for CSI-RS with repetition=on, for other CSI-RS, gNB should ensure QCL. |
| CSI-RS not used for RLM/BFD/CBD | SSB not used for RLM/CBD | gNB should ensure same QCL. |
| Aperiodic CSI-RS for CSI (scheduling offset below threshold) | Other signal | Prioritize other signal. |
| PDCCH | PDCCH | Follow a priority rule |
| SSB/CSI-RS for RLM/BFD | Other DL signal | Not allowed |
| SSB/CSI-RS for CBD | Other DL signal | Not allowed |

In Rel-17, inter-cell mTRP operation is going to be supported, which would create new cases:

- Case 1: SSB + SSB from a different PCI

- Case 2: SSB + PDSCH/PDCCH/CSI-RS QCLed with SSB from a different PCI

The 2 new cases above need to be resolved. For Case 1, a simple dropping rule should not be helpful, since UE needs to measure different SSBs from different cells. Case 2 may be connected with Case 1, but UE may not be able to receive both signals at the same time especially if the SSB is used for RLM/BFD/CBD or the SSB is from a third cell,

Proposal 8: For simultaneous reception, RAN1 should prioritize the following new cases, which has not been discussed in Rel-15:

- Case 1: SSB + SSB from a different PCI

- Case 2: SSB + PDSCH/PDCCH/CSI-RS QCLed with SSB from a different PCI

# Conclusion

In this contribution, we discussed multi-TRP BM enhancement. Based on the discussion, the following proposals have been achieved.

Beam report

Proposal 1: The beams in a beam pair reported in a group-based beam report instance can be simultaneously received by UE within an effective time window.

- The duration of the effective window can be reported by UE capability

BFR

Proposal 2: Support to configure a BFD RS index selected from the 2 candidate BFD RS sets configured by RRC to be associated with a TCI state

Proposal 3: For sDCI mode, UE should not be required to always keep TRP-specific BFR when it switches to sTRP mode

- UE may maintain the BFD counter and timer within a time window in case it switches back to sDCI mode

Proposal 4: Endorse the following text proposal on implicit BFD RS selection

# 6 Link recovery procedures <unrelated part omitted> If the UE is not provided q0 by failureDetectionResourcesToAddModList for a BWP of the serving cell, the UE determines the set q0 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH. If the UE is not provided q0,0 or q0,1 for a BWP of the serving cell, the UE determines the set q0,0 or q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for first and second CORESETs that the UE uses for monitoring PDCCH, where the UE is provided two coresetPoolIndex values 0 and 1 for the first and second CORESETs, or is not provided coresetPoolIndex value for the first CORESETs and is provided coresetPoolIndex value of 1 for the second CORESETs, respectively. If there are two RS indexes in a TCI state, the set q0, or q0,0, or q0,1q0includes RS indexes configured with qcl-Type set to 'typeD' for the corresponding TCI states. If a CORESET that the UE uses for monitoring PDCCH includes two TCI states and the UE is provided sfnSchemePdcch set to 'sfnSchemeA' or 'sfnSchemeB', the set q0 includes RS indexes in the RS sets associated with the two TCI states. The UE expects the set q0 to include up to two RS indexes. The UE expects the set q0,0 or the set q0,1 to include up to a number of NBFD RS indexes indicated by capabilityparametername. If a number of active TCI states for PDCCH receptions in the first or second CORESETs is larger than NBFD, the UE determines the set q0,0 or q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets associated with the active TCI states for PDCCH receptions in the first or second CORESETs corresponding to search space sets according to an ascending order for monitoring periodicity. If more than one first or second CORESETs correspond to search space sets with same monitoring periodicity, the UE determines the order of the first or second CORESETs according to a descending order of a CORESET index. The UE expects single port RS in the set q0, or q0,0, or q0,1. The UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in the set q1, or q1,0, or q1,1.

Proposal 5: Support to configure non-serving cell SSB for candidate beam detection.

Proposal 6: Support to configure cell-specific candidate beam detection RSRP threshold.

Proposal 7: When one SCell BFD RS set fails, UE can trigger the associated PUCCH-SR

- If multiple PUCCH-SRs are triggered, legacy SR multiplexing/dropping rule can be reused

Simultaneous reception

Proposal 8: For simultaneous reception, RAN1 should prioritize the following new cases, which has not been discussed in Rel-15:

- Case 1: SSB + SSB from a different PCI

- Case 2: SSB + PDSCH/PDCCH/CSI-RS QCLed with SSB from a different PCI