3GPP TSG- RAN WG1 Meeting #108-e                                                 R1-2201779

# e-Meeting, Feb 21th – Mar 3rd, 2022

Agenda Item:	8.7.2

Source:	Apple Inc.

Title:	Remaining details on enhanced DCI-based power saving adaptation

Document for:	Discussion/Decision

# Introduction

In this paper, we discuss remaining open issues related to enhanced DCI based power saving adaptation, including timer behavior, application timeline and interaction with HARQ.

# Discussion

## Timer behavior for PDCCH monitoring adaptation

It has been agreed that 2 bits will be included in the DCI for PDCCH monitoring adaptation. Four different cases have been agreed. Timer can be optionally configured for SSSG switching, which can be used for case 2,3 and 4.

For case 2 with 2 SSSG, timer behaviour follows R16 NR-U SSSG switching. Timer is reset when DCI bit carry ‘01’. Otherwise decrease the timer value by one after each slot.  When timer expires, fall back to default SSSG.

For case 3 with 3 SSSG, where one default SSSG can be configured with 2 non-default SSSGs, it has been agreed that one timer is configured for the non-default SSSG.

To simplify the overall design, the design of 3 SSSG should maximize the commonality as 2 SSSG.  Timer is reset when DCI bit carry ‘01’ or ‘10’. Otherwise decrease the timer value by one after each slot. When timer expires, fall back to default SSSG.

Proposal 1: In case 2 and case 3,

- When Beh 2/2A and/or 2B are signaled in one DCI, timer is reset when DCI bit carry ‘01’ or ‘10’.

- Otherwise decease the timer by one after each slot.

- When timer expires, fall back to default SSSG.

For case 4, the 2 bits signaling have been agreed in [1] to indicate ‘00’ Beh 2, ‘01’ Beh 2A, ‘10’ Beh 1A for duration T1, and ‘11’ Beh 1A for duration T2. The main open question is the timer behaviour with the interaction between PDCCH skipping and SSSG switching.

In case of 2 SSSG and 2 skipping size, it is not clear whether the timer should reset only when SSSG1 is indicated, i.e., when DCI bit carry ‘01’ only, or timer should reset when PDCCH skipping or SSSG1 is indicated, i.e., ‘01’, ‘10’, ‘11’ are indicated. If timer reset only when SSSG1 is indicated, it is likely that the timer will expire during PDCCH skipping. To avoid the complicated timer fallback behaviour in this case, it is desirable to reset the timer when PDCCH skipping is indicated as well. In addition, the configure timer value is always larger than T1/T2, or timer does not count down during PDCCH skipping.

Proposal 2:  In case 4 when both Beh 1/1A/2/2A is enabled,

- Timer is reset if the UE detect a DCI format 0-1, 0-2, 1-1 and 1-2 and the value for PDCCH adaptation bit value is set to ‘01’, ‘10’, ‘11’.

- Otherwise, decrease the timer value by one after each slot, if UE does not apply PDCCH skipping.

- When timer expires, fall back to default SSSG.

2.2 Application timeline

When PDCCH monitoring adaptation is triggered by scheduling DCI, various options have been proposed for skipping and SSSG switching adaptation timeline, with interaction to HARQ retransmission. In [2], the application delay cases are described as follows,

- Case 1: Upon detecting a scheduling DCI format 1-1/1-2 indicating PDCCH skipping (i.e., Beh 1A),

- Case 2: Upon detecting a scheduling DCI format 0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A),

- Case 3: Upon detecting a scheduling DCI format 1-1/1-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

- Case 4: Upon detecting a scheduling DCI format 0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

- Case 5: Upon SSSG timer expiry.

For PDCCH skipping and SSSG switching, different alternatives have been captured in [2] based on whether there is an interaction with retransmission or not.

- Alt 1 (no interaction with retransmission).

- Alt 2 (has interaction with retransmission and applied after HARQ-ACK transmission):

- Alt 3 (has interaction with retransmission and can be applied before HARQ-ACK transmission):

From performance point of view, UE power saving and UE perceived throughput are two main metrics need to be balanced. Typically, larger skipping size will have higher UE power saving benefit at the expense of longer delay and lower UE perceived throughput. Perform PDCCH skipping regardless of whether HARQ retransmission is needed will further reduce the UE perceived throughput.

We support Alt 2 as the solution for PDCCH skipping and SSSG switching adaptation. Apply the triggering commend after ACK transmission minimize the impact to HARQ retransmission. It is also maximizing UE power saving gain since skipping PDCCH monitoring while receiving PDSCH does not save UE power.

In addition, with ACK transmission, the UE acknowledge the correct receiving of the DCI implicitly. If NACK is sent by the UE, the gNB does not know whether the UE missed DCI itself, or failed PDSCH decoding. There can be misalignment between the UE and the gNB on PDCCH monitoring occasion. To handle this problem, when NACK is received by the gNB, the previous triggering command is cancelled, and the gNB needs to send another triggering command with retransmissions scheduling DCI.

Proposal 3: HARQ retransmission should be taken into consideration for PDCCH skipping and SSSG switching adaptation.

Proposal 4: For PDCCH skipping, when triggered by DCI format 1-1 and 1-2, PDCCH skipping applies after the last OFDM symbol of ACK transmission.

Proposal 5: For PDCCH skipping, when PDCCH monitoring adaptation is triggered by DCI format 0-1 and 0-2, PDCCH skipping applies after drx-RetransmissionTimerUL expires.

For SSSG switching, additional processing time is needed to process the new SSSG configuration and get the UE ready to monitor PDCCH based on the new configuration. This processing time is required every time there is a SSSG switching, such as after ACK transmission, or timer expires.

Proposal 6: For SSSG switching, when triggered by DCI format 1-1 and 1-2, SSSG switching applies at least Pswitch symbols after the slot of ACK transmission.

Proposal 7: For SSSG switching, when triggered by DCI format 0-1 and 0-2, SSSG switching applies at least Pswitch symbols after the slot when drx-RetransmissionTimerUL expires.

Proposal 8: When timer expires, UE should start monitoring default SSSG at least Pswitch symbols after the slot when timer expires.

# Conclusion

In the contribution, we discuss further details on enhanced DCI based adaptation for UE power saving.  The proposals are:

Proposal 1: In case 2 and case 3,

- When Beh 2/2A and/or 2B are signaled in one DCI,  timer is reset when DCI bit carry ‘01’ or ‘10’.

- Otherwise decease the timer by one after each slot.

- When timer expires, fall back to default SSSG.

Proposal 2:  In case 4 when both Beh 1/1A/2/2A is enabled,

- Timer is reset if the UE detect a DCI format 0-1, 0-2, 1-1 and 1-2 and the value for PDCCH adaptation bit value is set to ‘01’, ‘10’, ‘11’.

- Otherwise, decrease the timer value by one after each slot, if UE does not apply PDCCH skipping.

- When timer expires, fall back to default SSSG.

Proposal 3: HARQ retransmission should be taken into consideration for PDCCH skipping and SSSG switching adaptation.

Proposal 4: For PDCCH skipping, when triggered by DCI format 1-1 and 1-2, PDCCH skipping applies after the last OFDM symbol of ACK transmission.

Proposal 5: For PDCCH skipping, when PDCCH monitoring adaptation is triggered by DCI format 0-1 and 0-2, PDCCH skipping applies after drx-RetransmissionTimerUL expires.

Proposal 6: For SSSG switching, when triggered by DCI format 1-1 and 1-2, SSSG switching applies at least Pswitch symbols after the slot of ACK transmission.

Proposal 7: For SSSG switching, when triggered by DCI format 0-1 and 0-2, SSSG switching applies at least Pswitch symbols after the slot when drx-RetransmissionTimerUL expires.

Proposal 8: When timer expires, UE should start monitoring default SSSG at least Pswitch symbols after the slot when timer expires.

# Reference

- Chairman Notes, 3GPP TSG RAN WG1 Meeting #107-e, e-Meeting, Nov 11th – Nov 19th, 2021

- R1-2112878, Final FL summary of DCI-based power saving adaptation, Vivo, Nov 11th – 19th, 2021.