3GPP TSG RAN WG1 #108-e		                               R1-2201790

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.14.2

Source:	Apple

Title:	Remaining Issues of Timing Relationship Enhancements for IoT NTN

Document for:	Discussion/Decision

# Introduction

It was concluded [1] that release 17 RAN1 work on IoT NTN is completed. The maintenance on solutions for IoT over NTN is conducted in RAN1 #108-e meeting.

One of the RAN1-lead objectives in release 17 IoT NTN is timing relationship enhancements. In this contribution, we provide our views on remaining issues related to timing relationship enhancements for IoT NTN.

# Discussion

It was agreed [2] that network can configure UE-specific TA reporting either a TA or UE location for connected mode UE. In case a TA is configured, NR NTN solutions are a baseline for the following UE-specific TA handling issues: signaling (quantity, range, number of bits), granularity of reporting, frequency of reporting and means of reporting.

It was agreed in NR NTN that UE can report the information about its TA in connected mode, triggered by an event based on its TA value. The reported TA is the least integer number of slots greater than or equal to the corresponding TA value, where 15 kHz is used as the reference subcarrier spacing value for the unit of TA reported in FR1.

The similar conclusions could be applied to IoT NTN. Specifically, UE can report the information about its TA in connected mode, triggered by an event based on its TA value. The reported TA is the least integer number of subframes greater than or equal to the corresponding TA value.

Proposal 1: UE reporting of information about its TA in connected mode is supported. The reporting is triggered by an event based on UE’s TA value.

Proposal 2: The reported TA is the least integer number of subframes greater than or equal to the corresponding TA value.

# Conclusion

In this contribution, we provided our views on remaining issues of timing relationship enhancements for IoT NTN. Our proposals are as follows:

Proposal 1: UE reporting of information about its TA in connected mode is supported. The reporting is triggered by an event based on UE’s TA value.

Proposal 2: The reported TA is the least integer number of subframes greater than or equal to the corresponding TA value.

# References

- RP-213349, “Status report for RP94 IoT NTN WI,” Dec. 2021.

- R1-2112554, “FL summary 3 of AI 8.15.2 timing relationship for IoT-NTN,” Nov. 2021.