3GPP TSG RAN WG1 #108-e		R1-2201813

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:     5

Source:	Spreadtrum Communications

Title:	      Discussion on LS on MBS issues

Document for:	Discussion and decision

Introduction

RAN2 has sent RAN1 one LS on MBS issues [1]. For reference, the overall description is repeated below:

RAN2 would like to thank RAN1 for providing the list of L1 parameters to be configured with RRC signalling. RAN2 has discussed some aspects related to this topic and would like to ask RAN1 the following question. Question: Currently, RAN2 running RRC design assumes that only a single CFR (indicated by locationAndBandwidth-Broadcast) is configured for MCCH/MTCH reception of MBS broadcast and it is common for MCCH and all MTCHs. RAN2 would like to confirm this understanding with RAN1. Furthermore, RAN2 has decided to include MCCH/MTCH search space configuration of MBS broadcast as part of PDCCH-ConfigCommon, to have a unified configuration framework for all common search spaces.

In this contribution, we discuss and present our views on RAN2’s question.

Discussion

Regarding CFR configuration for MCCH and MTCH, some related agreements have been achieved in previous RAN1 meetings [2][3] as shown below:

RAN1#106e Agreement From RAN1 perspective, the CFR for broadcast reception of RRC_IDLE/INACTIVE UEs, includes at least the following configurations: - One set of parameters configured for PDSCH for broadcast reception with GC-PDSCH - One set of parameters configured for PDCCH for broadcast reception with GC-PDCCH - FFS: whether some parameters configured for PDSCH/PDCCH are optional/needed for the supported cases of CFR. - FFS: If necessary, depending on the cases supported, starting PRB and the number of PRBs - The reference for starting PRB is Point A. (Following the same approach to determine reference for starting PRB as that defined in AI8.12.1.) Agreement Only one CFR can be configured for group-common PDCCH/PDSCH carrying MCCH for broadcast reception with UEs in RRC_IDLE/INACTIVE state. RAN1#107e Agreement For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs: - The CFR frequency resources used for MCCH and MTCH are configured by SIBx; - PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH is configured by SIBx - PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MTCH is configured by MCCH. If the PDCCH-config/PDSCH-config for MTCH is not configured, the PDCCH-config/PDSCH-config for GC-PDCCH/PDSCH carrying MCCH configured by SIBx is reused for GC-PDCCH/PDSCH carrying MTCH.

Based on the above the agreements, it can be seen that up to one CFR can be configured for MCCH.

Regarding CFR for MTCHs, in last RAN1 meeting, we have discussed this issue, but unfortunately, we have not the consensus. For connected state, only one CFR can be configured in one BWP for the reception of one or multiple MBS services. Likewise, we also think that up to one CFR for all MTCHs can be configured by MCCH, but the CFR frequency resource is the same as the frequency resource of CFR for MCCH configured by SIBx.

Proposal 1: Up to one CFR for all MTCH(s) can be configured by MCCH.

- The CFR for MTCH(s) if configured by MCCH has the same frequency resource as CFR for MCCH configured by SIBx

Regarding RAN2 question, our understanding is that up to 1 CFR can be configured for MCCH and MTCH, respectively, wherein SIBx configures one CFR for MCCH and another CFR is configured by MCCH for MTCH.

Observation 1: Up to 1 CFR can be configured for MCCH, and MTCH(s), respectively.

Conclusion

In this contribution, we provide our opinions on CFR configuration for MCCH and MTCHs.

Proposal 1: Up to one CFR for all MTCH(s) can be configured by MCCH.

- The CFR for MTCH(s) if configured by MCCH has the same frequency resource as CFR for MCCH configured by SIBx

Observation 1: Up to 1 CFR can be configured for MCCH, and MTCH(s), respectively.

Reference

- R1-2200882, “LS on MBS issues”, RAN2, Huawei

- RAN1 Chairman’s Notes, RAN1#106-e, e-Meeting

- RAN1 Chairman’s Notes, RAN1#107-e, e-Meeting