3GPP TSG RAN WG1 #108-e		R1-2201815

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:     8.12.1

Source:	Spreadtrum Communications

Title:	      Discussion on the remaining issues on MBS group scheduling for

RRC_CONNECTED UEs

Document for:	Discussion and decision

Introduction

In this paper, we will present our opinions on some remaining issues on MBS group scheduling for RRC_CONNECTED UEs, and provide some text proposals on this issue.

Discussion

In Rel-15/16, if a UE is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) of one or more cells, the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs having same QCL-TypeD properties as the CORESET, on the active DL BWP of a cell from the one or more cells, which corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index, and the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions.

In current 38.213 spec [1], CSS type-3 is for multicast scheduling, and DCI format 2_x cannot be configured in the same CSS configuration with multicast DCI format 4_1 and 4_2. If following Rel-15/16 QCL-typeD property selection rule, it means that MBS can always prioritize unicast. It is not consistent with the principle we have kept that MBS can be the same or even lower priority than unicast. For example, we have already agreed that for PDCCH overbooking, the monitoring priority of Type-3 CSS for multicast DCI format is determined based on the search space set indexes of the Type-3 CSS set for MBS and USS sets. The same principle should be applied for QCL-typeD property selection rule. Thus, we have the following proposal:

Proposal 1: In overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) of one or more cells, the monitoring priority of CORESET assocaited with Type-3 CSS for DCI format 4-1 and 4-2 is determined based on the search space set indexes of the Type-3 CSS set for DCI format 4-1 and 4-2  and USS sets for a serving cell.

According to current 38.214 spec [2], UE can be configured with a list of up to M’ TCI-State configurations within the higher layer parameter PDSCH-Config-Multicast to decode PDSCH associated with a G-RNTI or a G-CS-RNTI according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M’ depends on the UE capability.

In RAN1#107b-e meeting, there are some related discussions on TCI state configuration/activation for MBS multicast, and it seems that there is no consensus among people on whether to introduce separated TCI state pool for MBS multicast. In our understanding, MBS multicast sharing the same TCI state pool with unicast is not proper in some scenarios.  For example, for single cell with multiple TRPs case, unicast PDSCH can be from one TRP, multicast can be from another TRP. The large scale parameters of channel for unicast and multicast are totally different. More flexibility can be provided for gNB scheduling if separated TCI states configuration and activation/deactivation for MBS can be considered.

Observation 1: Introducing separated TCI state configuration/activation/deactivation for MBS is beneficial for some scenarios, e.g., single cell with multiple TRPs operation

No matter whether separated TCI state configuration/activation/deactivation for MBS is supported, considering the maximum number of configured TCI state per CC would affect UE’s complexity, e.g., buffer/memory size, following the principle of minimizing UE complexity when introducing MBS which is explicitly expressed by WID, in our mind, the maximum number of configured TCI states per cell, currently supported for unicast, should be kept unchanged for UE to additionally support multicast reception. How to allocate configured TCI state between unicast and multicast is up to gNB.

Regarding the number of active TCI state per BWP when supporting group-common PDSCH which would affect UE’s tracking capability, we also think the total number of active TCI state across unicast and multicast should not be beyond the legacy UE’s capability, to keep the UE complexity not increased.

Based on the above, we suggest to have the following conclusions:

Conclusion 1:

The maximum number of configured TCI states per cell, currently supported for unicast, is kept unchanged for UE to additionally support multicast reception.

- How to allocate configured TCI state between unicast and multicast is up to gNB.

Conclusion 2:

The maximum number of active TCI states per BWP, currently supported for unicast, is kept unchanged for UE to additionally support multicast reception.

- How to allocate active TCI state between unicast and multicast is up to gNB.

For MBS multicast, we also have supported group-common PDCCH to activate and release SPS group-common PDSCH, but we notice that they have not been reflected in latest specification 38.213. So we suggest to have the following proposal.

Proposal 2: Suggest to adopt the following text proposal in 38.213

------------------------------------------Start of Text Proposal#1 for TS 38.213--------------------------------------

- 10.2	PDCCH validation for DL SPS and UL grant Type 2

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, or a

G-CS-RNTI provided by g-cs-RNTI, and

-----------------------------Unchanged part omitted--------------------------

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2/4_1/4_2 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-4: Special fields for a single or multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2/4_1/4_2 |
| --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

------------------------------------------End of Text Proposal#1 for TS 38.213--------------------------------------

Conclusion

In this contribution, we provide our opinions on MBS group scheduling for RRC_CONNECTED UEs:

Observation 1: Introducing separated TCI state configuration/activation/deactivation for MBS is beneficial for some scenarios, e.g., single cell with multiple TRPs operation

Conclusion 1:

The maximum number of configured TCI states per cell, currently supported for unicast, is kept unchanged for UE to additionally support multicast reception.

- How to allocate configured TCI state between unicast and multicast is up to gNB.

Conclusion 2:

The maximum number of active TCI states per BWP, currently supported for unicast, is kept unchanged for UE to additionally support multicast reception.

- How to allocate active TCI state between unicast and multicast is up to gNB.

Proposal 1: In overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) of one or more cells, the monitoring priority of CORESET assocaited with Type-3 CSS for DCI format 4-1 and 4-2 is determined based on the search space set indexes of the Type-3 CSS set for DCI format 4-1 and 4-2  and USS sets for a serving cell.

Proposal 2: Suggest to adopt the following text proposal in 38.213

------------------------------------------Start of Text Proposal#1 for TS 38.213--------------------------------------

- 10.2	PDCCH validation for DL SPS and UL grant Type 2

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, or a

G-CS-RNTI provided by g-cs-RNTI, and

-----------------------------Unchanged part omitted--------------------------

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2/4_1/4_2 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-4: Special fields for a single or multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2/4_1/4_2 |
| --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

------------------------------------------End of Text Proposal#1 for TS 38.213--------------------------------------

Reference

- 3GPP TS 38.213: "NR; Physical layer procedures for control", h00

- 3GPP TS 38.214: "NR; Physical layer procedures for data", h00